3GPP TSG SA WG5 Meeting 136-e		TDoc S5-212152

- electronic meeting, online, 1 - 9 March 2021

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Introduction of NS converged charging sol#x.1 single CCS KI#x

- Document for:	Approval

Agenda Item:	7.5.5

# 1	Decision/action requested

This pCR is to introduce Network slice converged charging sol#x.1 single CCS KI#x

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 32.845 Study on Charging Aspects of Network Slicing

[3]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[4]	GSMA 5GJA NG.116: "Generic Network Slice Template".

# 3	Rationale

This pCR is to introduce a solution #x.1 for Key issue#x – single CCS

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

### 6.x.2	Solution#x.1 Volume based Network slice converged charging – Single CCS

#### 6.x.2.1	General description

This solution addresses the Key Issue#x for REQ-NSCH-09 potential requirement and is based on CHF/5G Converged Charging System capabilities.

In this solution there is a single Converged Charging System (CCS) with its functional modules:

-	Charging Function (CHF) exposing Nchf_ConvergedCharging service

-	Account Balance Management Function (ABMF) hosting subscribers’s account balance

-	Charging Gateway Function (CGF) for CDRs files transfer to the BD.

-	Rating Function (RF) for rating the resource usage.

Both types of subscriber are handled by the CCS, i.e Network Slice tenant and end-user, respectively referred-to as NS-Tenant and UE for the sake of simplification:

-	Nchf_ConvergedCharging service is exposed by CHF for individual UE.

-	ABMF contains NS-Tenant account balance as well as UE account balance.

-	CDRs relate to both UE and NS-Tenant.

-	Rating Function (RF) is for rating the resource usage by the NS-Tenant as well as the resource usage by the UE.

How the account balance is provisioned with the subscriber's profile (for both NS-tenant and individual end-user) and various subscribed-to services is unspecified and expected to be covered at BSS level.

The NS-Tenant account balance is assumed to be provisioned with all necessary information related to the Network Slice matching SLS requirements the NS-Tenant has paid for. The subscription description includes in particular the list of service profiles subscribed-to by the NS-Tenant, each service profile with assigned group of S-NSSAI(s).

The individual UE account balance is assumed to be provisioned with all necessary information related to subscribed-to services the UE has paid for. The subscription description includes in particular the subscribed NSSAI (set of S-NSSAI(s)).

The 5G data connectivity charging from SMF specified in TS 32.255 [x] is used for UE PDU session control by CCS.

The volume based converged charging per S-NSSAI is based on CCS aggregating individual UEs PDU session volume under converged charging.

6.x.2.2	Architecture description

The architecture for this solution is the existing TS 32.255 [x] 5G data connectivity converged charging:

Figure 6.x.2.2.1: Network Slice Converged charging architecture – Volume based

#### 6.x.2.3	Flow description

The figure 6.x.2.3.1 describes the high level charging procedure for Volume based Network slice converged charging with a single CCS.

Figure 6.x.2.3.1: Volume based Network slice converged charging

The steps are according to TS 32.255 [x] clause 5.2.2.2.2 with the following additions:

0ch.	CCS ABMF creates a new account for NS-Tenant S-NSSAI on "NS_quota(volume)".

9ch-b1.	Same as 9ch-b TS 32.255 [x]

9ch-b2.	For the S-NSSAI received for the individual UE: the CHF checks whether the UE PDU session is authorized.

9ch-c.	CHF provides appropriate answer for the PDU session.

X.	SMF determines service data flows start which needs volume quota

Xch-a.	the SMF sends the request to the CHF, for units (Volume) to be granted for the service to start.

Xch-b.	Account, rating and reservation control volume quota for the individual UE also based on NS_quota(volume) NS-Tenant S-NSSAI account balance. If no sufficient funds in NS-Tenant's account, the CHF rejects the individual UE request with appropriate cause.

Editor's Note: there may be other reasons than "no sufficient funds" (e.g blocked by another PDU session…) with CHF behaviors different from rejection (e.g. quota reallocation): such scenarios are ffs.

Xch-c.	CHF provides appropriate answer for the individual UE.

End of changes

End of changes