3GPP TSG SA WG5 Meeting 136-e		TDoc <S5-212377>

- electronic meeting, online, 1 - 9 March 2021									revision of S5-212174

- Source:	Matrixx

- Title:	TR 32.847 – Solution for Key Issue #1 based on new charging service

- Document for:	Approval

Agenda Item:	7.5.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 32.847-v011 “Study on Charging Aspects for Network Slicing Phase 2”.

[2]	3GPP TR 23.700-40: "Study on enhancement of network slicing; Phase 2".

# 3	Rationale

In this contribution, a network slice quota management and enforcement provided by a new CHF solution which addresses key issue #1 of TR 32.847 [1] enforced by a new NF on network slice admission control for maximum number of UEs refer KI#1 of TR 23.700-40 [2].

The CHF/5G Converged Charging System functionalities will be enhanced for Network slicing:

- new NetworkSliceConvergedCharging service which will interact between the new NF and the CHF.

- enhancing quota management capabilities to maintain the quota "max. Nb of Reg UEs" as received during NS configuration [create, update] by OAM.

# 4	Detailed proposal

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 32.845: "Charging management; Study on charging aspects of network slicing”.

[3]	3GPP TS 28.202: "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[4]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2".

[5]	3GPP TR 23.700-40: “Study on enhancement of network slicing; Phase 2”.

[6]	GSMA 5GJA NG.116: "Generic Network Slice Template".

[7]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[8]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[x]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

Next change

### 6.1.2	Solution #1 New NetworkSliceConvergedCharging service

### 6.1.2.1	Procedure Description

This solution addresses the Key Issue #1 by considering the new 5GC NF which has been defined as Network Slice Admission Control Function (NSACF).  While NSACF counts all UE registrations in S-NSSAI as received from the AMF(s), the CHF/5G Converged Charging System functionalities provide the charging service and quota management capabilities to this NSACF.

During the network slice provisioning, the CHF, under the ConvergedCharging service, receives and stores the "max. Nb of Reg UEs" which is considered by CCS as initial value for the network slice quota (NS_quota). This NS_quota is used by the CCS for the Network Slice quota management.

A new NetworkSliceConvergedCharging service is introduced for this Network Slice quota management between the CHF and the NSACF.

Editor’s note: The details of NetworkSliceConvergedCharging Service is FFS.

During the UEs registration, the NSACF provides the network slice admission control for all AMF(s) by counting the number of registered UEs against quota granted by the CHF under NetworkSliceConvergedCharging service consumption by the NSACF.

The following figure 6.1.2.1-1 represents the high-evel entities involved in the solution.

Figure 6.1.2.1-1 High level functional solution overview

Slice requirements are obtained by CHF at NSI creation via the ‘service profile’ per TS 28.202 [3] (clause 5.1.3 "Network Slice Management charging information"), which includes setting per S-NSSAI the max Nb of Reg UEs as NS_quota”.

Editor’s note: Futher adjustments will be added once the corresponding implementation of NSACF has been consolidated in SA2.

Next change

### 6.1.2.2	Flow description

.

The figure 6.1.2.2-1 describes the high-level charging procedure for Network slice converged charging based on NS_quota with initial value, provided by the CHF to the NSACF during slice provisioning procedures.

Figure 6.1.2.2-1: NetworkSlice quota management - registration

The steps are with the following additions:

0ch. CHF gets per S-NSSAI the "max Nb of Reg UEs" obtained during NS creation  and sets this as initial value for NS_quota of Nb of Reg UEs.

0. The NSACF counts the Nb of Reg UEs and check against the initial value for NS_quota (max Nb of Reg UEs).

1. NS admission control for UEs registrations per TS 23.502 [x].

Xch-a: At least one S-NSSAI has reached the initial value for NS_quota of  Nb of Reg UEs.

Xch-b: the NSACF invokes the Nchf_NSConvergedCharging service for requesting more quota (i.e. NS_quota (max Nb of Reg UEs) of the exhausted S-NSSAI.

xch-c. The S-NSSAI for which the initial NS_quota is reached will be either granted with more quota or the request for more quota is indicated as rejected, based on CCS capabilities in collaboration with OAM.

xch-d. CHF provides response to NSACF with appropriate setting of for S-NSSAI(s) based on the status of step xch-c.

2. NS admission control for UEs registrations per TS 23.502 [x].

Editor’s note: Futher adjustments will be added once the corresponding implementation of NSACF has been approved in SA2.

End of change