# 3GPP TSG-SA5 Meeting #136-e	S5-212136

# electronic meeting, online, 1 March - 9 March 2021

# Source:	CATT

# Title:	pCR Update solution for Direct Discovery charging

# Document for:	Approval

# Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 32.846: “Study on charging aspects of Proximity-based Services in 5GS”.

[2]	3GPP TR 23.752: “Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)”.

3	Rationale

5GS support Direct Discovery procedure over NR based PC5 for commercial services and public safety service.

4	Detailed proposal

1st Modified Section

6.1.4.2	Solution #1.2: ProSe Direct Discovery charging over PC5 for Key issues #1.1

6.1.4.2.1	Consideration for ProSe Direct Discovery charging over PC5.

Regarding discovery/management to support public safety, pre-configured or provisioned information can be used for the ProSe Direct Discovery procedure as specified in TS 23.303 [9] clause 5.3.7 (Direct Discovery for Public Safety use). In 5GS, Direct Discovery for both commercial services and public safety service over NR based PC5 should be supported. When the UE decides that reporting criteria are met, according to the pre-configuration, the UE creates the corresponding usage information report to Network.

5G DDNMF is defined to manage the dynamic ProSe Direct Discovery. Functionalities of the 5G DDNMF and the interactions with the UEs are similar to that of the DDNMF defined in TS 23.303 [8].

The Charging Enablement Function (CEF) is defined in TS 28.201 [10], for ProSe converged charging the CEF could be a consumer of NF service provider for ProSe (e.g.,5G DDNMF) and charging (Nchf) service provider.

Besides the information list in Table 6.1.4.1.1-1, the 5GS should collect the following charging information:

| Information Element | Description |
| --- | --- |
| User Location Information | The location of the UE |
| UE identity | The identity of the ProSe UE |
| Serving PLMN ID | PLMN identity of the PLMN which signalled the carrier frequency |
| ProSe Relay UE ID | A link layer identifier that uniquely represents the ProSe UE-to-Network Relay UE |
| ProSe Target Layer-2 ID | The identifier of target UE, uniquely represents a specific one-to-one or one-to-many discovery/communication |
| ProSe Group IP multicast address | The IP multicast address to be used for performing ProSe direct discovery/communication. |
| Coverage Info | This IE provides information on the coverage status (i.e., whether the UE is served by NG-RAN or not) and the time when the coverage status changed to its current state. |
| Radio Resources indicator | This IE identifies whether the operator-provided radio resources or the configured radio resources were used for ProSe direct discovery /communication |
| Usage Data Container | This field holds the container associated to a trigger conditions (e.g. go out of coverage, come back to coverage, etc.) on a specific ProSe communication |

Table 6.1.4.2.1-1 Structure of the ProSe Discovery Information

Editor's note:	The information may also be used for Direct Communication, it is FFS for whether more information elements are needed.

End of Modified Sections