3GPP TSG SA WG5 Meeting 136-e		TDoc S5-212291

# electronic meeting, online, 1 - 9 March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0017 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements, use cases and services for C-PCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S5 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2021-02-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A complete solution for centralized PCI optimization is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition and changes of requirements, use cases and services for Centralized PCI optimization. The solution consists of existing Performance Managmenet and existing Trace services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Centralized PCI optimization is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | N | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication Management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS 28.532: "Management and orchestration; Generic management services"

[4]	3GPP TS 38.321 "NR; Medium Access Control (MAC) protocol specification".

[5]	3GPP TS 28.552 "Management and orchestration; 5G performance measurements".

[6] 	3GPP TS 32.511 " Telecommunication management; Automatic Neighbour Relation (ANR) management; Concepts and requirements".

[7]	3GPP TS 38.300 "NR; Overall description; Stage-2".

[8]	Void

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[10]	Void

[11]	3GPP TS 28.531 "Management and orchestration; Provisioning".

[12]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[13]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[14] 	Void

[15]	3GPP TS 32.508: "Telecommunication management; Procedure flows for multi-vendor plug-and-play eNodeB connection to the network".

[16]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[17]	Void

[Y]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management"

[X]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)"

Next change

#### 6.1.2.1	PCI configuration

REQ- CPCI-CONFIG-FUN-1 producer of provisioning MnS should have a capability allowing an authorized consumer to configure or re-configure the PCI value(s) for NR cell(s).

REQ- CPCI-CONFIG-FUN-2 producer of provisioning MnS should have a capability to notify the authorized consumer with the PCI value(s) being assigned to NR cell(s).

REQ-CPCI-CONFIG-FUN-3 producer of performance assurance MnS should have a capability to notify the authorized consumer about handover degradation which may be caused by PCI collision or PCI confusion problems for NR cells.

REQ-CPCI-CONFIG-FUN-X Trace data producer MnS should have a capability to supply the authorized consumer with data allowing it to detect PCI collision or PCI confusion problems for NR cells.

REQ-CPCI-CONFIG-FUN-Y Producer of performance assurance MnS should have a capability to notify the authorized consumer about handover improvement which is the result of a resolved PCI collision or PCI confusion problem for NR cells.

Next change

##### 6.4.2.1.2	PCI re-configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically re-configure the PCIs of NR cells, due to the PCI collision or PCI confusion problems. |  |
| Actors and Roles | C-SON to support PCI re-configuration. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS - The producer of fault supervision MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - The C-SON function has been in operation, and enabled. |  |
| Begins when | The C-SON function requests the producer of performance assurance MnS to collect handover performance related measurements reported by NG-RAN. |  |
| Step 1 (M) | Based on the measurements above, the C-SON function requests the producer of trace MnS to collect Radio Link Failure traces from UEs in cells where PCI collision or PCI confusion is suspected. |  |
| Step 2 (M) | The C-SON function analyses the PCI related information and detects that NR cells have experienced PCI conflict or confusion issues. |  |
| Step 3 (M) | The C-SON function determines the new PCI value(s) and requests the producer of provisioning MnS to re-configure the PCI value for the NR cell(s) experienced PCI conflict or confusion issues. |  |
| Step 4 (M) | The producer of provisioning MnS notifies the C-SON function about the resolution of PCI collision or PCI confusion problems for NR cell(s). |  |
| Step 5 (M) | The C-SON function requests the producer of performance assurance MnS to collect handover performance related measurements reported by NG-RAN in order to assess whether the PCI collision or confusion was corrected. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The PCI value of a NR cell has been selected. |  |
| Traceability | REQ-CPCI-CONFIG-FUN-1, REQ-CPCI-CONFIG-FUN-2, REQ-CPCI-CONFIG-FUN-3, REQ-CPCI-CONFIG-FUN-X, REQ-CPCI-CONFIG-FUN-Y |  |

Next change

#### 7.2.1.1	MnS component type A

Table 7.2.1.1-1: PCI type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation --- modifyMOIAttributes operation - - deleteMOI operation - - notifyMOIAttributeValueChanges operation - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations and notifications defined in clause 11.5.1 of TS 28.532 [3]: - establishStreamingConnection operation - terminateStreamingConnection operation - reportStreamData operation - addStream operation - deleteStream operation - getConnectionInfo operation - getStreamInfo operation |  |
| Operations defined in clauses 11.3.1.1.1 and 11.6.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - notifyFilePreparationError notification - subscribe operation - unsubscribe operation - listAvailableFiles operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.2.1.2	MnS Component Type B definition

##### 7.2.1.2.1	Control information

The parameter is used to control the C-SON PCI configuration function.

Table 7.2.1.2.1-1: PCI control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| PCI configuration control | This attribute allows authorized consumer to enable/disable the C-SON PCI configuration functionality. | disable, enable |

##### 7.2.1.2.2	Parameters to be updated

The table below lists the parameter related to the C-SON PCI configuration function.

Table 7.2.1.2.2-1: PCI update

| Updated parameters | Definition | Legal Values |
| --- | --- | --- |
| NR PCI | This parameter contains the PCI of the NR cell. | Integer |

#### 7.2.1.3	MnS Component Type C definition

##### 7.2.1.3.1	Notifications information

The table below lists the notifications related to PCI configuration are generated from the NR cells.

Table 7.2.1.3.1-1: PCI notification

| Notification information | Description | Note |
| --- | --- | --- |
| PCI collision notification | The collision notification is used to indicate two neighbouring cells are using the same PCIs. |  |
| PCI Confusion notification | The confusion notification is used to indicate that a serving cell has 2 neighbouring cells that are using the same PCI value. |  |

##### 7.2.1.3.2	Performance measurements

Performance measurements related to the PCI configuration are collected from the NR cells.

Table 7.2.1.3.2-1. PCI related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Excessive Radio Link Failure | The measurements can be used to discover cells with excessive Radio Link Failures (see TS 28.552 [5] clauses 5.1.1.6.1.8, 5.1.1.6.1.9, 5.1.1.6.2.1 and 5.1.1.6.2.2) |  |

##### 7.2.1.3.3	Trace Reporting

Trace information related to the PCI configuration are collected from NR cells.

| Trace Report | Description | Note |
| --- | --- | --- |
| RLF messages | Radio Link Failure messages are generated by UEs experiencing Radio Link Failures caused by PCI collision and PCI confusion (see TS 32.423 [Y] clause 4.30 and TS 38.423 [X] clause 8.4.7) |  |

End of changes