3GPP TSG-SA5 Meeting #136e	S5-212489

# E-Meeting, 1st – 9th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0315 | rev | 1 | Current version: | 16.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the Bindings for NSM Charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NSMCH |  |  |  |  |  | Date: |  |  | 2021-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Charging information for NSM Charging is specified in the TS 28.202. The corresponding Open API is defined in the TS 32.291, but the bindings for NSM charging is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the bindings for NSM Charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The bindings for NSM Charging is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-212171 |  |  |  |  |  |  |  |  |

First change

7.X	Bindings for NS Management charging

Table 7.X-1: Bindings of CDR field, Information Element and Resource Attribute for NS Management charging

| Information Element | CDR Field | Resource Attribute |
| --- | --- | --- |
|  |  | ChargingDataRequest |
| Tenant Identifier | Tenant Identifier | /tenantIdentifier |
| MnS Consumer Identifier | MnS Consumer Identifier | /mnSConsumerIdentifier |
| NSM Charging information | NSM Charging information | /nSMChargingInformation |
| Management operation | Management operation | /nSMChargingInformation/managementOperation |
| Identifier of NetworkSlice Instance | Identifier of NetworkSlice Instance | /nSMChargingInformation/idNetworkSliceInstance |
| List of Service profile charging information | List of Service profile charging information | /nSMChargingInformation/listOfserviceProfileChargingInformation |
| Service Profile Id | Service Profile Id | /nSMChargingInformation/serviceProfileIdentifier |
| S-NSSAIs List | S-NSSAIs List | /nSMChargingInformation/sNSSAIList |
| SST | SST | /nSMChargingInformation/sST |
| Latency | Latency | /nSMChargingInformation/latency |
| Availability | Availability | /nSMChargingInformation/availability |
| Resource Sharing Level | Resource Sharing Level | /nSMChargingInformation/resourceSharingLevel |
| Jitter | Jitter | /nSMChargingInformation/jitter |
| Reliability | Reliability | /nSMChargingInformation/reliability |
| Maximum Number of UEs | Maximum Number of UEs | /nSMChargingInformation/maxNumberofUEs |
| Coverage Area | Coverage Area | /nSMChargingInformation/coverageArea |
| UE Mobility Level | UE Mobility Level | /nSMChargingInformation/uEMobilityLevel |
| Delay Tolerance | Delay Tolerance | /nSMChargingInformation/delayToleranceIndicator |
| DL Throughput Per Slice | DL Throughput Per Slice | /nSMChargingInformation/dLThptPerSlice |
| DL Throughput Per UE | DL Throughput Per UE | /nSMChargingInformation/dLThptPerUE |
| UL Throughput Per Slice | UL Throughput Per Slice | /nSMChargingInformation/uLThptPerSlice |
| UL Throughput Per UE | UL Throughput Per UE | /nSMChargingInformation/uLThptPerUE |
| Max Number of PDU sessions | Max Number of PDU sessions | /nSMChargingInformation/maxNumberofPDUsessions |
| KPIs Monitoring list | KPIs Monitoring list | /nSMChargingInformation/kPIMonitoringList |
| Supported Access Technology | Supported Access Technology | /nSMChargingInformation/supportedAccessTechnology |
| V2X Communication Mode | V2X Communication Mode | /nSMChargingInformation/v2XCommunicationModeIndicator |
| Additional service profile charging information | Additional service profile charging information | /nSMChargingInformation/addServiceProfileInfo |
| Management operation status | Management operation status | /nSMChargingInformation/managementOperationStatus |
| Operational state | Operational state | /nSMChargingInformation/managementOperationalState |
| Administrative state | Administrative state | /nSMChargingInformation/managementAdministrativeState |
|  |  | ChargingDataResponse |
| - | - | - |

End of change