GPP TSG-SA5 Meeting #137-e	S5-213412

# Online, , 10th May 2021 - 19th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0508 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on mapping GST attributes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EMA5SLA |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Information in L.2 on mapping GST attributes is not up to date |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mapping fo GST attributes to SliceProfile and ServiceProfile is updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is inconsistent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | L.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

# L.2	GSMA GST, ServiceProfile and sliceProfile

The GSMA GST is used as the SLA information for the communication between the NSC (e.g. vertical industry) and the NSP. The SLA requirements can be fulfilled from management aspect and control aspect in a coordinated way. The SLS includes ServiceProfile information model.

As shown in figure L.2.1, the GST parameters [50] are translated and used as one important input to NRM ServiceProfile.  The ServiceProfile defines the wanted service requirement for a wanted service by the network slice consumer.  The ServiceProfile requirements are then translated to a Top SliceProfile (including specific TopSliceSubnetProfile attributes).  Based on Top SliceProfile requirements, the corresponding requirements for the dedicated domain specific network slice subnets in the RAN/TN/5GC are defined. For example, a 5GC SliceProfile (including specific CNSliceSubnetProfile attributes) is used to carry 5GC domain requirements, a NG-RAN SliceProfile  (including specific RANSliceSubnetProfile attributes) is used to carry NG-RAN domain requirements, and the TN requirements are derived and provide  input to TN domain. Some of the information in 5GC SliceProfile  and NG-RAN SliceProfile can be translated to configurable parameters  of network function for the control plane SLA support purpose.

As shown in Table L.2.1 in 5GC SliceProfile and NG-RAN SliceProfile parameters can be translated to configurable parameters related to network function behaviour for the control plane SLA support purpose. While other information (e.g delay tolerance, determistic communication support) in 5GC SliceProfile and NG-RAN SliceProfile are kept at OAM domain and is used to determine the overall behaviour of the network slice subnet.

The following table show the translation of GST attributes.

Table L.2.1: GST translation

| GST Attributes | ServiceProfile Parameter | SliceProfile Parameter | Configurable Parameter |
| --- | --- | --- | --- |
| Maximum number of UEs | maxNumberofUEs | maxNumberofUE | TBD |
| Maximum number of PDU sessions | maxNumberofPDUSessions | maxNumberofPDUSessions* | TBD |
| Downlink maximum throughput per UE | dLThptPerUE | dLThptPerUEPerSubnet | TBD |
| Uplink maximum throughput per UE | uLThptPerUE | uLThptPerUEPerSubnet | TBD |

*/ Available in Top SliceProfile and in 5GC SliceProfile

Editors note: The list of exact configurable parameters is to be revisted depending on the requirements from SA2 and RAN WGs.

NOTE:	Void.

Figure L.2.1 Relation between GSMA GST, ServiceProfile and SliceProfile

Next modified section