3GPP TSG SA WG5 Meeting 137-e		S5-213xxx

# electronic meeting, online, 10 - 19 May 2021

# Source:	Telefónica S.A.

# Title:	Add NG-RAN related management requirements

# Document for:	Approval

Agenda Item:	6.4.1

# 1	Decision/action requested

Discuss and approve on the proposal.

# 2	References

[1]	TS 28.557 Management of non-public networks; Stage 1 and stage 2 v0.4.1

[x]	TS 28.541 5G Network Resource Model (NRM); Stage 2 and stage 3 v17.2.1

# 3	Rationale

Different identifiers are used for SNPN and PNI-NPN:

- A SNPN is identified by the combination of a PLMN ID and a Network identifier (NID). A cell may support up to 12 NIDs. There can also be an optional human-readable network name per NID.

- For PNI-NPNs, a CAG is identified by a CAG identifier which is unique within the scope of a PLMN ID. A cell may have a list up to 12 CAG identifiers. A CAG cell may optionally have human-readable network name per CAG identifier.

A NG-RAN node can serve multiple NPNs, including SNPNs and PNI-NPNs. To that end, the NG-RAN node needs to be preconfigured (via OAM) with lists of NID(s) and CAG(s) it supports, for SNPN and PNI-NPN, respectively. NG-RAN node communicates this information to AMF in NG SETUP REQUEST and RAN CONFIGURATION UPDATE NG-AP messages. For NG-RAN split deployments, the situation is a bit different: which CAGs and NIDs are supported is pre-configured (via OAM) in the gNB-DU, however the gNB-CU still mantains the control over which CAGs/NIDs are currently active. The reason is that not all distributed gNB-DUs under the same gNB-CU may necessarily support all CAGs and NIDs. Therefore, exchange of this information over the F1 interface is needed:

- gNB-DU is preconfigured (via OAM) with list of NID(s) and CAG(s) it supports, for SNPN and PNI-NPN, respectively

- gNB-DU communicates this information to gNB-CU using F1 SETUP REQUEST and GNB-DU CONFIGURATION UPDATE message

- the gNB-CU may activate/deactive certain CAGs/NIDs, for which cells to be activated list in F1 Setup Response, gNB-CU Configuration Acknowledge and gNB-CU Configuration Update can be extended with this information.

This document proposes to capture the above in clause 4, NG-RAN related management requirements.

# 4				Detailed proposal

This document proposes the following changes in TS 28.557 [1].

1st Change

### 4.3.3	NG-RAN related management aspects

A NG-RAN node can serve multiple NPNs, including SNPNs and PNI-NPNs. To that end, the NPN-OP shall configure the NG-RAN node accordingly, using 3GPP management system.

For NG-RAN non-split deployments, the gNB needs to be pre-configured (via 3GPP management system) with lists of NID(s) and CAG(s) it supports, for SNPN and PNI-NPN, respectively. The gNB communicates this information to AMF in NG-AP messages.

Editor’s note: Whether modifications in NR NRM fragment [x] are needed or not is FFS

For NG-RAN split deployments, the situation is as follows: each gNB-DU is pre-configured (via 3GPP management system) with lists of NID(s) and CAG(s) it supports, for SNPN and PNI-NPN, respectively; however, it is the gNB-CU which maintains the control over which CAGs/NIDs are currently active. The reason is that not all distributed gNB-DUs under the same gNB-CU may necessarily support all CAGs and NIDs. Each gNB-DU communicates the information to gNB-CU in F1 messages.

Editor’s note: Whether modifications in NR NRM fragment [x] are needed or not is FFS

End of change