3GPP TSG-SA5 Meeting #137e	S5-213189

# 10 May to 19 May 2021, E-meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0302 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements related to data modification notification subscription for UDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | UDR provides data management services for the subscriber and application related data allowing to its consumer (e.g., UDM, PCF and NEF) to subscribe to data modification notifications in UDR. Users’ service may not be fulfilled due to a poorly performing (e.g. overloaded) data management service, therefore it is indispensable that the performance of data management services can be monitored. The use case for measurements related to data management service, including data modification notification subscription has been captured in innex A.88, and this CR is to add the measurements related to data modification notification subscription for UDR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the measurements related to data modification notification subscription for UDR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of data modification notification subscription for UDR cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.13.1.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

#### 5.13.1.z	Data modification notification subscription

##### 5.13.1.z.1	Number of data modification notification subscribing requests

a)	This measurement provides the number of data modification (including change on existing data record and addition of data record) notification subscribing requests received by the UDR.

b)	CC

c)	Receipt of an Nudr_DM_Subscribe request by the UDR from an NF service consumer (see 3GPP TS 23.502 [7]).

d)	An integer value

e)	DM.SubscribeReq

f)	UDRFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.13.1.z.2	Number of successful data modification notification subscribings

a)	This measurement provides the number of succesful data modification (including change on existing data record and addition of data record) notification subscribings at the UDR.

b)	CC

c)	Transmission of an Nudr_DM_Subscribe response by the UDR to an NF service consumer indicating a successful data modification notification subscribing (see 3GPP TS 29.504 [a]).

d)	An integer value

e)	DM.SubscribeSucc

f)	UDRFunction

g)	Valid for packet switched traffic

h)	5GS

##### 5.13.1.z.3	Number of failed data modification notification subscribings

a)	This measurement provides the number of failed data modification (including change on existing data record and addition of data record) notification subscribings at the UDR.

b)	CC

c)	Transmission of an Nudr_DM_Subscribe response by the UDR to an NF service consumer indicating a failed data modification notification subscribing (see 3GPP TS 29.504 [a]), each message increments the relevant subcounter per failure cause by 1.

d)	Each subcounter is an integer value

e)	DM.SubscribeFail.cause
Where cause indicates the failure cause of the data modification notification subscribing.

f)	UDRFunction

g)	Valid for packet switched traffic

h)	5GS

End of modified section