3GPP TSG-SA5 Meeting #138-e 	S5-214179

# e-meeting, 23 - 31 August 2021

# Source:	Huawei

# Title:	Add requirements for management of SNPN

# Document for:	Approval

Agenda Item:	6.4.1

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	TS 28.557 Management of non-public networks; Stage 1 and stage 2 v0.5.0

3	Rationale

To manage an SNPN which is a 5GS (i.e. NG-RAN and 5GC) that can be optionally complemented with other access networks based on non-3GPP technologies (i.e. IEEE Wi-Fi), the standalone SNPN management system needs a dedicated NPN identifier. The combination of a PLMN ID and Network identifier (NID) is used to identify an SNPN. See further details in clause 4.4 of TS 28.557 [1].

It is proposed to add corresponding requirements for management of SNPN in draft TS 28.557 [1].

4	Detailed proposal

This document proposes the following changes in TS 28.557 [1].

1st Change

5.2.2	Requirements for management of SNPN

REQ-SNPN-FUN-01 The 3GPP management system shall have the capability to support standalone operation of an SNPN.

REQ-SNPN-FUN-02 The 3GPP management system shall have the capability to support management of dedicated NPN identifier (i.e. combination of a PLMN ID and a Network identifier (NID) which is used to identify an SNPN.

REQ-SNPN-FUN-03 The 3GPP management system shall have the capability to configure NID which consists of an assignment mode and an NID value.

REQ-SNPN-FUN-0X The 3GPP management system shall have the capability to configure an SNPN cell with a list of NIDs per PLMN Identity.

REQ-SNPN-FUN-0Y The 3GPP management system shall have the capability to interwork with one or more non-3GPP management systems to support the operation of a SNPN which includes 3GPP and non-3GPP segments.

End of change