3GPP TSG-SA5 Meeting #138-e 	S5-214225

# e-meeting, 23 - 31 August 2021

# Source:	Huawei, China Mobile

# Title:	Living document for review of GSMA GST

# Document for:	Approval

Agenda Item:	6.4.2

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1] GSMA NG.116 - Generic Network Slice Template Version 5.0.

3	Rationale

As a living document, the table below has captured the agreements reached in SA5#137e.

It is proposed to make further update on the table below to reflect the new agreements reached in SA5#138e, if any.

4	Detailed proposal

This document proposes the following changes in this living document.

1st Change

Comparison table between 3GPP ServiceProfile attributes and GSMA GST attributes

In context of the network slice lifecycle, comparison table between 3GPP ServiceProfile attributes and GSMA GST attributes is listed in below table Annex A-1. The table can be extended to include more information for GST/NEST mapping.

Table Annex A-1: Comparison between 3GPP ServiceProfile attributes and GSMA GST attributes

| GST attribute name in GSMA NG.116 Generic Network Slice Template v5.0 | GST attribute definition in GSMA NG.116 Generic Network Slice Template v5.0 | ServiceProfile attribute in 3GPP TS 28.541 v17.3.0 | 3GPP SA5 comments | Potential related domains (5GC is 5G Core Network, NG-RAN is defined in TS 23.501, TN is Transport Network) |
| --- | --- | --- | --- | --- |
| Availability | (Communication service) availability: percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area, see also 3GPP TS 22.261. | availability | SA5 has sent LS (S5-196868) to GSMA to suggest considering the 3GPP definition of "communication service availability", see clause 3.1 in TS 22.261. 3GPP ServiceProfile already supports communication service availability. SA5#132e: 5GJA has sent reply LS (S5-203024) to SA5 and has approved the same definition according to SA5 suggestion. This attribute is updated in GST v3.0. | OAM NG-RAN 5GC TN |
| AreaOfService | This attribute specifies the area where the UEs can access a particular network slice. | coverageArea | SA5 has sent LS (S5-196868) to GSMA because SA5 suggests considering the generic based on geographical partitioning as the only approach since this approach is independent from the network deployment, and independent from deployment changes from network operator. SA5#132e: 5GJA has sent reply LS (S5-203024) to SA5. SA5 agrees with "The list of regions is specific for each country and the way to define these regions is the decision of the NSC and NSP" as stated in GSMA GST version 3.0. | OAM NG-RAN 5GC TN |
| Delay tolerance | Provide the NSC with service delivery flexibility, especially for the vertical services that are not chasing a high system performance. For instance, the service will be delivered once the mobile system has sufficient resources or during the off-peak hours. For this type of traffic, it is not too critical how long it takes to deliver the amount of data, e.g. within hours, days, weeks, etc. | delayTolerance |  | OAM NG-RAN 5GC TN |
| Deterministic communication | This attribute defines if the network slice supports deterministic communication for periodic user traffic. Periodic traffic refers to the type of traffic with periodic transmissions. Availability: This parameter describes if the network slice supports deterministic communication. Periodicity: This parameter provides a list of periodicities supported by the network slice. | deterministicComm |  | OAM NG-RAN 5GC TN |
| Downlink throughput per network slice | This attribute relates to the aggregated data rate in downlink for all UEs together in the network slice (this is not per UE). Guaranteed downlink throughput (O) Maximum downlink throughput (O) | dLThptPerSlice |  | OAM 5GC |
| Downlink maximum throughput per UE | This attribute describes the maximum data rate supported by the network slice per UE in downlink. This may be associated with a Service Category parameter. | dLThptPerUE |  | OAM NG-RAN 5GC TN |
| Energy efficiency | This attribute describes whether the network slice supports the energy efficiency KPI. Network slice energy efficiency KPI is defined in 3GPP Release 17 TS 28.554 Section 6.7. It is defined as a ratio between the performance of a network slice and its energy consumption. | energyEfficiency | SA5 has sent LS (S5-196868) to GSMA to highlight that, so far, the energy efficiency of a network slice is not defined. SA5#132e: 5GJA has sent reply LS (S5-203024) to SA5. In GST v3.0: This attribute is FFS in 3GPP Rel17. SA5 Rel-17 EE SI includes the network slice EE topic. | OAM |
| Group communication support | This parameter describes which type of group communication is provided by the network slice. | FFS |  |  |
| Isolation level | Isolation is one of the key expectations of network slicing. A network slice instance may be fully or partly, logically and/or physically, isolated from another network slice instance [24]. This attribute describes different types of isolation: • Physical – network slices are physically separated (e.g. different rack, different hardware, different location, etc.) • Logical – network slices are logically separated. | networkSliceSharingIndicator |  | OAM 5GC NG-RAN TN |
| Maximum supported packet size | This attribute describes the maximum packet size supported by the network slice and may be important for URLLC (Ultra-Reliable Low Latency Communication) and MIoT (Massive IoT) , or to indicate a supported maximum transmission unit (MTU). | maxPktSize |  | OAM 5GC NG-RAN |
| Mission critical support | Mission-critical (MC) leads to a priority of the network slice relative to others, for C-plane (Control Plane) and U-plane (User Plane) decisions. This is relative to a customer provider relationship and to a PLMN (Public land Mobile Network). | FFS |  |  |
| MMTel support | This attribute describes whether the network slice supports IP Multimedia Subsystem (IMS) and Multimedia Telephony Service MMTel. This parameter describes whether the GSMA PRD IR.92 [22] compliant MMTel deployment is supported in the network slice. | FFS |  |  |
| NB-IoT Support | This attribute describes whether NB-IoT is supported in the RAN in the network slice. | nBIoT |  | NG-RAN |
| Network Slice Customer network functions | A NSC can own some network functions. This attribute provides a list of network functions to be provided by the NSC. If the list is empty, or this attribute is not included, the NSC is not providing any network function relevant for the network slice. | FFS |  |  |
| Maximum number of PDU sessions | This attribute describes the maximum number of concurrent PDU supported by the network slice. | maxNumberofPDUSessions |  | OAM 5GC |
| Number of UEs | This attribute describes the maximum number of UEs that can use the network slice simultaneously. | maxNumberofUEs |  | OAM 5GC |
| Performance monitoring | This attribute provides the capability for NSC and NOP to monitor Key Quality Indicators (KQIs) and Key Performance Indicators (KPIs). KQIs reflect the end-to-end service performance and quality while KPIs reflect the performance of the network. Availability: This parameter contains a list of KQIs and KPIs available for monitoring. If the list is empty this attribute is not available in the network slice and the other parameters might be ignored. Monitoring sample frequency This parameter describes how often the KQIs and KPIs are monitored. | kPIMonitoring |  | OAM 5GC NG-RAN |
| Performance prediction | This attribute defines the capability to allow the mobile system to predict the network and service status. Predictive QoS (Quality of Service) can be done for various Key Quality Indicators (KQIs) and Key Performance Indicators (KPIs). KQIs reflect the end-to-end service performance and quality, while KPIs reflect the performance of the network. The prediction is done for a specific point of time in the future and for a specific geolocation. Note: Only the KQIs of communication services offered by the NSP can be predicted. For over the top services, the NSP is not able to access the KQIs. Availability: This parameter contains a list of KQIs and KPIs available for prediction. If the list is empty the attribute is not available in the network slice and the other parameters might be ignored. Prediction frequency: This parameter describes how often KQIs and KPIs prediction values are provided. | FFS | SA5 has sent LS (S5-196868) to GSMA to clarify what is the relationship between this attribute and service performance assurance? Need some clarification on the usage of this attribute. SA5#132e: 5GJA has sent reply LS (S5-203024) to SA5. [GSMA] System’s prediction capability is only one way to achieve service performance assurance, but not the only way, hence these two attributes may have some indirect dependency. If the service performance is not assured, for instance, the typical 2C services, the customer could still order prediction capability from the operator to understand where the bottleneck is, and if something could be done from application level in advance to avoid problem. | OAM |
| Positioning support | This attribute describes if the network slice provides geo-localization methods or supporting methods. Availability: This parameter describes if this attribute is provided by the network slice and contains a list of positioning methods provided by the slice. Prediction frequency: This parameter describes how often location information is provided. This parameter simply defines how often the customer is allowed to request location information. Accuracy: This parameter describes the accuracy of the location information. Accuracy depends on the respective positioning solution applied in the network slice. | positioning |  | OAM NG-RAN |
| Radio spectrum | Defines the radio spectrum supported by the network slice. This is important information, as some terminals might be restricted in terms of frequencies to be used. | FFS |  |  |
| Reliability | Editor's NOTE: This GST attribute has been deleted from GST 3.0 | FFS | SA5 has sent LS (S5-196868) to GSMA to suggest considering the 3GPP definition of "reliability", see clause 3.1 in TS 22.261. | OAM NG-RAN 5GC |
| Root cause investigation | Root cause investigation is the capability provided to NSC to understand or investigate the root cause of network service performance degradation or failure. | FFS |  |  |
| Session and Service Continuity support | The attribute defines the continuity of a Protocol Data Unit (PDU) session. The following three Session and Service Continuity (SSC) modes are specified [1]: • SSC mode 1 - the network preserves the connectivity service provided to the UE (the IP address is preserved) • SSC mode 2 - the network may release the connectivity service delivered to the UE and release the corresponding PDU Session (the network may release IP address(es) that had been allocated to the UE) • SSC mode 3 - changes to the user plane can be visible to the UE, while the network ensures that the UE suffers no loss of connectivity service (the IP address is not preserved in this mode when the PDU Session Anchor changes) • None – UE loses the connectivity service | FFS |  |  |
| Simultaneous use of the network slice | This attribute describes whether a network slice can be simultaneously used by a device together with other network slices and if so, with which other classes of network slices. | sliceSimultaneousUse | SA5 has sent LS (S5-196868) to GSMA to ask question: Is this an attribute related to UE capability to support network slicing or not? Need more clarification on the purpose and usage of this attribute. SA5#132e: 5GJA has sent reply LS (S5-203024) to SA5. [GSMA] This attribute describes whether a network slice can be simultaneously used by a device together with other network slices and, if so, with which other classes of network slices. The device mentioned above must support to use more than one network slice. NG.116 v3.0 will provide a better attribute description. | OAM 5GC NG-RAN |
| Slice quality of service parameters | This attribute defines all the QoS relevant parameters supported by the network slice. For some of these parameters 3GPP has already defined standard values [1]. By preselecting a 5G QoS Identifier (5QI) these parameters will automatically be filled out with the standardised values. Functional + operational - 3GPP 5QI - Resource Type - Priority level - Packet Delay Budget - Packet Error Rate - Jitter - Maximum Packet Loss Rate | jitter | 3GPP 5QI defined in TS 23.501 includes: - Resource Type - Priority level - Packet Delay Budget - Packet Error Rate Why list "- Resource Type - Priority level - Packet Delay Budget - Packet Error Rate" with 3GPP 5QI together? | OAM 5GC NG-RAN |
| Support for non-IP traffic | This attribute provides non-IP Session support (Ethernet session and forwarding support) of communication devices | FFS |  | OAM NG-RAN 5GC |
| Supported device velocity | Maximum speed supported by the network slice at which a defined QoS and seamless transfer between TRxPs (Transmission Reception Point(s)), which may belong to different deployment layers and/or radio access technologies (multi-layer /-RAT), can be achieved. | uESpeed |  | OAM NG-RAN 5GC |
| Synchronicity | This attribute provides synchronicity of communication devices. Two cases are most important in this context: • Synchronicity between a base station and a mobile device and • Synchronicity between mobile devices. Availability Accuracy | synchronicity | SA5 has sent LS (S5-196868) to GSMA to ask: Is this related to sync at the application level? SA5#132e: 5GJA has sent reply LS (S5-203024) to SA5. [GSMA] This is application requirement, but mainly aims to provide hint how system level should provide synchronicity. | OAM NG-RAN |
| UE density | This attribute describes the maximum number of connected and/or accessible devices per unit area (per km2) supported by the network slice. | termDensity |  | OAM NG-RAN |
| Uplink throughput per network slice | This attribute relates to the aggregated data rate in uplink for all UEs together in the network slice (this is not per UE). Guaranteed uplink throughput Maximum uplink throughput | uLThptPerSlice |  | OAM 5GC |
| Uplink maximum throughput per UE | Maximum uplink throughput This attribute describes the maximum data rate supported by the network slice per UE in uplink. These parameters could be used in order to offer different contract qualities like gold, silver and bronze. | uLThptPerUE |  | OAM NG-RAN 5GC TN |
| User management openness | This attribute describes the capability for the NSC to manage their users or groups of users’ network services and corresponding requirements. For instance, if NSC Y orders a network slice which is capable to support X users of Y, then Y should be capable to decide which X users could use this network slice. Hence, Y could manage the users, in terms of add, modify or delete users to receive network services provided by the specific network slice. | FFS |  | OAM |
| User data access | Data access: The attribute defines how the network slice (or mobile network) should handle the user data. Tunnelling mechanism The attribute defines the tunnelling mechanism; how the user data can be delivered to the external private data network. | FFS | SA5 has sent LS (S5-196868) to GSMA to ask: Need clarification on the name of the attribution. What is the purpose of this attribute? SA5#132e: 5GJA has sent reply LS (S5-203024) to SA5. [GSMA] This attribute specifies where the customer data will terminate. For example, eMBB slice all the data will go directly to the internet. Some of the NSCs may require terminating all the data in their private company network and in this case UPF needs to support some tunnelling mechanism. The third option is that the data will be exchanged only between the devices that have access to the given slice. | OAM 5GC |
| V2X communication mode | This parameter describes if the V2X communication mode is supported by the network slice. | FFS | SA5 has sent LS (S5-196868) to GSMA to suggest considering 5G only for slicing, no EUTRA for V2X. SA5#132e: 5GJA has sent reply LS (S5-203024) to SA5. [GSMA] 5GJA took the suggestion into consideration and prepared a change request based on the suggestion. | OAM NG-RAN 5GC |
| Latency from (last) UFP to Application Server | This optional attribute specifies maximum or worst case one-way latency between UPF and application server offered by the slice. This does not include latency introduced by the application server. In the case of chained UPFs, this refers to the last UPF (in the chain) towards the application server. This attribute extends what is covered by the 3GPP QoS PDB attribute (see GST QoS attribute) which is only between UE and UPF. This is an optional attribute for network slices that offer latency objectives between UPF and application server residing within the operator network. | FFS |  | OAM 5GC |
| Network Slice Specific Authentication and Authorization (NSSAA) Required | This attribute specifies whether for the Network Slice, devices need to be also authenticated and authorized by a AAA server using additional credentials different than the ones used for the primary authentication (see Rel-16 of 3GPP TS 23.501 [1] clause 5.15.10 for a definition of the Network Slice Specific Authentication and Authorization feature). | FFS |  | OAM 5GC |
| Multimedia Priority Service | This parameter specifies whether Multimedia Priority Service (MPS) is supported. | FFS |  | OAM NG-RAN 5GC |
| Supported data network | This attribute describes the data networks the network slice provides access to. | FFS |  | OAM NG-RAN 5GC |

Editor's NOTE:

For the column "Potential related domains" in the table, the content of this column will be revisited according to the Rel-17 agreement, especially according to the sliceProfile NRM conclusion which shows the domains info concretely.

End of change