# 3GPP TSG SA WG5 Meeting #138e	S5-214459

# Online, , 23 Aug 2021- 31 Aug 2021

# Source:	Samsung

# Title:	pCR 28.538 EAS Lifecycle Management usecase and requirements

# Document for:	Approval

Agenda Item:	6.4.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

The lifecycle management of EAS is an integral part of edge computing. This contribution provides the use case and requirements for the same.

# 4	Detailed proposal

First modification

# A	Edge Computing Management Capabilities

## A.1	Lifecycle Management

### A.1.1	Description

The lifecycle management of the edge components is to be enabled by the 3GPP Management System. The lifecycle management includes instantiation, termination and scaling of the edge components.

### A.1.2	EAS Lifecycle Management

This use case pertains to the case when the EAS is to be deployed on the operators EDN being managed by the operator’s management system. The lifecycle of EAS (instantiation, termination, scaling, etc) need to be managed.

There can be multiple Edge Data Network (EDN) present/serving a particular edge location. This makes it critical for application service provider to have their EAS deployed at appropriate EDN(s) to provide high performance services for the UE. This use case enables deployment of an EAS in a particular EDN, or in a set of EDNs, as per the EAS requirements. A consumer request for EAS(s) instantiation providing EAS requirements including (not limited to) deployment requirements, QoS requirements, service areas, affinity/anti-affinity with other EAS etc. The 3GPP management system determines in which EDN the EAS(s) will be deployed, based on the deployment requirements. The interactions with the NFV MANO will be performed to allocate appropriate virtual resources for the EAS VNF.  Post instantiation, the 3GPP management system notifies the consumer that the EAS(s) has been instantiated successfully.

This use case also enables consumer to request for termination of the EAS(s) in a particular EDN. A consumer consumes the management service of 3GPP management system to terminate the EAS with the EAS identifier(s). The 3GPP management system terminates the EAS VNF(s) based on the EAS identifier(s), and disconnects the EAS from the 5GC network functions. The 3GPP management system notifies the consumer that the EAS(s) has been terminated successfully.

### A.1.3 	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-EAS-DEPLOY-FUN-1 | 3GP3GPP management system should have the capability to instantiate the EAS, as per request from authorized consumers. | EAS Lifecycle Management |
| REQ-EAS-DEPLOY-FUN-2 | 3GPP management system should have the capability to deploy EAS at a suitable EDN which can support the EAS requirements e.g serving location, required latency, affinity/anti-affinity with other EAS, service continuity. | EAS Lifecycle Management |
| REQ-EAS-DEPLOY-FUN-3 | 3GPP management system should have the capability to send the notification indicating the status of EAS instantiation to the authorized subscribers. | EAS Lifecycle Management |
| REQ-EAS-DEPLOY-FUN-4 | 3GPP management system should have the capability to terminate the EAS with the EAS identifier, as per request from authorized consumers. | EAS Lifecycle Management |
| REQ-EAS-DEPLOY-FUN-5 | 3GPP management system should have the capability to send the notification indicating the status of EAS termination. | EAS Lifecycle Management |

End of First modification