3GPP TSG-SA5 Meeting #138-e 	S5-214034

# e-meeting, 23 - 31 August 2021

# Source:	3GPP RAN3

# Title:	LS on Report Amount for M4, M5, M6, M7 measurements

# Document for:	Information, Discussion

Agenda Item:  6.1

3GPP TSG-RAN WG3 Meeting #112-e	R3-212961

E-meeting, 17-28 May 2021

Title:	LS on Report Amount for M4, M5, M6, M7 measurements

Response to:

Release:	Rel-17

Work Item:	NR_ENDC_SON_MDT_enh-Core

Source:	RAN3

To:	RAN2, SA5

Cc:

Contact person:	Angelo Centonza

Angelo(dot)centonza(at)ericsson(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

The Report Amount IE instructs the RAN about the number of reports to be collected for a given MDT measurement. It is part of the M1 Configuration IE, that is signalled over a number of interfaces such as the NGAP (see definition from TS38.413 below)

9.3.1.171	M1 Configuration

This IE defines the parameters for M1 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M1 Reporting Trigger | M |  | ENUMERATED (periodic, A2event-triggered, A2event-triggered periodic, …) |  |
| M1 Threshold Event A2 | C-ifM1A2trigger |  |  |  |
| >CHOICE Threshold Type | M |  |  |  |
| >>RSRP |  |  |  |  |
| >>>Threshold RSRP | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| >>RSRQ |  |  |  |  |
| >>>Threshold RSRQ | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| >>SINR |  |  |  |  |
| >>>Threshold SINR | M |  | INTEGER (0..127) | This IE is defined in TS 38.331 [18]. |
| M1 Periodic Reporting | C-ifperiodicMDT |  |  |  |
| >Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60) | This IE is defined in TS 38.331 [18]. |
| >Report Amount | M |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity) | Number of reports. |

RAN3 has discussed that the Report Amount IE is not available for other MDT measurements such as M4, M5, M6, M7.

This leads to the problem of not being able to limit the number of measurement reports reported by the entity configured with the measurement. The latter is due to the lack of a mechanism to stop reports for a specific immediate MDT measurement.

Taking the D1 part of the M6 delay measurement as an example. RAN2 defines the following measurement configuration in TS38.331:

PeriodicalReportConfig ::=                  SEQUENCE {

rsType                                      NR-RS-Type,

reportInterval                              ReportInterval,

reportAmount                                ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity},

reportQuantityCell                          MeasReportQuantity,

maxReportCells                              INTEGER (1..maxCellReport),

reportQuantityRS-Indexes                    MeasReportQuantity                                             OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport                   INTEGER (1..maxNrofIndexesToReport)                            OPTIONAL,   -- Need R

includeBeamMeasurements                     BOOLEAN,

useWhiteCellList                            BOOLEAN,

…,

[[

measRSSI-ReportConfig-r16                   MeasRSSI-ReportConfig-r16                                      OPTIONAL,   -- Need R

includeCommonLocationInfo-r16               ENUMERATED {true}                                              OPTIONAL,   -- Need R

includeBT-Meas-r16                          SetupRelease {BT-NameList-r16}                                 OPTIONAL,   -- Need M

includeWLAN-Meas-r16                        SetupRelease {WLAN-NameList-r16}                               OPTIONAL,   -- Need M

includeSensor-Meas-r16                      SetupRelease {Sensor-NameList-r16}                             OPTIONAL,   -- Need M

ul-DelayValueConfig-r16                     SetupRelease { UL-DelayValueConfig-r16 }                       OPTIONAL,   -- Need M

reportAddNeighMeas-r16                      ENUMERATED {setup}                                             OPTIONAL    -- Need R

]]

}

As seen, the reportAmount is a mandatory field in RRC, so it needs to be set also for the ul-DelayValueConfig-r16 (namely D1).

RAN3 would like to check with RAN2 whether the above understanding is correct or not and whether it incurs in any drawbacks (given the absence of the Report Amount for M6 in RAN3 specifications). RAN3 would like to check with SA5 whether SA5 considers the report amount beneficial for M6 and other MDT measurements.

# 2	Actions

To RAN2 and to SA5

ACTION: 	RAN3 would like to ask RAN2 and SA5 feedback on the issues above.

# 3	Dates of next RAN3 meetings

RAN3 #113-e	16-26 Aug 2021	Electronic Meeting