# 3GPP TSG SA WG5 Meeting #139e	S5-215144

# Online, , 11 Oct 2021- 20 Oct 2021

# Source:	Samsung

# Title:	pCR 28.538 ECSFunction definition

# Document for:	Approval

Agenda Item:	6.4.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution defined the ECSFunction IOC.

# 4	Detailed proposal

First modification

# 6	Edge NRM

Editors Note: This section will contain the edge related NRM or just the reference to it in 28.541.

## 6.1		Information Model definitions for Edge NRM

### 6.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, Top | Top |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-2 Edge NRM containment/naming relationship

Figure 6.2.1-3 Transport view of EES NRM

Figure 6.2.1-4 Transport view of ECS NRM

### 6.2.2	Inheritance

Figure 6.2.2-1 Edge Inheritance Relationship

End of First modification

Second modification

## 6.3	Class definition

6.3.1		ECSFunction

6.3.1.1 		Definition

This IOC represent the properties of a ECS in a 3GPP network. For more information about ECS, see 3GPP TS 23.558.

6.3.1.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| eCSIdentifier | M | T | T | F | T |
| eDNConnectionInfo | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| eESFunctonRef | M | T | T | F | T |

#### 6.3.1.3	Attribute constraints

#### 6.3.1.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

6.3.3		EDNConnectionInfo <<datatype>>

6.3.3.1 		Definition

This datatype represent the EDN connection information.

6.3.3.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dNN | M | T | T | F | T |
| eESEndPoint | M | T | T | F | T |
| eDNServiceArea | M | T | T | F | T |

#### 6.3.3.3	Attribute constraints

None

#### 6.3.3.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

6.3.3		ServingLocation <<datatype>>

6.3.3.1 		Definition

This datatype represent the location which is to be served by the node.

6.3.3.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| geographicalLocation | CM | T | F | F | T |
| topologicalLocation | CM | T | F | F | T |

#### 6.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| geographicalLocation Support Qualifier | Condition: either geographicalLocation or topologicalLocation shall be present. |
| topologicalLocation Support Qualifier | Condition: either geographicalLocation or topologicalLocation shall be present. |

#### 6.3.3.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

6.3.5		GeoLoc <<datatype>>

6.3.5.1 		Definition

This datatype represent the geographical location.

6.3.5.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| geoPoint | CM | T | T | F | T |
| civicAddress | CM | T | T | F | T |

#### 6.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| geoPoint Support Qualifier | Condition: either geoPoint or civicAddress shall be present. |
| civicAddress Support Qualifier | Condition: either geoPoint or civicAddress shall be present. |

#### 6.3.5.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

6.3.6		GeoP <<datatype>>

6.3.6.1 		Definition

This datatype represent the geographical location.

6.3.6.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| lat | M | T | T | F | T |
| long | M | T | T | F | T |

#### 6.3.6.3	Attribute constraints

#### 6.3.6.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

6.3.7		TopoLoc <<datatype>>

6.3.7.1 		Definition

This datatype represent the topological location.

6.3.7.2		Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellID | O | T | T | F | T |
| tAI | O | T | T | F | T |
| pLMNID | O | T | T | F | T |

#### 6.3.7.3	Attribute constraints

#### 6.3.7.4	Notifications

The common notifications defined in subclause Y.3 are valid for this IOC, without exceptions or additions.

## 6.4	Attribute definition

6.4.1		Attribute Properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| eCSIdentifier | It identifies the ECS, see 3GPP TS 23.558. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eESFunctionRef | This is the DN of EESFunction. allowedValues: DN of the EESFunction MOI. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eDNConnectionInfo | It defines the set of information needed to connect to an EDN. | type: EDNConnectionInfo multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| dNN | The DNN used for EDN. | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eESEndPoint | The endpoint address (e.g. URI, IP address) of the EES | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| eDNServiceArea | This parameter defines the service location for the EDN. | type: ServingLocation multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| topologicalLocation | This refers to the Topological Service Area, see 3GPP TS 23.558. | type: TopoLoc multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| geographicalLocation | This refers to the Geographical Service Area, see 3GPP TS 23.558. | type: GeoLoc multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| geoPoint | This defines the location in terms of latitude and longitude. | type: geoP multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| lat | This defines the single latitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| long | This defines the single longtitude coordinate. | type: Float multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| civicAddress | This defines the location in terms of a civic address | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| cellID | The list of cell IDs defining the topological service area | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| tAI | The list of Tracking Area IDs defining the topological service area | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| pLMNID | The list of PLMN IDs defining the topological service area | type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: None isNullable: False |

End of Second modification

Third modification

# P.1	General

This annex contains the OpenAPI definition of the Edge NRM in YAML format.

The Information Service (IS) of the Edge NRM is defined in clause 6.

Mapping rules to produce the OpenAPI definition based on the IS are defined in TS 32.160 [10].

# P.2	Solution Set (SS) definitions

## P.2.1	OpenAPI document "edgeNrm.yml"

openapi: 3.0.1

info:

title: 3GPP Edge NRM

version: 17.1.0

description: >-

OAS 3.0.1 specification of the Edge NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.538; Edge NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.538/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

EDNConnectionInfo:

type: object

properties:

dNN:

Type: string

eESEndPoint:

Type: string

eDNServiceArea:

$ref: '#/components/schemas/ServingLocation'

ServingLocation:

type: object

properties:

geographicalLocation:

$ref: '#/components/schemas/GeoLoc'

topologicalLocation:

$ref: '#/components/schemas/TopoLoc'

GeoLoc:

type: object

properties:

geoPoint:

$ref: '#/components/schemas/GeoP'

civicAddress:

Type: String

GeoP:

type: object

properties:

lat:

type: float

long:

type: float

TopoLoc:

type: object

properties:

cellID:

Type: string

tAI:

Type: string

pLMNID:

Type: string

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

DNFunction:

$ref: '5GCNrm.yaml#/components/schemas/DNFunction'

ECSFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

eCSIdentifier:

Type: string

eDNConnectionInfo:

$ref: '#/components/schemas/EDNConnectionInfo'

eESFunctionRef:

Type: string

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

ECSFunction-Single:

type: array

items:

$ref: '#/components/schemas/ECSFunction-Single'

End of second modification