3GPP TSG-SA5 Meeting #139-e 	S5-215421

# e-meeting, 11 - 19 October 2021

# Source:	Ericsson, Deutsche Telekom

# Title:	Correction of figures in exposure scenarios

# Document for:	Approval

Agenda Item:	6.5.4

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	TS 28.530: "Management and orchestration; Concepts, use cases and requirements"

# 3	Rationale

Clause 4.1.1.3 describes scenarios for exposure to BSS. In these scenarios external and internal interfaces are identified. An external interface is an interface between two companies (legally separated entities) and an internal interface is internal to a company for example between two departments inside the same company.

The interface between a BSS of one company and a BSS of another company is an external interface. The interface between the BSS and Service Manager of the same company is an internal interface. The interface between a Vertical (company) and an CSP (company) is via the BSS of the CSP.

In scenario 1 and scenario 2 the service management layer is missing.

It is proposed to update the figures accordingly.

# 4	Detailed proposal

1st change

#### 4.1.1.2	Exposure of Management Services

Exposure of management services indicates the case that an external MnS consumer which is outside 3GPP management system can indirectly access management capabilities offered by an MnS producer within a 3GPP management system. In order to enable the exposure of eMnS, an external customer, which has external MnS consumer, has to sign a contract, which contains the agreement on what eMnS optionally under what condition can be exposed, with an MNO. The signing of the contract may need the interaction with BSS. The condition can be certain constraint of eMnS access based on the contract e.g., the access quota of certain eMnS, the access frequency of certain eMnS, etc.

#### 4.1.1.3	Exposure via BSS

##### 4.1.1.3.1	General

Exposure of service data to companies that are external to the operator are regulated by contracts. Different customers may have access to different management capabilities. It may differ on what attributes/policies/intents are allowed to be changed, in which value ranges changes are allowed and which performance metrics are allowed to be exposed.

The slice data with the ServiceProfile is located in the network manager.

Even if the external interface always goes via BSS, there are different scenarios for how a customer e.g., a vertical, can influence the ServiceProfile.

The capabilities required from a Vertical have to be the same on the interface between NOP Network manager and CSP Service manager. The external interface is external to the administrative domain of the operator, while the internal interface is inside the administrative domain of the operator.

##### 4.1.1.3.2	Exposure scenarios

Scenario 1 as shown in Figure 4.1.1.3.2-1: The company that plays the role of NOP may interface to an external CSP that are doing the service management and have a machine-to-machine interface between the CSP’s "service manager" and the NOP’s "service manager" via their respective BSS.

Figure 4.1.1.3.2-1 Service to network interface being an external interface

Scenario 2 as shown in Figure 4.1.1.3.2-2: The CSP might have a machine-to-machine interface towards their customers (e.g. a vertical) via their BSS. The CSP needs to have an interface towards NOP when NOP is different company. The NOP may have a machine-to-machine interface towards the CSP via their respective BSS.

Check if the OSS-OSS figure should come back here.

Figure 4.1.1.3.2-2 External customer interface to CSP

Scenario 3 as shown in Figure 4.1.1.3.2-3: The  CSP may have an interface to other company (external customer) that are buying services (packaged as products) and have machine-to-machine interface to the CSP’s "service manager" via BSS.

Figure 4.1.1.3.2-3 BSS to customer interface being an external interface

Considering scenario 1, 2, and 3 the needs of an external customer always have to be reflected on the internal interface between Network manager and Service manager no matter how many companies are in the chain between the NOP and the External customer. Actions from a Vertical must be possible on the interface between Network manager and Service manager.

The management capabilities that an external customer (e.g., Vertical) has access to via a BSS have to be supported on the internal interface. How the capabilties are exposed to an external customer is not subject to standardization in 3GPP.

End of changes