3GPP TSG-SA5 Meeting #139-e 	S5-215224

# e-meeting, 11 - 20 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | DraftCR | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 DraftCR 28.532 for MSAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MSAC |  |  |  |  |  | Date: |  |  | 2021-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to conclusion of TR 28.817, the generic management service should be updated to support authentication and authorization, the exact change will be decided in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new operation to support authentication capability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Access control for management service cannot be realized, that results in big security risk on 3GPP management system, hence imperil 3GPP networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modification

## 11.x	Access control service

### 11.x.1	Operations and notifications

#### 11.x.1.1	authentication operation (M)

##### 11.x.1.1.1	Definition

The operation used to authenticate a management service (MnS) consumer to the authentication service producer.

When receives authentication request, authentication service producer gets the identifier and credential of the MnS consumer, probably other context information (e.g. address of the client) from the request, and creates an authentication  session for the request.

Based on identifier in the request, authentication service producer gets identity information, e.g. status of the identity, associated group(s) of the identity, credential of the identity, etc., from local data store (e.g. a directory). Then the authentication producer authenticates the MnS consumer  by validating the identity information and other context (e.g. time, location of the MnS consumer) according to authentication policies (e.g. authentication factor, protocol, supported time, location, status of the MnS consumer, etc. ) associated to the group(s) the MnS consumer belongs to.

The authentication service producer updates the authentication status of the authentication session for the MnS consumer in the directory after authenticated the MnS consumer. If authenticated successfully and authentication assertion is supported by the MnS consumer, the authentication service producer constructs authentication assertion and may update the assertion of the authentication session for the MnS consumer in the directory.

If authenticate successfully and authentication assertion is supported by the protocol, the authentication service producer sends successful response with assertion in the authentication session to the consumer.

If authenticate successfully and authentication assertion is not supported by the protocol, the authentication service producer sends successful response without assertion in the authentication session to the consumer.

Note: The authentication assertion is not needed for some authentication protocols. E.g. SSH/TLS or other protocol used for Netconf/Resconf, instead of sending back assertion to client, server maintain the authenticated state of the client. On the other hand, assertion is supported for rest API based authenticaion protocol, such as OpenID, SAML, etc.

##### If fail for authentication, the authentication service producer sends failure response to the consumer, and delete the authentication session locally.

##### 11.x.1.1.2	Input parameters

| Parameter Name | S | Information type | Comment |
| --- | --- | --- | --- |
| identifier | M | Identity4AC.identifier | See attribute definition in generic NRM ( see [11] ) |
| credential | M | Identity4AC.credential | See attribute definition in generic NRM ( see [11] ). The credential in the authentication request may not be same as the credential stored in authentication service producer, but it is derived from the credential stored in authentication service producer according to corresponding protocol. |
| assClient | O | Identity4AC.AuthSes.assClient | See attribute definition in generic NRM ( see [11] ). It identifies (e.g. FQDN) an associated (machine) client/application which acting on behalf of a human MnS consumer during authentication. e.g. it could be address or hostname of digital portal, user agent, or mediated management function, etc. |

##### 11.x.1.1.3	Output parameters

| Parameter Name | S | Information / Information Type | Comment |
| --- | --- | --- | --- |
| identifier | M | Identity4AC.identifier | same to the identifier in the request. |
| status | M | ENUM (Success, Failure) | Represent authentication result. |
| sessionId | O | Identity4AC.AuthSes.sessionId | See attribute definition in generic NRM ( see [11] ). Session id of a successful authentication. |
| context | O | Identity4AC.AuthSes.context | See attribute definition in generic NRM ( see [11] ). It could be, e.g. term of validity of a successful authentication session |
| assertion | O | Identity4AC.AuthSes.assertion | See attribute definition in generic NRM ( see [11] ). It is included in a successful authentication session if the protocol support assertion. |

End of modification