3GPP TSG-SA5 Meeting #139-e 	S5-215227

# e-meeting, 11 - 20 October 2021

# Source:	Ericsson

# Title:	Discussion paper on charging roaming profile handling

# Document for:	Endorsement

Agenda Item:	7.3

# 1	Decision/action requested

Agree on a way forward for the charging roaming profile 4.2.

# 2	References

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles"

[2]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2"

[3]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)"

[4]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3"

[5]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description"

# 3	Rationale

The handling of triggers in the roaming profile is unclear.

# 4	Detailed proposal

## 4.1	Roaming Charging Profile

### 4.1.1	Current definitions

TS 32.255 [2] clause 5.2.1.2.2:

Two level of triggers can be supplied by the CHF:

-	Triggers associated to the PDU session.

-	Triggers associated to a rating group within the PDU session.

The set of triggers along with their category (i.e. immediate or deferred report) and level (i.e. per PDU session or per rating group), which can be supplied by the CHF to the SMF for 5G data connectivity converged charging or offline only charging are detailed in the sub-clause 5.2.1.4 for Flow Based Charging.

TS 32.255 [2] clause 5.2.1.2.2:

Two level of triggers can be supplied by the CHF:

-	Triggers associated to the PDU session.

-	Triggers associated to a QoS Flow within the PDU session.

The set of triggers along with their category (i.e. immediate or deferred report) and level (i.e. per PDU session or per QoS Flow), which can be supplied by the CHF to the SMF for 5G data connectivity converged charging are detailed in the sub-clause 5.2.1.6 for QBC.

When QBC is used in the context of roaming, the set of triggers, their associated category, and trigger thresholds, compose the "Roaming Charging Profile", which governs the SMF charging data generation, synchronously between the V-SMF and the H-SMF when shared.

TS 32.255 [2] clause 5.2.1.7:

When QoS flow Based Charging specified in 5.2.1.6 is used in a context of roaming, a "Roaming Charging Profile" is defined to allow, when shared, QBC synchronized between both PLMNs and includes:

-	The set of chargeable events as per Table 5.2.1.6.1 and associated category.

-	The set of thresholds for chargeable events based on trigger thresholds.

-	An indication on whether the "Default partial record" or the "Individual partial record" mechanism per clause 5.2.3, is used by CHF.

### 4.1.2	Analysis

The interaction between the PDU session level triggers in QBC and FBC are not described, this will give issues for the triggers: tariff time change, expiry of data time limit per PDU session, expiry of data volume limit per PDU session, expiry of data event limit per PDU session, and expiry of limit of number of charging condition changes.

There are six alternative ways to solve this:

1.	Use both FBC and QBC PDU session triggers

2.	Use only QBC PDU session triggers

3.	Use QBC if both FBC and QBC is provided for the same PDU session triggers

4.	Use only FBC PDU session triggers

5.	Use FBC if both FBC and QBC is provided for the same PDU session triggers

6.	Use whichever triggers first

For 1, using both will require support of reporting the FBC and QBC PDU session level triggers, this means that the FBC and QBC will have different invocations completely separated from each other. This would in essence mean that also the FBC and QBC flows would be separated which is a bit against the thoughts of converged charging.

For 2, this would allow the QBC to override all FBC provided (i.e., all FBC provided triggers would be ignored if QBC provides any trigger), since the triggers are supposed to be synchronized between with visited and the whole idea is to allow both home and visited to have the same type of records to ease comparison.

For 3, this would allow the QBC to override an FBC provided (i.e., an FBC provided triggers would only be used if QBC doesn’t provide the same trigger) would be used in some cases leading to that the home and visited wouldn’t have the same type of records.

For 4, this would allow the FBC to override all QBC provided (i.e., all QBC provided triggers would be ignored if FBC provides any trigger), since the QBC triggers are supposed to be synchronized between with visited this would mean that this no longer would apply.

For 5, this would allow the FBC to override an QBC provided (i.e., an QBC provided triggers would only be used if FBC doesn’t provide the same trigger), since the QBC triggers are supposed to be synchronized between with visited this would mean that this no longer would apply.

For 6, using the first to trigger, gives an unpredictability of if the QBC or FBC trigger is used further complicating any comparison of the home and visited records.

## 4.2	Conclusion

The best approach would be to allow the SMF to ignore all trigger received for FBC if there are at least one QBC provided trigger on PDU session, alternative 2. Since this is in line with the negotiation of triggers without the complexity of creating different flows for FBC and QBC.