3GPP TSG-SA5 Meeting #139-e 	S5-215252

# e-meeting, 11 - 20 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0587 | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update relationship between GST and Network Slice NRM fragment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Telefónica S.A. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EMA5SLA |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The SLA concept includes both technical requirements (i.e. SLS) + business requirements. The GST (and therefore ServiceProfile and the rest of derived constructions) does not capture business requirements, but only SLS. Therefore, the SLA wording need to be replaced by SLS. - The concepts of Top SliceSubnet, 5GC SliceProfile and NG-RAN SliceProfile are not defined in TS 28.541. They need to be replaced by TopSliceSubnetProfile, CNSliceSubnetProfile and RANSliceSubnetProfile constructions. - The concepts of parameter and attribute are wrongly used in Table L.21. GST is composed of parameters, while ServiceProfile/SliceProfile constructions consist of attributes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replace SLA by SLS throughout the entire annex. - Replace {Top SliceSubnet, 5GC SliceProfile, NG-RAN SliceProfile} with {TopSliceSubnetProfile, CNSliceSubnetProfile, RANSliceSubnetProfile} throughout the entire annex. - Update Figure L.2.1 to capture the two above changes. - Update Figure L.2.1 to clarify which SDO/fora is responsible to manage which information. - Correct the use of parameter/attribute wording in Table L.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May lead to wrong implementations, since workflow makes reference to concepts that are not defined in TS 28.541. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | L.1, L.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

Annex L (normative):
Relation of GSMA GST, ServiceProfile and SliceProfile

# L.1	General

This annex describes the relation between GSMA GST [50] and the ServiceProfile and SliceProfile captured in the network slice NRM fragment (c.f. clause 6)

2nd modified section

# L.2	GSMA GST, ServiceProfile and sliceProfile

The GSMA GST is used as the SLS information for the communication between the NSC (e.g., vertical industry) and the NSP. The SLS requirements can be fulfilled from management aspect and control aspect in a coordinated way. The SLS includes ServiceProfile information model.

As shown in figure L.2.1, the GST parameters [50] are translated into the ServiceProfile.  The ServiceProfile, which defines the service requirements associated to the NSC, is translated into SliceProfile. In particular, the attributes captured in ServiceProfile are  used as input to TopSliceSubnetProfile attributes.  Based on the TopSliceSubnetProfile attributes, the corresponding requirements for the dedicated domain specific network slice subnets in the RAN/TN/5GC are defined. For example, the CNSliceSubnetProfile attributes are used to carry 5GC domain requirements, the RANSliceSubnetProfile attributes are used to carry NG-RAN domain requirements, and the TN requirements are derived and provide  input to TN domain.

Figure L.2.1 Relation between GSMA GST and network slice NRM fragment

As shown in Table L.2.1 some of the attributes in CNSliceSubnetProfile and RANSliceSubnetProfile parameters can be translated to configurable parameters related to network function behaviour to satisfy SLS of the service in the control plane. While other information (e.g delay tolerance, determistic communication support) in CNSliceSubnetProfile and RANSliceSubnetProfile are kept at OAM domain and is used to determine the overall behaviour of the network slice.

The following table show the translation of GST attributes.

Table L.2.1: GST translation

| GST parameters | ServiceProfile attributes | TopSliceSubnetProfile attributes | RANSliceSubnetProfile attrbutes | CNSliceSubnetProfile attributes | Configuration parameters |
| --- | --- | --- | --- | --- | --- |
| Maximu m number of UEs | maxNumberofUE s | maxNumberofUE | maxNumberofUEs | maxNumberofUEs | attributes in NSACF |
| Maximu m number of PDU sessions | maxNumberofCo nns | maxNumberofPDUSes sions | N/A | maxNumberofPDUSes sions | TBD |
| Downlink maximu m throughp ut per UE | dLThptPerUE | dLThptPerUE | dLThptPerUEPerSu bnet | dLThptPerUEPerSubn et | TBD |
| Uplink maximu m throughp ut per UE | uLThptPerUE | uLThptPerUE | uLThptPerUEPerSu bnet | uLThptPerUEPerSubn et | TBD |

Editor's note:	The list of exact configurable parameters is to be revisited depending on the requirements from SA2 and RAN WGs.

NOTE:	Void.

End of modified section