3GPP TSG-SA5 Meeting #140-e 	S5-216145

# e-meeting, 15 - 24 November 2021 														Revision of S5-20xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0349 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of the 5G LAN service charging in Annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2021-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the support of 5G LAN service charging, the 5G LAN VN group management and communication charging should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Annex D for the support of 5G LAN service charging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of the 5G LAN is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex D(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

Annex D (normative):
5G LAN-type service Charging

D.1	General

This Annex specifies charging support of 5G LAN-type service.

D.2	Architecture considerations

D.2.1	User plane architecture to support 5G LAN-type service

The general User Plane architectures with the additional following options to support the 5G LAN-type service are described in this clause 4.4.6 of TS 23.501[200].

- Figure 4.4.6.1-1 depicts the non-roaming user plane architecture to support 5G LAN-type service using local switch.

- Figure 4.4.6.1-2 depicts the non-roaming user plane architecture to support 5G LAN-type service using N19 tunnel.

- Figure 4.1.1 depicts the non-roaming user plane architecture to support 5G LAN-type service using N6 tunnel.

D.2.2	5G LAN-type service converged charging architecture

D.2.2.1	Support of 5G LAN VN Group Communication

5G data connectivity domain converged charging architecture specified in the clause 4.2 is applicable for the 5G LAN VN Group Communication. The SMF embedding the CTF generates 5G LAN VN Group communication charging information towards the CHF based on the User plane architecture to support 5G LAN-type service.

D.2.2.2	Support of 5G LAN VN Group management

The architectural option via CEF for 5G LAN VN Group membership management converged charging are depicted in figure D.2.2.2-1.

Editor’s Note: The figure D.2.2.2-1 need to be added.

The architectural option via NEF for 5G LAN VN Group membership management converged charging is specified in the TS 32.254[x]. The NEF embedding the CTF, generates 5G LAN VN group management charging information towards the CHF.

D.3	5G LAN-type service charging principles and scenarios

Editor’s Note: The SMF related charging principles and scenarios is specified in the clause 5.1 and 5.2. The other charging principles and scenarios (e.g. CEF related) are specified in the Annex D.

D.3.1	5G data connectivity charging principles

D.3.2	5G data connectivity converged online and offline charging scenarios

D.4	Definition of charging information

Editor’s Note: The SMF related charging information is specified in the clause 5.1 and 5.2. The other charging information (e.g. CEF related) are specified in the Annex D.

D.4.1	Data description for 5G LAN-type service charging

D.4.2	5G LAN-type service charging specific parameters

End of change