3GPP TSG SA WG5 Meeting 140-e		TDoc S5-216358

# electronic meeting, online, 15 - 24 November 2021

# Source:	Ericsson

# Title:	pCR 28.316 Plug and Connect; Data formats; DHCP Requests

# Document for:	Approval

Agenda Item:	6.4.19

# 1	Decision/action requested

The meeting is asked to approve this contribution.

# 2	References

[1]	3GPP TS 28.316 v0.1.0 Management and orchestration; Plug and Connect; Data formats.

[2]	3GPP TS 32.160 Management and orchestration; Management service template.

# 3	Rationale

This contribution describes client identification in DHCP requests for TS 28.316 [1].

# 4	Detailed proposal

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.314: " Management and orchestration; Plug and Connect; Concepts and requirements”.

[3]	3GPP TS 28.315: "Management and orchestration; Plug and Connect; Procedure flows".

[4]	IETF RFC 3925: "Vendor-Identifying Vendor Options for Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[5]	IETF RFC 8415: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[6]	IETF RFC 2132: "DHCP Options and BOOTP Vendor Extensions".

[7]	IANA, "Private Enterprise Numbers", <http://www.iana.org/assignments/enterprise-numbers>.

Second change

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS28.314 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and in TS 28.314 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], TS 28.314 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and in TS 28.314 [2].

Third change

# 4	Data formats for Plug and Connect

## 4.1	Client identification in DHCP requests

### 4.1.1 	General

This clause describes DHCP options for use with DHCP for IPv4 (DHCPv4) and DHCP for IPv6 (DHCPv6) that NE uses to identify itself in DHCP requests in Plug and Connect (PnC) IP autoconfiguration service as outlined in TS 28.315 [3].

#### 4.1.1.1	DHCPv4

The NE performing the Initial IP Autoconfiguration procedure specified in clause 5.2 of 3GPP TS 28.315 [3], using IPv4 based network stack, may identifiy itself using Vendor Class Identifier DHCPv4 option 60 as specified in RFC 2132 [6]. The format of Vendor Class identifier shall follow the rules specified in clause 9.13 of RFC 2132 [6], which is illustrated in table 4.1.1.1.1:

Table 4.1.1.1.1: Format of Vendor Class Identifier

| Code | Length | Vendor Class Identifier |  |  |
| --- | --- | --- | --- | --- |
| 60 | n | i1 | i2 | … |
| octet | octet | octet | octet | octet |

The NE may identify itself as Mulit-Vendor Plug and Connect (MvPnC) compatible DHCPv4 client using DHCPv4 option 60 in the following way:

- DHCPv4 option code 60;

- Length 5 bytes;

- Vendor class identifier "MvPnC".

The use of Vendor Class Identifier DHCPv4 option 60 with specific value MvPnC is illustrated in table 4.1.1.1.2.

Table 4.1.1.1.2: Use of Vendor Class Identifier

| Code | Length | Vendor Class Identifier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 60 | 5 | M | v | P | n | C |

The NE may alternatively identify itself by using the Vendor-Identifying Vendor Class DHCPv4 option 124 as specified RFC 3925 [4]. This option contains one or more Vendor-Identifying Vendor class information each identified by Enterprise Number as registered with IANA [7]. The format of Vendor-Identifying Vendor Class shall follow the rules specified in clause 3 of RFC 3925 [4], which is illustrated in table 4.1.1.1.3:

Table 4.1.1.1.3: Format of Vendor-Identifying Vendor Class

| Code | Length 1-255 | Enterprise Number 1 | Vendor Class Data 1 |  | Enterprise Number 2 | Vendor Class Data 2 |  | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Data Length 1 | Opaque data 1 |  | Data Length 2 | Opaque data 2 | … |
| 124 | n | x | n | Data | y | n | Data | ... |
| octet | octet | 4 octets | octets | n octets | 4 octets | octet | n octets |  |

The NE may identify itself as MvPnC compatible DHCPv4 client using DHCPv4 option 124 in the following way:

- DHCPv4 option code 124;

- Vendor enterprise number 10415 for “3GPP”;

- Vendor class data length 5 bytes;

- Vendor class data "MvPnC".

The use of Vendor- Identifying Vendor Class DHCPv4 option 124 with specific value MvPnC is illustrated in table 4.1.1.2.4.

Table 4.1.1.1.4: Use of Vendor-Identifying Vendor Class

| Code | Length 1-255 | Enterprise Number 1 | Vendor Class Data 1 |  |  |  |  |  | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Data Length 1 | Opaque data 1 |  |  |  |  | … |
| 124 | n | 10415 | 5 | M | v | P | n | C | ... |
| octet | octet | 4 octets | octet | octet | octet | octet | octet | octet | … |

The order of vendor-identifying vendor class contained in option 124 does not matter, and any other vendor-identifying vendor class data with a different IANA enterprise number, if required by the vendor, may appear before or after the 3GPP vendor class.

#### 4.1.1.2	DHCPv6

The NE performing the Initial IP Autoconfiguration procedure specified in clause 5.2 of 3GPP TS 28.315 [3], using IPv6 based network stack, shall identify itself using the Vendor Class DHCPv6 option 16 as specified in RFC 8415 [5]. The format of Vendor Class shall follow the rules specified in clause 21.16 of RFC 8415 [5], which is illustrated in table 4.1.1.2.1:

Table 4.1.1.2.1: Format of Vendor Class

| Code | Length | Enterprise Number | Vendor Class Data |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vendor Class Length 1 | Opaque Data 1 | Vendor Class Length 2 | Opaque Data 2 | … |
| 16 | n | x | n | Data | n | Data | … |
| 2 octets | 2 octets | 4 octets | 2 octets | n octets | 2 octets | n octets | … |

The NE may identify itself as MvPnC compatible DHCPv6 client by using the Vendor Class DHCPv6 option 16 in the following way:

- DHCPv6 option code 16;

- Data length 11 bytes;

- Vendor enterprise number 10415 for “3GPP”

- Vendor class length 5 bytes;

- Vendor class data "MvPnC".

The use of Vendor Class DHCPv6 option 16 with specific value MvPnC is illustrated in table 4.1.1.2.1.

Table 4.1.1.2.2: Use of Vendor Class

| Code | Length | Enterprise Number | Vendor Class Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vendor Class Length 1 | Opaque data 1 |  |  |  |  |
| 16 | 11 | 10415 | 5 | M | v | P | n | C |
| 2 octets | 2 octets | 4 octets | 2 octets | octet | octet | octet | octet | octet |

End of changes