| 3GPP TR 28.825 V0.3.0 (2021-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on Management Aspects of 5G Network Sharing (FS_MANS) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Concepts and overview	7

5	Scenarios and key issues	7

5.1	Scenario #1 NG-RAN MOCN Network Sharing with same Cell Identity broadcast	7

5.1.1	Scenario description	7

5.1.2	Management requirements	7

5.1.3	Potential solution	8

5.2	Scenario #2 NG-RAN MOCN Network Sharing with multi Cell Identity broadcast	8

5.2.1	Scenario description	8

5.2.2	Management requirements	8

5.2.3	Potential solution	9

5.3	Scenario #3 NG-RAN MOCN Network Sharing	10

5.3.1	Scenario description	10

5.3.2	Management requirements	11

5.3.3	Potential solution	11

5.4	Scenario #4 MOCN NG-RAN sharing with multiple CellIds	13

5.4.1	Scenario description	13

5.4.2	Mangement requirements	13

5.4.3	Potential solution	14

6	Analysis and comparison of potential solutions	16

6.1	Analysis and comparison of potential solutions of enhancement of NR NRM to support NG-RAN sharing	16

7	Conclusions and recommendations	18

Annex X (informative): Change history	19

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This report is to study on enhancement of management aspects of 5G network sharing.

1	Scope

The present document studies on concept, use cases and potential requirements related to the management aspects of MOCN network sharing, and the potential solutions for 5G network sharing management architecture.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                           3GPP TS 32.130: " Telecommunication management; Network sharing; Concepts and requirements".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

MOCN                                 Multiple Operator Core Network

MOP                                     Master Operator

PLMN                                   Public Land Mobile Network

POP                                      Participating Operator

4	Concepts and overview

Network sharing is a mechanism to reduce network costs and to efficiently utilize network capacity, which is very important for network operators. The motivation for management aspects of network sharing is to provide the enhanced capabilities to suit the new demands from the sharing mechanism.

For network sharing, enhancing NR NRM for 5G MOCN network sharing scenarios has not reached a consensus yet. Judging from the current discussion of WI MANS, some scenarios and requirements may not be satisfied in this R17 stage, and it is uncertain whether these scenarios and requirements will be excluded in the future. The management requirements for MOCN with same and multiple Cell Identity broadcast scenarios are not clear either.

To solve the problem, this TR, is to identify and capture all the existing valuable inputs involving concept, use cases, potential requirements and NR NRM support of MOCN network sharing scenarios, as well as potential solutions for 5G network sharing management architecture.

5	Scenarios and key issues

5.1	Scenario #1 NG-RAN MOCN Network Sharing with same Cell Identity broadcast

5.1.1	Scenario description

For NG-RAN MOCN Network Sharing with same cell identity broadcast scenario, different POPs use the same NR Cell Identity but different PLMN Id, which is captured by following figure in TS 32.130 [1].

Figure 5.1.1-1 NG-RAN MOCN Network Sharing with same cell identity broadcast scenario

5.1.2	Management requirements

Req-MOCN_SameCellId_Cfg-CON-1. The 3GPP management system of the MOP shall have the capability to configure NgC and NgU individually for each POP.

Req-MOCN_SameCellId_Cfg-CON-2.  The 3GPP management system of the MOP shall have the capability to configure PLMNId individually for each POP.

Req-MOCN_SameCellId_Per-CON-3.   The 3GPP management system of the MOP shall have the capability to collect and report some measurements (e.g. active UEs measurements, packet delay measurements) in PLMN granularity for each POP.

5.1.3	Potential solution

1. For the Req-MOCN_SameCellId_Cfg-CON-1, individual EP_NgC instance and EP_NgU instance can be created and configured for each POP.

2. For the Req-MOCN_SameCellId_Cfg-CON-2, the attribute “PLMNInfoList” in NRCellDU IOC in existing NR NRM can be used to represent the POPs supported by the cell.

3. For the Req-MOCN_SameCellId_Per-CON-3, existing PLMN granularity subcounter can be used to present operator granularity measurements.

5.2	Scenario #2 NG-RAN MOCN Network Sharing with multi Cell Identity broadcast

5.2.1	Scenario description

For NG-RAN MOCN Network Sharing with multiple cell identity broadcast scenario, different POPs use different PLMN Id, NR Cell Identity and TAC, which is captured by following figure.

Figure 5.2.2-1 NG-RAN MOCN Network Sharing with multiple cell identity broadcast scenario

5.2.2	Management requirements

Req-MOCN-MultiCellId-Cfg-CON-1 The 3GPP management system of the MOP shall have the capability to configure NgC and NgU individually for each POP.

Req-MOCN-MultiCellId-Cfg-CON-2 The 3GPP management system of the MOP shall have the capability to configure PLMN-IdentityInfo (including PLMNId, NR Cell Identity, TAC) individually for each POP.

Req-MOCN-MultiCellId-Perf-CON-3 The 3GPP management system of the MOP shall have the capability to collect and report some measurements (e.g. active UEs measurements, packet delay measurements) in PLMN granularity for each POP.

Req-MOCN-MultiCellId-Cfg-CON-4 The 3GPP management system of the MOP shall have the capability to configure the common F1 interface for all POPs.

Req-MOCN-MultiCellId-Cfg-CON-5 The 3GPP management system of the MOP shall have the capability to configure the individual F1 interface for each POP.

Req-MOCN-MultiCellId-Cfg-CON-6 The 3GPP management system of the MOP shall have the capability to configure the individual NR cell relation individually for each POP.

5.2.3	Potential solution

1. For the Req-MOCN_MultiCellId_Cfg-CON-1, individual EP_NgC instance and EP_NgU instance can be created and configured for each POP.

2. For the Req-MOCN-MultiCellId-Cfg-CON-2, following enhanced NR NRM solution is proposed:

Introduce new OperatorDU <<IOC>> name-contained by GNBDUFunction<<IOC >> to represent operator specific gNB level information (including operator specific gNB Id, gNBIdlength), and new NROperatorCellDU<<IOC>> name-contained by OperatorDU<<IOC>> to represent operator specific cell level information (including operator specific nRTAC, CelllocalId, PLMNInfoList). Which means individual OperatorDU instance and NROperatorCellDU needs to be created and configured for each POP.

Figure 5.2.3-1 NR NRM enhancement to suport NG-RAN sharing MOCN network sharing with multiple cell identity broadcast feature

- OperatorDU<<IOC>>

The OperatorDU <<IOC>> contains attributes to support operator specific gNB level information (including gNB Id, gNBIdLength) to support 5G Multi-Operator Core Network (5G MOCN) network sharing with multiple cell identity broadcast feature. Individual instance of OperatorDU <<IOC>> should be created and configured for each POP. When configured the attributes override those in parent GNBDUFunction instance.

The OperatorDU<<IOC>> is only used to support MOCN with multiple cell identity broadcast feature. If the MOCN with multiple Cell Identity broadcast feature is not supported, this IOC is not used.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |

- NROperatorCellDU<<IOC>>

The NROperatorCellDU <<IOC>> contains attributes to support operator specific cell level information (including cellLocalId, pLMNInfoList, nRTAC) to support 5G Multi-Operator Core Network (5G MOCN) network sharing with multiple Cell Identity broadcast feature. Individual instance of NROperatorCellDU <<IOC>> should be created and configured for each POP. When configured the attributes override those in associated NRCellDU instance.

The NROperatorCellDU <<IOC>> is only used to support MOCN with multiple cell identity broadcast feature. If the MOCN with multiple cell identity broadcast feature is not supported, this IOC is not used.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| pLMNInfoList | M | T | T | F | T |
| nRTAC | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRCellDURef | M | T | T | F | T |

Following two updates are needed for GNBDUFunction IOC and NRCellDU IOC in the existing NR NRM:

-  The Support Qualifier for attribute "gNBId " and " gNBIdLength " in existing GNBDUFunction IOC needs to be changed to "CM", the condition is " If MOCN Network Sharing is configured the value is not used "

-	The Support Qualifier for attribute "cellLocalId " , "pLMNInfoList" and "nRTAC" in existing NRCelDU IOC needs to be changed to "CM", the condition is " If MOCN Network Sharing is configured the value is not used "

3. For the Req-MOCN-MultiCellId-Cfg-CON-3, Operator specific measurements can be collected and reported within OperatorDU<<IOC>> and NROperatorCellDU<<IOC>>, Operator agnostic measurements can be collected and reported within GNBDUFunction<<IOC>> and NRCellDU<<IOC>>.Editor’s Note: above potential solution need to be revisited according to further discussion based on other existing requirements or new requirements

4. For the Req-MOCN-MultiCellId-Cfg-CON-4, existing EP_F1C instance and EP_F1U instance name contained by GNBDUFunction IOC can be used to represent the common F1 interface configuration for all POPs.

5. For the Req-MOCN-MultiCellId-Cfg-CON-5, following NR NRM fragment needs to be added.

Figure 5.2.3-1 NRM for F1 related EPs to support individual F1 interface for NG-RAN MOCN network sharing with multiple cell identity broadcast scenario

For the scenario of individual F1 interface for NG-RAN MOCN network sharing with multiple Cell Identity broadcast, EP_F1C and EP_F1U name contained by OperatorDU can be used to represent individual F1 interface for each POP.

6. For the Req-MOCN-MultiCellId-Cfg-CON-6, individual NRCellRelation instances can be created and configured per POP.

5.3	Scenario #3 NG-RAN MOCN Network Sharing

5.3.1	Scenario description

NG-RAN MOCN Network Sharing scenario is captured by following figure in TS 32.130 [1]. This is to support network sharing scenarios for the exist NRM. This scenario the same with scenario#1 but with different requirements and potential solutions. In addition to being oriented to public users, 5G networks are also oriented to a variety of industry applications, and should be integrated into more flexible and cost-effective service methods.

Figure 5.3.1-1 NG-RAN MOCN Network Sharing scenario

5.3.2	Management requirements

Req-MOCN-CellId-Cfg-CON-1, the 3GPP management system of the MOP shall support to query and count all performance, load and otherstatistical indicators generated by physical and logical equipment network elements (including shared and non-shared gNB).

Req-MOCN-CellId-Cfg-CON-2, the 3GPP management system of the MOP and POP shall support to query all the alarm information generated by all physical equipment and logical network elements (including shared and non-shared gNBs).

5.3.3	Potential solution

To the Req-MOCN-CellId-Cfg-CON-1 and Req-MOCN-CellId-Cfg-CON-2 , according to the above two options, we propose a new option(figure below), which adds IOC LogicalDU, inherited from ManagedFunction IOC. The LogicalDU includes IOC LogicalCellDU. The arrow1 means a NRCellDU may associate to several NRLogicalCellDU. This can be created and configured specific for one operator (including MOP and POP). This solution combines option 1 and option 2 together, the achitecture looks more clear.

Figure 5.3.3-1 NR NRM enhancement to suport NG-RAN sharing MOCN network sharing

- New IOC LogicalDUFunction

The new IOC LogicalDUFunction at the same level as the original GNBDUFunction: split the gNBId and gNBIdLength of GNBDUFunction into LogicalDUFunction, and mark the original attribute as optional:

The original GNBDUFunction attribute list, updated with yellow as the conditional configuration, not configured when the network is shared.

| Attribute name | Support Qualifier |
| --- | --- |
| gNBDUId | M |
| gNBDUName | O |
| gNBId | CM |
| gNBIdLength | CM |
| rimRSReportConf | O |

For the new IOC LogicalDUFunction, the IOC exists in the network sharing scenario, otherwise it does not exist.

| Attribute name | Support Qualifier |
| --- | --- |
| gNBId | M |
| gNBIdLength | M |

- New IOC LogicalCellDU

The new IOC LogicalCellDU is at the same level as the original NRCellDU, and the two are associated through nRCellDuRef.

In the original NRCellDU attribute list, update the yellow item as the conditional configuration:

| Attribute name | Support Qualifier |
| --- | --- |
| cellLocalId | CM |
| operationalState | M |
| administrativeState | M |
| cellState | M |
| pLMNInfoList | CM |
| nRPCI | M |
| nRTAC | CM |
| arfcnDL | M |
| arfcnUL | CM |
| arfcnSUL | CM |
| bSChannelBwDL | M |
| ssbFrequency | CM |
| ssbPeriodicity | M |
| ssbSubCarrierSpacing | CM |
| ssbOffset | M |
| ssbDuration | M |
| bSChannelBwUL | CM |
| bSChannelBwSUL | CM |

The new IOC LogicalCellDU, the IOC exists in the network sharing scenario, otherwise it does not exist.

| Attribute name | Support Qualifier |
| --- | --- |
| cellLocalId | M |
| pLMNInfoList | M |
| nRTAC | CM |
| administrativeState | M |
| Attribute related to role |  |
| nRCellDURef | M |

5.4	Scenario #4 MOCN NG-RAN sharing with multiple CellIds

5.4.1	Scenario description

For MOCN NG-RAN sharing with multiple cell identity broadcast scenario, different POPs use different PLMN Id, NR Cell Identity and TAC. For this scenario, several requirements have been identified which are listed in clause 5.4.2. To fulfil these requirements, the enhancement of the NRMs is needed.

5.4.2	Mangement requirements

Req-MOCN-MultiCellId-Cfg-CON-1 The 3GPP management system shall have the capability to configure PLMN-IdentityInfo (including PLMNId, CellId, TAC) individually for each supported network operator.

Req-MOCN-MultiCellId-Perf-CON-2 The 3GPP management system shall have the capability to collect and report some measurements (e.g. active UEs measurements, packet delay measurements) in PLMN granularity for different network operator.

REQ-NS_NG-CON-1		The 3GPP management system of the MOP shall support a capability to manage both the non-shared network elements and shared network elements in a NG-RAN network at the same time.

REQ-NS_NG-CON-2		The 3GPP management system of the MOP shall support a capability to configure the NG-RAN network element to start the sharing of one currently non-shared NG-RAN network element or stop the sharing of one currently shared NG-RAN network element.

REQ-NS_NG-CON-3		The 3GPP management system of the MOP shall support a capability to configure the POP-specific attributes of the shared NG-RAN individually based on the POPs’ requirements.

5.4.3	Potential solution

To fulfil the requirements listed in clause 5.4.2, the following enhancements are proposed.

- Add new IOCs DUCommonPart and CellCommonPart

- DUCommonPart(O)

This IOC represents the common part of a shared DU.

Note: If NG-RAN sharing with multiple Cell Identities is not supported, then this IOC is not used.

The DUCommonPart IOC includes attributes inherited from TOP IOC  and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| masterPLMNId | M | T | T | F | T |
| sharedDUId | M | T | T | F | T |

- CellCommonPart (O)

This IOC represents the common part of a physical cell on a DU.

Note: If NG-RAN sharing with multiple Cell Identities is not supported, then this IOC is not used.

The CellCommonPart IOC includes attributes inherited from TOP IOC and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| physicalCellDUId | M | T | T | F | T |
| operationalState | M | T | F | F | T (see Note 2) |
| administrativeState | M | T | T | F | T (see Note 2) |
| cellState | M | T | F | F | T (see Note 2) |
| nRPCI | M | T | T | F | T |
| ssbFrequency | CM | T | T | F | T |
| ssbPeriodicity | M | T | T | F | T |
| ssbSubCarrierSpacing | CM | T | T | F | T |
| ssbOffset | M | T | T | F | T |
| ssbDuration | M | T | T | F | T |
| arfcnDL | M | T | T | F | T |
| arfcnUL | CM | T | T | F | T |
| arfcnSUL | CM | T | T | F | T |
| bSChannelBwDL | M | T | T | F | T |
| bSChannelBwUL | CM | T | T | F | T |
| bSChannelBwSUL | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRSectorCarrierRef | M | T | T | F | T |
| bWPRef | M | T | T | F | T |
| nRFrequencyRef | CO | T | T | F | T |
| victimSetRef | CM | T | T | F | T |
| aggressorSetRef | O | T | T | F | T |
| Note 1: No state propagation is implied. Note 2: The attribute value change is conveyed by the notifyStateChange notification. |  |  |  |  |  |

- Change support qualifier of existing IOC NRCellDU

Condition: NG-RAN sharing with multiple Cell Identities is not supported.

- Change the relationships of IOCs BWP and NRSectorCarrier for the MOCN NG-RAN sharing with multiple Cell-Ids scenario

6	Analysis and comparison of potential solutions

6.1	Analysis and comparison of potential solutions of enhancement of NR NRM to support NG-RAN sharing

For the enhancement of NR NRM to support NG-RAN sharing, several potential solutions have been discussed, which can be mainly classified into the following types:

Potential solution 1: Add new IOCs (i.e. DUCommonPart, CellCommonPart) to represent the common part and use the existing IOCs (i.e. GNBDUFunciton and NRCellDU) to represent the operator specific part, see clause 5.4.

Potential solution 2: Add new IOCs (i.e. OperatorDU, NROperatorCellDU) to represent the operator specific part and use the existing IOCs (i.e. GNBDUFunciton and NRCellDU) to represent the common part, the new added IOCs are name contained by GNBDUFunction; see clause 5.2.

Potential solution 3: Add new IOCs (i.e. LogicalDU, NRLogicalCellDU) to represent the operator specific part and use the existing IOCs (i.e. GNBDUFunciton and NRCellDU) to represent the common part, the new added IOCs are name contained by ManagedElement; see clause 5.3.

Based on the detail description of potential solutions listed above, the analysis and comparison is collected in the table below. In the table, the potential solution 2 and potential solution 3 are merged in one column because there are only small differences between these two solutions.

|  | Scenarios/Issues | Potential solution 1 | Potential solution 2/3 |
| --- | --- | --- | --- |
| 1 | - Non-sharing scenario; - RAN sharing without multi-CellIds scenario | NRM: no change F1: no change | NRM: no change F1: no change |
| 2 | RAN sharing with multi- CellIds – change on NRM(s) | - Add new IOCs: DUCommonPart, CellCommonPart; - Change support qualifier of existing IOC (NRCellDU: 18 attributes) Higher impact on existing NRM, and higher data migration required | - Add new IOCs: Solution 2: OperatorDU, NROperatorCellDU; Solution 3: LogicalDU, NRLogicalCellDU; - Change support qualifier of existing IOC (NRCellDU: 3 attributes, GNBDUFunction: 2 attributes) Lesser impact on existing NRM, and lesser data migration required |
| 3 | RAN sharing with multi- CellIds – change on F1 interface（F1-C and F1-U） | Pro: No change | 2 new F1 interfaces will be added. In the mixed network (i.e. some gNBs are shared with multi-CellIds, and the other gNBs are not), the gNBs not shared with multi-CellIds will use the F1 between GNBDUFunction and GNBCUCPFuntion/ GNBCUUPFunction, and the gNBs shared with multi-CellIds will use the F1 between OperatorDU/LogicalDU and GNBCUCPFuntion/ GNBCUUPFunction or both. |
| 4 | The relationship between GNBDUFunction/NRCellDU and BWP/NRSectorCarrier | In the mixed network (i.e. some gNBs are shared with multi-CellIds, and the other gNBs are not). the BWP/NRSectorCarrier will be name-contained by DUCommonPart, and referred by CellCommonPart. | Pro: No change. |
| 5 | management granularity on cell level | Have the capability to perform administrative management operation on operator specific cell level (e.g. block/unblock using administrativeState attribute) | impossible to perform administrative management operation on NROperatorCellDU/NRLogicalCellDU level (e.g. cannot block/unblock without administrativeState attribute) |
|  | Note: There is no requirement for administrative management operation on operator specific cell level in the present document. |  |  |
| 6 | Relationship of new added IOCs | Name-contianed by ManagedElement - Cannot get the relationship of the operator specific part and common part directly, the references need to be used. - In hot backup scenario, the POP specific part can be switched to the standby gNB (the common part) by modification of the reference, re-creation of the POP specific part is not needed. | Solution 2: OperatorDU is name-contained by GNBDUFunction - The relationship of the operator specific part and common part can be got directly - In hot backup scenario, all the OperatorDUs need to be deleted and re-created when the switch from active gNB to standby gNB is needed. Solution 3: LogicalDU is name-contained by ManagedElement Same as solution 1 |

7	Conclusions and recommendations

The following issues are identified in this study:

- Concept, several use cases and potential requirements (such as isolation requirement) related to the management aspects of MOCN network sharing

- Several potential solutions for 5G network sharing management architecture.

In conclusion, it is shown that this SI identifies and captures some existing valuable inputs involving concept, use cases, potential requirements and NR NRM support of MOCN network sharing scenarios, as well as potential solutions for 5G network sharing management architecture as required in objectives. There are four scenarios of MOCN network sharing with different potential requirements and solutions, and this can be adopted from one of them or combination of some of them as recommendations for the work item of MANS.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 09-2021 | SA5#138e | S5-214054 S5-214055 S5-214206 | - | - | - | Update to implement the agreed pCRs in SA5#138e: - S5-214054 Initial skeleton (v0.0.0) of TR 28.825 - S5-214055 Add structure for TR 28.825 - S5-214206 pCR TR 28.825 add NR Cell relation configuration requirements and corresponding solution for 5G MOCN network sharing with multi-Cell Id broadcast scenario | 0.1.0 |
| 10-2021 | SA5#139e | S5-215619 S5-215107 S5-215620 S5-215621 |  |  |  | Update to implement the agreed pCRs in SA5#139e: - S5-215619 pCR TR 28.825 Add scenario, requirements and potential solution for 5G MOCN network sharing - S5-215107 pCR TR 28.825 Add requirements and potential solution for F1 interface configuration for 5G MOCN network sharing with multiple Cell Id broadcast scenario - S5-215620 pCR TR 28.825 Add requirement and potential solution for NRM enhancement to support network sharing - S5-215621 pCR 28.825 Add potential solution for NRM enhancement to support NG-RAN sharing | 0.2.0 |
| 11-2021 | SA5#140e | S5-216199 S5-216200 S5-216201 S5-216202 S5-216203 S5-216627 |  |  |  | Update to implement the agreed pCRs in SA5#140e: - S5-216199 pCR 28.825 add reference and abbrevation - S5-216200 pCR 28.825 add concepts and overview - S5-216201 pCR 28.825 rapporteur's proposal - S5-216202 pCR 28.825 add conclusions and recommendations - S5-216203 pCR 28.825 cleanup - S5-216627 pCR 28.825 Add analysis and comparison of potential solutions | 0.3.0 |