3GPP TSG-SA5 Meeting #140-e 	S5-216614

# e-meeting, 15 - 24 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | - | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DAPS handover services and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | E_HOO |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add services and procedures in order to complete DAPS handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add services and procedures for DAPS handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.z, 7.1.z.1, 7.1.z.2, 7.1.z.2.1, 7.1.z.2.2, 7.1.z.3, 7.1.z.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This is input to the Rel-17 28.313 draft CR for WI E_HOO. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-216320 |  |  |  |  |  |  |  |  |

First change

### 7.1.z	MRO for DAPS handover

#### 7.1.z.1	MnS component type A

MRO for DAPS handover re-uses the component A for MRO, see clause 7.1.2.1.

#### 7.1.z.2	MnS Component Type B definition

##### 7.1.z.2.1	Control information

The parameter is used to control the DAPS handover function.

Table 7.1.z.2.1-1: MRO for DAPS handover control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| DAPS HO function control | This attribute allows the operator to enable/disable the DAPS HO functionality. See attribute dapsHoControl in TS 28.541 [13]. | Boolean On, off |

##### 7.1.z.2.2	Parameters to be updated

MRO for DAPS handover re-uses the same parameters to be updated as MRO, see clause 7.1.2.2.3.

#### 7.1.z.3	MnS Component Type C definition

##### 7.1.z.3.1	Performance measurements

Performance measurements related to MRO for DAPS handover are captured in Table 7.1.z.3.1.-1:

Table 7.1.z.3.1-1. MRO for DAPS handover related performance measurements

| Performance measurements | Description | Note |
| --- | --- | --- |
| Number of requested DAPS handover preparations | Counts the number of successful and unsuccessful inter-gNB DAPS handover preparations sent (see TS 28.552 clause 5.1.1.6.x.1) |  |
| Number of successful DAPS handover preparations | Counts the number of unsuccessful inter-gNB DAPS handover preparations sent (see TS 28.552 clause 5.1.1.6.x.2) |  |
| Number of failed DAPS handover preparations | Counts the number of unsuccessful inter-gNB DAPS handover preparations sent (see TS 28.552 clause 5.1.1.6.x.3) |  |
| Number of requested DAPS handover resource allocations | Counts the number of successful and unsuccessful inter-gNB DAPS handover preparations (see TS 28.552 clause 5.1.1.6.x.4) |  |
| Number of successful DAPS handover resource allocations | Counts the number of successful inter-gNB DAPS handover preparations (see TS 28.552 clause 5.1.1.6.x.5) |  |
| Number of failed DAPS handover resource allocations | Counts the number of unsuccessful inter-gNB DAPS handover preparations (see TS 28.552 clause 5.1.1.6.x.6) |  |
| Number of requested DAPS handover executions | Counts the number of outgoing inter-gNB DAPS handover candidates requested (see TS 28.552 clause 5.1.1.6.x.7) |  |
| Number of successful DAPS handover executions | Counts the number of successful inter-gNB DAPS handover executions received (see TS 28.552 clause 5.1.1.6.x.8) |  |
| Number of failed DAPS handover executions | Counts the the number of failed inter-gNB DAPS handover executions received (see TS 28.552 clause 5.1.1.6.x.9) |  |
| Number of DAPS handover requested | Counts the number of outgoing intra-gNB DAPS handovers requested (see TS 28.552 clause 5.1.1.6.y.1) |  |
| Number of successful DAPS handovers | Counts the number of successful intra-gNB DAPS handovers (see TS 28.552 clause 5.1.1.6.y.2) |  |
| Number of requested DAPS handover preparations | Counts the number of outgoing intra-gNB DAPS handover preparations requested, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.a) |  |
| Number of successful DAPS handover preparations | Countes the number of successful intra-gNB DAPS handover preparations, for a split gNB deployment (see TS 28.552 clause 5.1.3.7.1.b) |  |

End of changes