3GPP TSG-SA5 Meeting #140e	S5-216043

# e-meeting 15th - 24th  November 2021

# Source:	Nokia, Nokia Shanghai Bell, Huawei

# Title:	Extend Attributes of the Intent IOC

# Document for:	Approval

Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.5.0”.

# 3	Rationale

# This contribution proposes to Extend attributes of the Intent IOC  for an Intent-driven Management Service

.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

## 6.2	Information model definition for Intent (MnS component typeB)

### 6.2.1	Information model definition for Intent

#### 6.2.1.1	Class diagram

##### 6.2.1.1.1	Relationship

@startuml TS 28.541 figure 6.2.1-2 (as of MArch 2021)

' UML diagram for 3GPP TS 28.541 clause 6

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class ManagedEntity <<ProxyClass>>

class Intent <<InformationObjectClass>>

class intentExpectation <<dataType>>

class intentTarget <<dataType>>

class context<<dataType>>

class IntentReport <<DataType>>

ManagedEntity "1" *-- "*" Intent: <<names>>

ManagedEntity "1" *-- "*" IntentReport

Intent "1" *-r- "*" intentExpectation

Intent "1" *-d- "*" context

intentExpectation "1" *-r- "*" intentTarget

intentExpectation "1" *-d- "*" context

'ManagedEntity -d- IntentReport: <<FFS>>

Intent *-l- IntentReport

intentTarget "1" *-d- "*" context

note top of ManagedEntity

Represents the following IOCs:

Subnetwork or

Other ICs are FFS

end note

note "{xor}" as N1

(Intent, context) .. N1

N1 .. (intentTarget, context)

N1 .. (intentExpectation, context)

@enduml

Figure 6.2.1.1.1-1 Relationship UML diagram for intent

Editor’s Note: The detailed model for Intent, IntentReport and IntentExpectation objects (e.g. is it <<IOC>>, <<DataType>>, or string) is FFS as their relationship needs to be decided later based on the content of these three objects

##### 6.2.1.1.2	Inheritance

@startuml TS 28.541 figure 6.2.1-2 (as of MArch 2021)

' UML diagram for 3GPP TS 28.541 clause 6

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top <<InformationObjectClass>>

class Intent <<InformationObjectClass>>

'class intentExpectation <<dataType>>

'class Context <<dataType>>

'class IntentTarget <<dataType>>

Top <|-- Intent

'Top <|-- intentExpectation

@enduml

Figure 6.2.1.1.2-1 Inheritance UML diagram for generic intent

#### 6.2.1.2	Class definition

##### 6.2.1.2.1 	Intent <<IOC>>

###### 6.2.1.2.1.1	Definition

This IOC represents the properties of an Intent. The Intent IOC contains one or multiple intentExpectation(s) which includes MnS consumer’s requirements, goals and constraints given to a 3GPP system.

The Intent IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is “Intent” and the value of attribute objectInstance is the DN of the instance of Intent IOC.

###### 6.2.1.2.1.2	Attributes

The Intent includes attributes inherited from TOP IOC (defined in TS 28.622) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentExpectations | M | T | T | F | T |
| userLabel | M | T | T | F | T |
| IntentContexts | O | T | T | F | T |
| intentFulfilStatus | M | T | F | F | T |

Editor’s Note: whether other the attributes are needed for the Intent IOC needs further discussion.

###### 6.2.1.2.1.3	Attribute constraints

None

##### 6.2.1.2.2 	IntentExpectation

IntentExpectation class represent MnS consumer’s requirements, goals and constraints given to a 3GPP system.

Editor’s Note: more description for IntentExpectation will be added later based on the further discussion.

###### 6.2.1.2.2.1	Definition

IntentExpectation class represent MnS consumer’s requirements, goals and constraints given to a 3GPP system.

###### 6.2.1.2.2.2	Attributes

The IntentExpectation includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ExpectationId | M | T | T | T | T |
| ExpectationObjectType | M | T | T | F | T |
| ExpectationObjectContexts | CM | T | T | F | T |
| ExpetationTargets | CM | T | T | F | T |
| ExpetationContexts | O | T | T | F | T |

Editor’s Note: whether other the attributes are needed for the IntentExpectation IOC needs further discussion.

Editor’s Note: The naming of the terms may need further discussion.

Editor’s Note: whether both expectionObjectContexts and ExpetationTargets are mandatory is FFS.

###### 6.2.1.2.2.3	Attribute constraints

None

##### 6.2.1.2.3	IntentReport

###### 6.2.1.2.3.1	Definition

IntentReport class represent intent fulfilment feedback information that MnS consumer can obtained from a 3gpp system.

Editor’s Note: more description for IntentReport will be added later based on the further discussion.

###### 6.2.1.2.3.2	Attributes

TBD

###### 6.2.1.2.3.3	Attribute constraints

TBD

#### 6.2.1.3	DataType definition

##### 6.2.1.3.1 	expectationTarget <<dataType>>

###### 6.2.1.3.1.1	Definition

This <<dataType>> represents the  Targets of the IntentExpectation that are required to be achieved .

###### 6.2.1.3.1.2	Attributes

The expectationTarget includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetName | M | T | T | T | T |
| targetCondition | M | T | T | F | T |
| targetValueRange | M | T | T | F | T |
| targetContexts | O | T | T | F | T |

###### 6.2.1.3.1.3	Attribute constraints

None

##### 6.2.1.3.2 	context << dataType >>

###### 6.2.1.3.2.1	Definition

This IOC represents the properties of a context. A context describes the list of constraints and conditions that should evaluate to True when the targets are fulfilled but are themselves not to be enforced. The context may apply to the intent, the intent expectation, the intent targets or to the managed object as filter information used to identify the manged objects to which the targets are intended.

###### 6.2.1.3.2.2	Attributes

The context includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| contextAttribute | M | T | F | T | T |
| contextCondition | M | T | T | F | T |
| contextValueRange | M | T | T | F | T |

###### 6.2.1.3.2.3	Attribute constraints

None

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectation | It indicates the expectations including requirements, goals and constraints given to a 3GPP system. It states the list of specific outcomes desired to be realized for a specific type of managed object | type: FFS multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED Editor’s Note: whether other allowed values should be supported is FFS, and the name for the attribute intentFulfilStatus is FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentContext | It describes the list of constraints and conditions that should apply for the entire intent even if there may be specific constraints and conditions defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| intentExpectationIdentifier | A user-friendly (and user assignable) name of the intentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectionobjectType | expectionobjectType defines with an expectation, the type of object (e.g. type of managed object) to which the intentExpectation should apply. This means, which type of object the requirements specified by the expectation are meant for. expectionObjectType is mandatory. It is used together with the ExpectionObjectContext to identify the specific entity to which the intentExpectation should apply. E.g. the intentExpectation may be stated for a slice (type of Object) with identitier IIOT_Atomotive_2021 (identifier as context). Alternatively, the intentExpectation may be stated for a slice (type of Object) serving IIoT users (context 1) with slice profile supporting automotive connectivity (context 2). allowedValues: NA | type: string multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectionobjectContext | It describes the list of constraints and conditions to be used as filter information to identify the specific intentObject to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent , for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. E.g. the intentExpectation may be stated for a slice (type of Object) with identitier IIOT_Atomotive_2021 (identifier as context). Alternatively, the intentExpectation may be stated for a slice (type of Object) serving IIoT users (context 1) with slice profile supporting automotive connectivity (context 2). expectationObjectContext may also be used to select a specific instance of Object, i.e., given the type of object in "ExpectionObjectType", the object instance is identified using the identifier of the object instance under Object context e.g. to refer to cell instance number 234, we state the Object context as {cell_id, "=", 234}. The object identifier is in that case the context allowedValues: depends on Object in the IntentExpectation | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectionTargets | It describes the list of specific outcomes on metrics and observables related to the Object (e.g. the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s)) that are desired to be realized for a given intentExpectation. allowedValues: depends on Object in the IntentExpectation | type: expectationTarget multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectationContext | It describes the list of constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Object in the IntentExpectation | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| targetName | It describes a specific attribute of a managed object on which an outcomes may be stated, either a configuration or observable of that managed object. The attributes may be a parameter, gauge, counter, KPI, weighted metric, etc. related to that managed object allowedValues: depends on Object in the IntentExpectation | type: string multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| targetCondition | It expresses the limits within which the targetNameis allowed/supposed to be allowedValues: is equal to; is less than; is greater than; Note: Others conditions like "is within the range" or "is outside the range" can be expressed in terms of these basic conditions | type: enum multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the TargetCondition. | type: FFS multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| targetContext | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| contextType | Defines the roles for which a given context shall play. allowedValues: {"ExpectionObjectContext", "ExpectationContext", "TargetContext", "IntentContext"} | type: enum multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to a managed object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric,, etc) or an attribute related to the operating conditions of the managed object (szch as weather conditions, load conditions, etc). | type: DN multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: is equal to; is less than; is greater than; Note: Others conditions like "is within the range" or "is outside the range" can be expressed in terms of these basic conditions | type: enum multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |
| contextValueRange | It describes the range of values that explicatable to the ContextAttribute and the ContextCondition. | type: FFS multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: Null isNullable: True |

2nd Change

Annex <A> (informative):

Mapping the 3GPP and the TM Forum intentExpectation Models

The TM forum defines the structure of an intent as a list of expectations with each expectation containing the requirements goals and constraints to be achieved. The expectation is defined to contain 2 attributes - the imm:target and the imm:params. On the hand, the intentExpectation defined in 3GPP (see clause 6.2.1.2.2) contain more attributes some of which (the expectionObjectType, expectionObjectContexts, expectationTargets and expectationContexts)can be mapped to the TM Forum model.

Following the table to illustrate the attributes mapping between 3GPP Intent Expectation and TM Forum IntentExpectation

| 3GPP Intent Expectation | TM Forum Intent Expectation |
| --- | --- |
| Class Property | Attribute |
| expectionObjectType | imm:target |
| expectionObjectContexts | imm:params |
| expectationTargets |  |
| expectationContexts |  |

End of changes