3GPP TSG-SA5 Meeting #140e	S5-216448 (revision of S5-216043)

# e-meeting 15th - 24th  November 2021

# Source:	Nokia, Nokia Shanghai Bell, Huawei Asiainfo

# Title:	Extend Attributes of the Intent IOC

# Document for:	Approval

Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.5.0”.

# 3	Rationale

# This contribution proposes to Extend attributes of the Intent IOC  for an Intent-driven Management Service

.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

## 6.2	Information model definition for Intent (MnS component typeB)

### 6.2.1	Information model definition for Intent

Editor’s Note: The following information model needs to be revisited based on the further discussion, and the alignment/coordination work with other SDO needs to be considered, which may impact the following information model.

#### 6.2.1.1	Class diagram

##### 6.2.1.1.1	Relationship

@startuml TS 28.541 figure 6.2.1-2 (as of MArch 2021)

' UML diagram for 3GPP TS 28.541 clause 6

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class ManagedEntity <<ProxyClass>>

class Intent <<InformationObjectClass>>

class intentExpectation <<dataType>>

class intentTarget <<dataType>>

class context<<dataType>>

class IntentReport <<DataType>>

ManagedEntity "1" *-- "*" Intent: <<names>>

ManagedEntity "1" *-- "*" IntentReport

Intent "1" *-r- "*" intentExpectation

Intent "1" *-d- "*" context

intentExpectation "1" *-r- "*" intentTarget

intentExpectation "1" *-d- "*" context

'ManagedEntity -d- IntentReport: <<FFS>>

Intent -l-> IntentReport

intentTarget "1" *-d- "*" context

note top of ManagedEntity

Represents the following IOCs:

Subnetwork or

Other ICs are FFS

end note

note "{xor}" as N1

(Intent, context) .. N1

N1 .. (intentTarget, context)

N1 .. (intentExpectation, context)

@enduml

Figure 6.2.1.1.1-1 Relationship UML diagram for intent . Note: Above XOR means an instance of Context can only be name contained by either the Intent, IntentExpectation or IntentTarget.

Editor’s Note: The detailed model for Intent, IntentReport and IntentExpectation objects (e.g. is it <<IOC>>, <<DataType>>, or string) is FFS as their relationship needs to be decided later based on the content of these three objects

Editor’s Note: whether using the context or constraint is FFS, which needs to discuss together  with intent definition.”

“Editor’s Note: the relationship related to context in above figure need  further study

##### 6.2.1.1.2	Inheritance

@startuml TS 28.541 figure 6.2.1-2 (as of MArch 2021)

' UML diagram for 3GPP TS 28.541 clause 6

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top <<InformationObjectClass>>

class Intent <<InformationObjectClass>>

'class intentExpectation <<dataType>>

'class Context <<dataType>>

'class IntentTarget <<dataType>>

Top <|-- Intent

'Top <|-- intentExpectation

@enduml

Figure 6.2.1.1.2-1 Inheritance UML diagram for generic intent

#### 6.2.1.2	Class definition

##### 6.2.1.2.1 	Intent <<IOC>>

###### 6.2.1.2.1.1	Definition

This IOC represents the properties of an Intent. The Intent IOC contains one or multiple intentExpectation(s) which includes MnS consumer’s requirements, goals and constraints given to a 3GPP system.

The Intent IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is “Intent” and the value of attribute objectInstance is the DN of the instance of Intent IOC.

###### 6.2.1.2.1.2	Attributes

The Intent includes attributes inherited from TOP IOC (defined in TS 28.622) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentExpectations | M | T | T | F | T |
| userLabel | M | T | T | F | T |
| IntentContexts | O | T | T | F | T |
| intentFulfilStatus | M | T | F | F | T |

Editor’s Note: whether other the attributes are needed for the Intent IOC needs further discussion.

###### 6.2.1.2.1.3	Attribute constraints

None

##### 6.2.1.2.2 	IntentExpectation

IntentExpectation class represent MnS consumer’s requirements, goals and constraints given to a 3GPP system.

Editor’s Note: more description for IntentExpectation will be added later based on the further discussion.

###### 6.2.1.2.2.1	Definition

IntentExpectation class represent MnS consumer’s requirements, goals and constraints given to a 3GPP system.

###### 6.2.1.2.2.2	Attributes

The IntentExpectation includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ExpectationId | M | T | T | T | T |
| ExpectationObject | O | T | T | F | T |
| ExpetationTargets | M | T | T | F | T |
| ExpetationContexts | O | T | T | F | T |

Note: The use case specific Intent Expectations are defined utilizing the constructs of this generic IntentExpectation model.

Editor’s Note: whether other the attributes are needed for the IntentExpectation IOC needs further discussion.

Editor’s Note: The naming of the terms may need further discussion.

Editor’s Note: whether both expectionObjectContexts and ExpetationTargets are mandatory is FFS.

###### 6.2.1.2.2.3	Attribute constraints

None

#### 6.2.1.3	DataType definition

##### 6.2.1.3.1 	expectationTarget <<dataType>>

###### 6.2.1.3.1.1	Definition

This <<dataType>> represents the  Targets of the IntentExpectation that are required to be achieved .

###### 6.2.1.3.1.2	Attributes

The expectationTarget includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetName | M | T | T | T | T |
| targetCondition | M | T | T | F | T |
| targetValueRange | M | T | T | F | T |
| targetContexts | O | T | T | F | T |

###### 6.2.1.3.1.3	Attribute constraints

None

##### 6.2.1.3.2 	context << dataType >>

###### 6.2.1.3.2.1	Definition

This IOC represents the properties of a context. A context describes the list of constraints and conditions that should evaluate to True when the targets are fulfilled but are themselves not to be enforced. The context may apply to the intent expectation or to the intent targets as filter information used to identify the manged objects to which the targets are intended.

###### 6.2.1.3.2.2	Attributes

The context includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| contextAttribute | M | T | F | T | T |
| contextCondition | M | T | T | F | T |
| contextValueRange | M | T | T | F | T |

###### 6.2.1.3.2.3	Attribute constraints

None

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectation | It indicates the expectations including requirements, goals and context(including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for a certain object | type: FFS multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED Editor’s Note: whether other allowed values should be supported is FFS, and the name for the attribute intentFulfilStatus is FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentContext | It describes the list of constraints and conditions that should apply for the entire intent even if there may be specific constraints and conditions defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| intentExpectationId | A user-friendly (and user assignable) name of the intentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectionObject | expectation carries requirements (expectations, goals and constrains) on an expectationObject. expectionObject refers to an object (e.g. instance of managed object) to which the intentExpectation should apply. This means, which object the requirements specified by the expectation are meant for. expectionObject is optional as the intent expectation can be to create a new Object. allowedValues: NA | type: DN multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectionTargets | It describes the list of specific outcomes on metrics and observables related to the Object (e.g. the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s)) that are desired to be realized for a given intentExpectation. allowedValues: depends on Object in the IntentExpectation | type: expectationTarget multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectationContext | It describes the list of constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Object in the IntentExpectation | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| targetName | It describes a specific attribute of the object on which the outcomes are stated, either a configuration or observable of that object. The attributes may be a parameter, gauge, counter, KPI, weighted metric, etc. related to that object allowedValues: depends on Object in the IntentExpectation | type: string multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| targetCondition | It expresses the limits within which the targetNameis allowed/supposed to be allowedValues: is equal to; is less than; is greater than; Note: Others conditions like "is within the range" or "is outside the range" can be expressed in terms of these basic conditions | type: enum multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the TargetCondition. | type: FFS multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| targetContext | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric,, etc) to which the expection should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: DN multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: is equal to; is less than; is greater than; Note: Others conditions like "is within the range" or "is outside the range" can be expressed in terms of these basic conditions | type: enum multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |
| contextValueRange | It describes the range of values that explicatable to the ContextAttribute and the ContextCondition. | type: FFS multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: Null isNullable: True |

2nd Change

Annex <A> (informative):

Mapping the 3GPP and the TM Forum intentExpectation Models

The TM forum defines the structure of an intent as a list of expectations with each expectation containing the requirements goals and constraints to be achieved. The expectation is defined to contain 2 attributes - the imm:target and the imm:params. On the hand, the intentExpectation defined in 3GPP (see clause 6.2.1.2.2) contain more attributes some of which (the expectionObject, expectionObjectContexts, expectationTargets and expectationContexts)can be mapped to the TM Forum model.

Following the table to illustrate the attributes mapping between 3GPP Intent Expectation and TM Forum IntentExpectation

| 3GPP Intent Expectation | TM Forum Intent Expectation |
| --- | --- |
| Class Property | Attribute |
| expectionObject | imm:target |
| expectionObjectContexts | imm:params |
| expectationTargets |  |
| expectationContexts |  |

End of changes