3GPP TSG-SA5 Meeting #140-e 	S5-216360

# e-meeting, 15 - 24 November 2021

# Source:	AsiaInfo

# Title:	Add attributes of the IntentReport

# Document for:	Approval

Agenda Item:	6.4.10

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1] 3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.6.0”.

3	Rationale

This contribution proposes to add attributes of the IntentReport.

4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

6.2.1.1	Class diagram

6.2.1.1.1	Relationship

Figure 6.2.1.1.1-1 Relationship UML diagram for intent

Editor’s Note: The detailed model for Intent, IntentReport and IntentExpectation objects (e.g. is it <<IOC>>, <<DataType>>, or string) is FFS as their relationship needs to be decided later based on the content of these three objects 6.2.1.1.2	Inheritance

Figure 6.2.1.1.2-1 Inheritance UML diagram for intent

2nd Change

6.2.1.2	Class definition

6.2.1.2.3	IntentReport

6.2.1.2.3.1	Definition

IntentReport class represents intent fulfilment feedback information that MnS consumer can obtained from a 3GPP system.

Editor’s Note: more description for IntentReport will be added later based on the further discussion.

6.2.1.2.3.2	Attributes

The IntentReport includes attributes inherited from TOP IOC (defined in TS 28.622) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| userLabel | M | T | F | T | T |
| intentDN | M | T | F | T | T |
| Operationstatus | M | T | F | T | T |
| IntentExpectationReport | M | T | F | T | T |

6.2.1.2.3.3	Attribute constraints

TBD

6.2.1.3.A	Operationstatus <<dataType>>

6.2.1.3.A.1	Definition

This <<dataType>> represents the properties of the created intent status (OperationSucceeded or OperationFailed) and the possible reasons for the OperationFailed status.

6.2.1.3.A.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Operationresult | M | T | T | F | T |
| reason | M | T | T | F | T |

6.2.1.3.A.3	Attribute constraints

None

6.2.1.3.B 	IntentExpectationReport <<dataType>>

6.2.1.3.B.1	Definition

This <<dataType>> represents the properties of the status for each intent IntentExpectation.

6.2.1.3.B.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| managedObject | M | T | T | F | T |
| TargetReport | M | T | T | F | T |

6.2.1.3.B.3	Attribute constraints

None

6.2.1.3.C 	TargetReport <<dataType>>

6.2.1.3.C.1	Definition

This <<dataType>> represents the properties of the status for each intent IntentTarget.

6.2.1.3.C.2	Attribute

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| FulfillStatus | M | T | T | F | T |
| reason | M | T | T | F | T |

6.2.1.3.C.3	Attribute constraints

None

3rd Change

6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectation | It indicates the expectations including requirements, goals and constraints given to a 3GPP system. The intentExpectation includes the intentAreas which describes a list of coverage areas for the intent to be applied. Editor’s Note: the detailed intentExpectation is for further discussion | type: FFS multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the current status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", "NOT_FULFILLED" Editor’s Note: whether other allowed values should be supported is FFS, and the name for the attribute intentFulfilStatus is FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentDN | DN of a referred Intent IOC | type: DN multiplicity: 0..1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| Operationstatus | It describes the created intent result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: " OperationSucceeded", " OperationFailed " | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| IntentExpectationReport | It indicates the reports for each IntentExpectation. | type: FFS multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TargetReport | It represents the status for each IntentTarget. It includes the intentFulfilStatus which describes the status for each target and the reason why the target cannot be fulfilled. | type: FFS multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reason | It represents the reason when the intentFulfilStatus is NOT_FULFILLED. It describes why the target cannot be fulfilled. | type: String multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: True |

End of changes