3GPP TSG-SA5 Meeting #140-e 	S5-216470

# e-meeting, 15 - 24 November 2021                                                Revision of S5-216268

# Source:	China Mobile, Nokia

# Title:	pCR 28.104 add inter-gNB beam selection optimization

# Document for:	Approval

Agenda Item:	6.4.18

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.809 “Management and orchestration; Study on enhancement of Management Data Analytics (MDA)”.

3	Rationale

This pCR is to add the description, use case and requirements of inter-gNB beam selection optimization, which is based on the clause 6.5.2 in TR 28.809[1].

4	Detailed proposal

1st Modified Section

7.2	MDA capabilities

7.2.Z	Inter-gNB Beam Selection Optimization

7.2.Z.1	Description

The MDA capability is about analytics of inter-gNB beam selection optimization.

7.2.Z.2	Use cases

With the deployment of 5G networks, Massive MIMO has been used on a large scale. Beamforming, as a key technology to reduce user interference, which can suppress interference signals in non-target directions and enhance sound signals in target directions, is always combined with Massive MIMO to further decrease interference. A cell can make use of multiple beams for serving residing users (SSB or CSI-RS) with each user served by a single beam at a time. The cell level quality can be represented as an aggregated metric over one or more beams. So, although handover is performed between two 5G cells, the granularity of handover can be further broken down to beam level.

The handover of beams could be performed if the network resource or the user's state have changed to obtain better network performance. Beam optimization includes the handover between different beams and configuration of beam parameters.

In order to avoid selecting the wrong beam to perform RACH on the target cell and causing RLF of the UE, MDA can be used to recommend a means to prioritize and/or select the beam in case of handover for a specific target cell. MDA can provide a beam level HO optimization analysis considering information on the handover performance of different beam combinations between the source and target cell pairs. Beams of the target cell with a successful handover are preferred in the selection.

MDA could also provide recommended actions and priority options for beam selection. Based on the recommended actions, the MDA MnS consumer adjusts the priorities for the beam selection at HO, i.e., the beam combinations that are likely to succeed are prioritized, less optimal beam combinations are down prioritized. The target cell may also obtain analytics to allocate RACH resources in a way that ensures HO success.

In order to optimize antenna and beam configuration, so as to reduce energy loss and enhance network performance, MDA can be used to analyze the current network status.

7.2.Z.3		Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-HO_BEAM_OPT_CON-1 | MDA for beam selection optimization shall be able to provide the analytics of the handover performance of beam pair combinations between cell pairs. | Inter-gNB Beam Selection Optimization |
| REQ-HO_BEAM_OPT_CON-2 | MDA for for beam selection optimization shall be able to provide an indication if a beam pair is to be prioritized or down prioritized | Inter-gNB Beam Selection Optimization |
| REQ-HO_BEAM_OPT_CON-3 | MDA for for beam selection optimization shall be able to provide feasible antenna and beam configuration analysis. | Inter-gNB Beam Selection Optimization |

End of Modified Sections