3GPP TSG-SA5 Meeting #141-e	S5-221040

# Online, , 17th Jan 2022 - 26th Jan 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0194 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data change notifications YANG-in-Rest format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-17 |  |  |  |  |  | Date: |  |  | 2022-01-05 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Define the mapping of the notifyMOIChanges to a "YANG-in-Rest" format to be used for reporting data changes of the YANG_Netconf solution set. This format uses the current Rest based notification with YANG based data in the path and value parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the mapping of the notifyMOIChanges to a "YANG-in-Rest" format. Some stage 2 changes also included to clarify deleting an attribute. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data change notifications are not available in the YANG_Netconf solution set. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.11.2, 12.1.3.2, 12.1.3.2.1, 12.1.3.2.2, 12.1.3.2.3, 12.1.3.2.4, 12.1.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See also S5-221039 as background information. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 11.1.1.11.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 11.1.1.7.2 | This parameter specifies the class name of the local root in the MIB |
| objectInstance | M | See clause 11.1.1.7.2 | This parameter specifies the instance of the local root in the MIB |
| notificationId | M | See clause 11.1.1.7.2 | See clause 11.1.1.7.2 |
| notificationType | M | const string “notifyMOIChanges” | See clause 11.1.1.7.2 |
| eventTime | M | See clause 11.1.1.7.2 | See clause 11.1.1.7.2 |
| systemDN | M | See clause 11.1.1.7.2 | See clause 11.1.1.7.2 |
| mOIChanges | M | SEQUENCE OF SET { notificationId (M), correlatedNotifications (O), additionalText (O), sourceIndicator (O), path (M), operation (M), value (CM) } | This parameter describes the NRM updates to be reported. The notificationId is an identifier of one MOI change. The path specifies the MOI created or deleted, or the MOI with replaced attribute values. The path may also identify an attribute or parts of an attribute in case the attribute is a structured data type. The operation specifies the type of operation that has been applied to the MOI specified by the path. It can have the values "CREATE", "DELETE" and "REPLACE". "CREATE" and "DELETE" refers to a MOI creation or deletion, respectively. "DELETE" may also refer to the deletion of an attribute or part of an attribute. "REPLACE" refers to the replacement of a complete attribute value of an existing MOI, or parts thereof in case the attribute is a structured data type. When a MOI creation is reported, the value carries an (optional) complete MOI representation. When a MOI deletion is reported, the value carries an (optional) complete MOI representation. When the replacement of the value of one or more attributes of a MOI is reported, the value carries the MOI representation without the attributes not changed. When the replacement of the value of a part of an attribute is reported, the value carries the new value of that part. When arrays are modified (by e.g. adding an array item, removing an array item or replacing an array item) the complete array shall be included in value. The reported MOI changes is an ordered list, since the creation of parent objects needs to be reported before the creation of child objects, and, vice versa, the deletion of child objects needs to be reported before the deletion of parent objects. |

Next change

#### 12.1.3.2	Mapping of notifications

##### 12.1.3.2.1	Introduction

The notifications notifyMOICreation, notifyMOIDeletion, notifyMOIAttributeValueChanges should not be used in the YANG_Netconf solution set as notifyMOIChanges provides the same functionality and more.

##### 12.1.3.2.2	Notification notifyMOICreation

The notification is not be mapped to the Netconf/YANG solution.

##### 12.1.3.2.3	Notification notifyMOIDeletion

The notification is not be mapped to the Netconf/YANG solution.

##### 12.1.3.2.4	Notification notifyMOIAttributeValueChange

The notification is not be mapped to the Netconf/YANG solution.

##### 12.1.3.2.5	Notification notifyMOIChanges

The YANG_Netconf solution set uses the same mapping as the RESTful HTTP-based solution set as described in clause 12.1.1.2.5 with the changes and additions described below.

Any changes reported are based on the YANG NRM definitions, even though the RESTful notification mapping is reused.

Attributes set using YANG default values, but not explicitly set by the consumer, are also reported. This may be the result of an MOI creation, or deletion of an attribute (part).

"value" shall be mapped to a JSON snippet according to RFC 7951.

Case 1: Creation of an MOI is reported with:

- operation: CREATE

- path: YANG Instance-identifier pointing to the list entry representing the MOI

- value: a complete MOI representation including the id attribute and any other attributes created starting with the "list" data node of the MOI encoded according to RFC7951.

For example, the following instance of a "moiChanges" array item reports an object creation:

{

"notificationId": 123456789,

"path": "/3gpp-common-managed-element:ManagedElement[id=\"node3\"]/3gpp-common-measurements:PerfMetricJob[id=\"job1\"]",

"operation": "CREATE",

"value": {

"_3gpp-common-measurements:PerfMetricJob": [

{

"id": "job1",

"attributes": {

"jobId": "9865",

"fileReportingPeriod": "30"

}

}

]

}

}

Case 2: Deletion of an MOI is reported with:

- operation: DELETE

- path: YANG Instance-identifier pointing to the list entry representing the MOI

- value: not present

For example, the following instance of a "moiChanges" array item reports an object deletion:

{

"notificationId": 123456789,

"path": "/3gpp-common-managed-element:ManagedElement[id=\"node3\"]/3gpp-common-measurements:PerfMetricJob[id=\"job1\"]",

"operation": "DELETE"

}

Case 3: Creation or replacement of the value of one or more (complete) attributes of a MOI is reported with:

- operation: REPLACE

- path: YANG Instance-identifier pointing to the list entry representing the MOI

- value: carries the MOI representation excluding unchanged attributes starting with "list" data node of the MOI encoded according to RFC7951.

  - When creating or changing a multivalue attribute is reported (represented either by a YANG list or leaf-list), e.g. adding, removing, replacing or moving individual items, the complete multivalue attribute shall be included in value. When changing a multivalue attribute, attribute values not listed in "value" are deleted.

For example, the following instance of a "moiChanges" array item reports creating the granularityPeriod attribute and changing the fileReportingPeriod attribute:

{

"notificationId": 123456789,

"path": "/3gpp-common-managed-element:ManagedElement[id=\"node3\"]/3gpp-common-measurements:PerfMetricJob[id=\"job1\"]",

"operation": "REPLACE",

"value": {

"_3gpp-common-measurements:PerfMetricJob": [

{

"id": "job1",

"attributes": {

"granularityPeriod": "30",

"fileReportingPeriod": "90"

}

}

]

}

}

Case 4:  Deleting a complete attribute is reported with:

- operation: DELETE

- path: YANG Instance-identifier pointing to the attribute

  - When deleting a multivalue attribute is reported (represented either by a YANG list or leaf-list), the last Xpath predicate(s) representing the leaf-list value or the key(s) to the list entries are omitted

- value: not present

For example, the following instance of a "moiChanges" array item reports deletion of all values in the objectInstances multivalue attribute:

{

"notificationId": 123456789,

"path": "/3gpp-common-managed-element:ManagedElement[id=\"node3\"]/3gpp-common-measurements:PerfMetricJob[id=\"job1\"]/attributes/objectInstances",

"operation": "DELETE"

}

Case 5: Creation or replacement of part of an attribute reported as follows (only used for structured data types represented by a list or container in YANG) :

- operation: REPLACE

- path: YANG Instance-identifier pointing to the attribute part. For multivalue attributes the individual value shall be addressed. (Other individual values are unchanged.)

- value: carries value of the attribute part.This value will replace the complete value pointed by path, which might result in deletion of some sub-parts.

For example, the following instance of a "moiChanges" array item reports the change of the "hysteresis" sub-attribute of the multivalue attribute "thresholdInfoList":

{

"notificationId": 123456789,

"path": "/3gpp-common-managed-element:ManagedElement[id=\"node3\"]/3gpp-common-measurements:ThresholdMonitor[id=\"job1\"]/attributes/thresholdInfoList[idx=\"thr1\"]/hysteresis ",

"operation": "REPLACE",

"value": "191"

}

Case 6: Deleting part of an attribute is reported as follows (only used for structured data types represented by a list or container in YANG):

- operation: DELETE

- path: YANG Instance-identifier pointing to the attribute or part of an attribute. For multivalue attributes the individual value shall be addressed. (Other individual values are unchanged.)

- value: not present

For example, the following instance of a "moiChanges" array item reports deletion of the "hysteresis" sub- attribute of the multivalue attribute "thresholdInfoList":

{

"notificationId": 123456789,

"path": "/3gpp-common-managed-element:ManagedElement[id=\"node3\"]/3gpp-common-measurements:ThresholdMonitor[id=\"job1\"]/attributes/thresholdInfoList[idx=\"thr1\"]/hysteresis ",

"operation": "DELETE"

}

End of  changes