3GPP TSG-SA5 Meeting #141-e 	S5-221138

# e-meeting, 17 -26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.130 | CR | 0018 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing use case and requirements for radio resources partitioning between POPs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S5 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei,China Unicom,China Telecom |  |  |  |  |  |  |  |  |  |
| Work item code: | MANS |  |  |  |  |  | Date: |  |  | 2022-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 28.541, the RRMPolicy is defined, in which the RRMPolicyMember include the PLMNId. So the defined RRMPolicy can be used for radio resource (e.g. PRB) between two POPs. In TS 32.130, the use case of Fully pooled radio resources between two POPs is described, however, the use case and requirements for radio resources partitioning between two POPs is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add use case and requirements for radio resources partitioning between two POPs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6, 5.4.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.6	Requirements for the OAM&P of shared NG-RAN

REQ-NS_NG-CON-1		The 3GPP management system of the MOP shall support a capability to manage both the non-shared network elements and shared network elements in a NG-RAN network at the same time.

REQ-NS_NG-CON-2		The 3GPP management system of the MOP shall support a capability to configure the NG-RAN network element to start the sharing of one currently non-shared NG-RAN network element or stop the sharing of one currently shared NG-RAN network element.

REQ-NS_NG-CON-3		The 3GPP management system of the MOP shall support a capability to configure the POP-specific attributes of the shared NG-RAN individually based on the POPs’ requirements.

REQ-NS_NG-CON-X		The 3GPP management system of the MOP shall have the capability to configure the radio resources partitioning policies for the POPs based on the agreement between POPs.

2nd Change

5.4.X	Radio resources partitioning between two POPs for the shared NG-RAN

In this use case, radio access network (i.e. one of multiple shared NG-RAN network elements) are shared between two POPs (POP A identified by PLMN A, POP B identified by PLMN B).

As agreed by POP A and POP B in their RAN sharing agreement:

- Radio resources (e.g. PRB, RRC connection, DRB) of the shared radio access network are partitioned for POP A and POP B, which means the radio resource partitioning policies (e.g. RRMPolicyRatio) of the radio access network is agreed for POP A and POP B. The detailed RRMPolicy definition for NG-RAN see the clause 4.3.36 of TS 28.541 [X]. Following is one example of RRMPolicy for corresponding POP A and POP B. In this example,

- POP A has 20% dedicated radio resources, 30% prioritized resources and 50% shared resources. In this case, there are 20% radio resources are dedicated for the POP A, 30% radio resources are guaranteed for POPA when it needs to use them, and 50% resources are shared with POP B.

- POP B has 30% prioritized resources and 20% shared resources. In this case, there are 30% radio resources are guaranteed for POP B when it needs to use them, and 20% resources are shared with POP A.

Figure 5.4.X-1 Example of structure RRMPolicy for radio access network resource partitioning for POP A and POP B

Note: If the network slicing feature is supported, the RRMPolicy is defined for each RRMPolicyMemberList which contains both PLMNId and S-NSSAI. In this case, the RRMPolicy is defined in RRMPolicyMember (combination of PLMNId and S-NSSAI) granularity.

MOP-NM (play the role of MnS consumer) obtains the radio resource partitioning policies the shared radio access network based on above agreement, and sends the radio resource partitioning policies to MOP-SR-DM (play the role of MnS prodcuer) to configure the shared NG-RAN NEs. MOP-SR-DM derives the radio resource partitioning policies for corresponding cells of each shared NG-RAN NE and configures the corresponding shared NG-RAN NE with corresponding radio resource partitioning policies for corresponding cells (see RRMPolicy defined in TS 28.541[X] which is contained by RRMPolicyManagedEntity).

MOP-SR-DM monitors the radio usage resources measurement (e.g. UL/DL PRB used for data traffic, RRC connections) for POP A and POP B, and send such radio usage resources measurements to MnS consumer.  Based on the radio resources usage measurements and other information (e.g. service traffic requirements changes) for POP A and POP B, the radio resources partitioning policies may be updated based on the agreement between POP A and POP B. The MOP-NM requests the MOP-SR-DM to reconfigure the updated radio resources partitioning policies of the shared radio access network for corresponding POP A and POP B.

End of Changes