3GPP TSG-SA5 Meeting #141-e	S5-221253

# Online, , 17th Jan 2022 - 26th Jan 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0102 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 28.531 Asynchronous network slicing procedures using createMOI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2022-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This change is to propose modifications for the network Slice CreateMOI operation in async mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the procedure for createMOI for slice creation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Discussion

This paper introduces asynchronous network Slice provisioning using createMOI operation.

Proposal

Figure 1: Network Slice provisioning process – asynchronous

The steps of the asynchronous Network Slice provisioning process using createMOI is shown in Figure 1:

- MnS consumer subscribes for Network Slice job modification and Network Slice creation/deletion/modification notifications with Network Slice Management Function.

- A Network Slice provisioning request is sent by the MnS consumer using NetworkSliceJob. The Job contains the serviceProfile and provisioningPriority and other attributes as mentioned in the “input parameters”.

The Network Slice Management Function receiving the Network Slice provisioning request creates the NetworkSliceJob and sends the provisioning jobId immediately.

- Based on the network slice related requirement (networkSliceSharingIndicator configuration in serviceProfile) and the knowledge of the capabilities of existing deployed network slices, and then decides whether to use an existing Network Slice or create a new Network Slice.

3a.1) If creating a new Network Slice, the Network Slice Management Function may invoke the network slice creation procedure.

3a.2) Then it may invoke the Network Slice Subnet create procedure and it associates the newly created Network Slice with Network Slice Subnet

3b.1) If using an existing Network Slice and the existing Network Slice needs to be modified to satisfy the network slice related requirements, the Network Slice Management Function invokes the procedure to modify the existing Network Slice.

- Then the Network Slice Management function populates the NetworkSliceJob with the newly created/existing NetworkSlice DN and notifies the MnS consumer

- The Network Slice Management Function updates the status of NetworkSliceMOI to indicate the completion of Network Slice creation

- The Network Slice Management Function sends Network Slice createMOI complete notification for Network Slice and/or NetworkSliceJob attribute value change event notification to the MnS consumer.

Input parameters:

| serviceProfileList | M | LIST OF SEQUENCE< serviceProfile, value > | This parameter specifies the network slice related requirements defined in ServiceProfile in Clause 6.3.3 in TS 28.541 [6]. |
| --- | --- | --- | --- |
| administrativeState | O | ENUM (LOCKED, UNLOCKED, SHUTTINGDOWN) | It indicates the administrative state of the network slice or the network slice subnet. It describes the permission to use or prohibition against using the managed object instance, imposed through the OAM services. |
| provisioningPriority | O | Alphanumeric (e.g., String) | The relative priority of the slice provisioning request. This shall enable the prioritization of parallel Network Slice / Network Slice Subnet provisioning requests, particularly in case multiple requests compete for the same set of limited resources |
| preemptable | O | Boolean | This is to say whether requested network slice can be pre-empted or not |
| resourceType | O | string 0..* | Type of the resource for reservation |
| jobProgress |  | JobProgress | Job Progress details |

Output parameters:

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| operationalState | M | ENUM (ENABLED, DISABLED) | It indicates the operational state of the NetworkSlice Instance. It describes whether or not the associated resource is physically installed and working. |
| administrativeState | M | ENUM (LOCKED, UNLOCKED, SHUTTINGDOWN) | It indicates the administrative state of the network slice or the network slice subnet. It describes the permission to use or prohibition against using the managed object instance, imposed through the OAM services. |
| networkSliceRef | M | DN | This holds a DN of NetworkSlice relating to the NetworkSlice instance. |