3GPP TSG-SA5 Meeting #141-e 	S5-221123

- e-meeting, 17th – 26th Janaury 2022													revision of S5-21xabc

- Source:	Huawei Technologies

- Title:	pCR TR 28.827 Addition of the soultion for the Roaming Charging Profile

- Document for:	Approval

Agenda Item:	7.5.4

# 1	Discussion

This is a pCR to TR 28.827 discussing about the solutions for the Roaming Charging Profile.

# 2	References

[1]	3GPP TR 28.827 "Telecommunication management; Charging management; 	Study on 5G charging for additional roaming scenarios and actors".

# 3	Rationale

This pCR proposes to discuss about the soultion for the Roaming Charging Profile in TR 28.827.

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.827[1].

First change

#### 7.1.4.4	Solution #1.4: Roaming Charging Profile in V-SMF

##### 7.1.4.4.1	General

A possible solution for REQ-CH_VMNO-05 and key issues 1c, Roaming Charging Profile negotiation enhancement for 5G data connectivity charging provided to the home MNO by the visited MNO, covering both home routed roaming and local breakout case.

##### 7.1.4.4.2	Reference architecture

The reference architecture would be the same as in clause 4.2.2 of TS 32.255 [4].

##### 7.1.4.4.3	Message flows

###### 7.1.4.4.3.1	Solution #1.4.1: PDU session establishment for Home Routed Roaming

The message flows about roaming charging profile negotiation for 5G data connectivity charging are present in the figure 5.2.2.12.2.1, PDU session establishment, of TS 32.255[4].

In the step 13,

13ch-a. Based on "received PDU session conditions" trigger, a Charging Data Request [Update] is sent to CHF with charging information received from H-SMF, which includes the HPLMN selected "Roaming Charging Profile" and counts per QFI are started.

13ch-b. The CHF updates the CDR.

13ch-c. The CHF acknowledges by sending Charging Data Response[Update] to the V-SMF.

When the V-SMF received the "Nsmf_PDUSession_Create Response" with the roaming charging profile from the H-SMF, the Charging Data Request [Update] is sent the the V-CHF. The corresponding trigger "received PDU session conditions" should be clarified.

###### 7.1.4.4.3.2	Solution #1.4.2: PDU session establishment for LBO Roaming

Based the figure 7.1.4.3.2-1, Local breakout roaming for 5G data connectivity charging, in the clause 7.1.4.3.3,

In the step 4ch-a,a Charging Data Request [Initial, Roaming Charging profile] is sent to V-CHF from V-SMF and then Charging Data Response [Initial, Roaming Charging profile] is send to the V-SMF from the V-CHF.

The related Roaming Charging profile negotiaction between V-SMF and H-CHF should be described in the step 5ch-a and step 5ch-c, which described in the clause 7.2.4.2.3.

End of change