3GPP TSG-SA5 Meeting #141-e 	S5-221133

e-meeting, January 17 – 26 2022

- Source:	Huawei, China Mobile, China Unicom, Nokia, Deutsche Telekom, AsiaInfo

- Title:	Update intent information model in clause 6.2.1

- Document for:	Approval

- Agenda Item:	6.4.9

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.7.0”.

# 3	Rationale

This contribution proposes to update the intent information model in clause 6.2.1 and address corresponding issues identified by Editor's Notes:

- Regarding the Editor's Note in the beginning of  clause 6.2.1, it proposes to remove it in the present document at this moment, because Annex C already capture the mapping of 3GPP and TM Forum IntentExpectation Model, also further mapping or federated method will be investigated in R18 eIDMS_MN SID.

- Regarding the Figure 6.2.1.1.1-1, following updates are made:

  - Explicitily show the IntentContext, ExpectationContext, ObjectContext and TargetContext to make the diagram more clear, which are different type of context with same structure (e.g. Context<<dataType>>) .

  - Present the ExpectationObject<<dataType>> in the UML diagram to clearly illustrates IntentExpectation includes ExpectationObject and ExpectationTarget.

  - Remove the Editor's Notes under the Figure.

- Regarding the Clause 6.2.1.2.1 Intent <<IOC>>, it proposes to add attribute intentReport to align with the update in the UML diagram and move the attribute "intentFulfilStatus" as sub-attribute of IntentReport<<dataType>>.

- Regarding the Clause 6.2.1.2.2 IntentExpectation, it proposes to move to clause 6.2.1.3, because the IntentExpectation is a <<dataType>> instead of <<Class>>. Same update for IntentReport.

- Regarding the clause 6.2.1.3.1(new) IntentExpectation<<dataType>>, it proposes to introduce a new ExpectationObject<<dataType>> as attribute of IntentExpectation, which includes attributes "ObjectType", "ObjectInstance" and "ObjectContext". "ObjectInstance" is used in the scenario of MnS consumer's expectation is for a specific object instance and MnS consumer have the knowledge of the DN of this specific object instance, while "ObjectType" is used for MnS consumer's expectation is not for a specific object instance or MnS consumer have no knowledge of the DN of this specific object instance.

- Regarding the clause 6.2.1.4 Attribute definition, it proposed to update to align with above updates.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

- 6.2	Information model definition for Intent (MnS component typeB)

- 6.2.1	Information model definition for Intent

- 6.2.1.1	Class diagram

- 6.2.1.1.1	Relationship

Figure 6.2.1.1.1-1 Relationship UML diagram for intent

Figure 6.2.1.1.2-1 Inheritance UML diagram for intent

- 6.2.1.2	Class definition

- 6.2.1.2.1 	Intent <<IOC>>

- 6.2.1.2.1.1	Definition

This IOC represents the properties of an Intent driven management information between MnS consumer and MnS producer.

The Intent IOC contains one or multiple IntentExpectation(s) which includes MnS consumer’s requirements, goals and contexts given to a 3GPP system.

The Intent IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is “Intent” and the value of attribute objectInstance is the DN of the instance of Intent IOC.

- 6.2.1.2.1.2	Attributes

The Intent IOC includes attributes inherited from TOP IOC (defined in TS 28.622) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentExpectations | M | T | T | F | F |
| userLabel | M | T | T | F | F |
| intentContexts | O | T | T | F | F |
| intentFulfilStatus | M | T | F | F | T |

- 6.2.1.2.1.3	Attribute constraints

None

includes the following attributes:

2nd Change

- 6.2.1.3	DataType definition

- 6.2.1.3.1	IntentExpectation <<dataType>>

- 6.2.1.3.1.1	Definition

IntentExpectation <<dataType>>represents MnS consumer’s requirements, goals and contexts given to a 3GPP system. The information of IntentExpectation is generated by MnS consumer and delivered to MnS producer.

- 6.2.1.3.1.2	Attributes

The IntentExpectation includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| expectationId | M | T | T | T | F |
| expectationObjects | M | T | T | F | F |
| expectationTargets | M | T | T | F | F |
| expectationContexts | O | T | T | F | F |

Note: The scenario/requirements-specific IntentExpectations are defined utilizing the constructs of this generic IntentExpectation model.

- 6.2.1.3.1.3	Attribute constraints

TBD

- 6.2.1.3.3	ExpectationObject <<dataType>>

- 6.2.1.3.3.1	Definition

The ExpectationObject <<dataType>> represents the Object of the IntentExpectation that are required to be applied on.

- 6.2.1.3.3.2	Attributes

The expectationObject includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ObjectType | CM | T | T | F | F |
| ObjectInstance | CM | T | T | F | F |
| ObjectContexts | O | T | T | F | F |

- 6.2.1.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| ObjectType Support Qualifier | Condition: The intent expectation is not for a specific object instance or MnS consumer have no knowledge of the DN of this specific object instance. |
| ObjectInstance Support Qualifier | Condition: The intent expectation is for a specific object instance and MnS consumer have the knowledge of the DN of this specific object instance |

- 6.2.1.3.4 	ExpectationTarget <<dataType>>

- 6.2.1.3.4.1	Definition

The ExpectationTarget  <<dataType>> represents the targets of the IntentExpectation that are required to be achieved.

- 6.2.1.3.4.2	Attributes

The expectationTarget includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetName | M | T | T | F | F |
| targetCondition | M | T | T | F | F |
| targetValueRange | M | T | T | F | F |
| targetContexts | O | T | T | F | F |

- 6.2.1.3.4.3	Attribute constraints

None

- 6.2.1.3.5 	Context <<dataType>>

- 6.2.1.3.5.1	Definition

The Context <<dataType>> represents the properties of a context. A context describes the list of constraints and conditions that should evaluate to true when the targets are fulfilled but are themselves not to be enforced. The context may apply to the intent, the intent expectation, the intent targets or to the object.

- 6.2.1.3.5.2	Attributes

The Context includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| contextAttribute | M | T | T | F | F |
| contextCondition | M | T | T | F | F |
| contextValueRange | M | T | T | F | F |

- 6.2.1.3.5.3	Attribute constraints

None

3rd Change

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectations | It describes the expectations including requirements, goals and contexts (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for expectation object(s). allowedValues: Not Applicable | type: IntentExpectation multiplicity: 1..* isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the current status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentContexts | It describes the list of IntentContext(s) which represents the constraints and conditions that should apply for the entire intent even if there may be specific contexts defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectationId | A user-friendly (and user assignable) name of the intentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectationObjects | It describes the expectation objects of the IntentExpectation that are required to be applied on. allowedValues: Not Applicable | type: ExpectationObject multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |
| objectType | It describes the type of expectation object of the IntentExpectation that are required to be applied on. It can be class name of the managed object. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| objectInstance | It describes an specific object instance (e.g. instance of managed object) to which the intentExpectation should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| objectContexts | It describes the list of ObjectContext(s) which represents the constraints and conditions to be used as filter information to identify the object(s) to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. allowedValues: depends on ExpectationObject in the IntentExpectation | type: Context multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectionTargets | It describes the list of ExpectationTarget(s) which represent specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: ExpectationTarget multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectationContexts | It describes the list of context(s) which represents the constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: Context multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |
| targetName | It describes the name of the Expectation of the expectation target which represents specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be allowedValues: is equal to; is less than; is greater than: "is within the range" ; "is outside the range" c | type: Enum multiplicity: 1 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the TargetCondition. allowedValues: depends on the targetName | type: Real multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| targetContexts | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: Not Applicable | type: Context multiplicity: 1..* isOrdered: False isUnique: False defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: is equal to; is less than; is greater than; "is within the range" ;"is outside the range" . | type: Enum multiplicity: upto 1 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |
| contextValueRange | It describes the range of values that applicable to the ContextAttribute and the ContextCondition. AllowedValue: depends on the contextAttribute | type: Real multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |

3rd Change

- A.2	Information model definition for Intent

- A.2.1	Relationship UML diagram for intent (Figure 6.2.1.1.2-1)

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

class “<<proxyClass>> \n  ManagedEntity ” as ManagedEntity{}

class “<<InformationObjectClass>>\n Intent ” as Intent {}

class “<<dataType>>\n IntentExpectation” as IntentExpectation{}

class “<<dataType>>\n IntentContext” as IntentContext{}

class “<<dataType>>\n IntentReport” as IntentReport{}

class “<<dataType>>\n ExpectationObject” as ExpectationObject{}

class “<<dataType>>\n ObjectContext” as ObjectContext{}

class “<<dataType>>\n ExpectationTarget” as ExpectationTarget{}

class “<<dataType>>\n TargetContext” as TargetContext{}

class “<<dataType>>\n ExpectationContext” as ExpectationContext{}

ManagedEntity  "1" *-- "*" Intent

ManagedEntity "1" *-- "*" IntentReport

Intent --> IntentReport

Intent "1" *-- "1..*" IntentExpectation

Intent "1" *-- "*" IntentContext

IntentExpectation "1" *-- "*" ExpectationTarget

IntentExpectation "1" *-- "*" ExpectationObject

IntentExpectation "1" *-- "*" ExpectationContext

ExpectationObject "1" *-- "*" ObjectContext

ExpectationTarget "1" *-- "*" TargetContext

note left of ManagedEntity

Represents the folllowing IOCs:

Subnetwork

end note

@enduml

End of changes