3GPP TSG-SA5 Meeting #141-e 	S5-221182rev1

- e-meeting, 17 -26 January 2022

- Source:	Huawei, China Unicom

- Title:	PCR TS 28.104 add Energy saving analytics output solution --stage2

- Document for:	Approval

Agenda Item:	6.4.15

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 28.104 “Management and orchestration; Management Data Analytics”

# 3	Rationale

TS 28.104 [1] has defined the use case and requirement of MDA assisted Energy Saving in chapter 7.2.4. This contribution is proposed to add MDA capability for MDA assisted Energy Saving in [1].

# 4	Detailed proposal

It is proposed to apply following changes in [1].

1st modified section

### 8.4.4	MDA assisted energy saving

#### 8.4.4.1    Energy saving analysis

##### 8.4.4.1.1	MDA type

The MDA type for MDA assisted energy saving is: MDAAssistedEnergySaving.EnergySavingAnalysis.

##### 8.4.4.1.2	Enabling data

TBD.

##### 8.4.4.1.3	Analytics output

The specific information elements of the analytics output (MDA report) for MDA assisted energy saving analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.4.3-1.

Table 8.4.4.1.3-1: Analytics output for MDA assisted energy saving analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| IssueCell | The identification of issue cell(s) which will cause the energy efficiency issue (including high energy consumption and low energy efficiency of the gNB or area). | M | type: DN multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| IssueType | It indicates the the energy issue type of issue cell, e.g. high consumption or low efficiency. | M | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficLoadTrends | Predictions related to the trends of traffic load (e.g., PRB utilization rate) which could be used as references for making decision on energy saving. The predictions include the traffic load of the issue cell(s) and neithboring cell(s). | M | type:TrafficLoadTrend multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

2nd modified section

X.Y.1	<<DataType>>TrafficLoadTrend

#### X.Y.1.1	Definition

This data type specifies the type of TrafficLoadTrend.

#### X.Y.1.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| CellId | It indicates the cell for which the traffic load prediction is performed. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Timestamp | It provides the time point corresponding to the predicted traffic load. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficLoad | It provides the predicted traffic load, e.g., PRB utilization rate and its value. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of modified section