3GPP TSG-SA5 Meeting #141-e 	S5-221185rev1

- e-meeting, 17 -26 January 2022

- Source:	Huawei, China Unicom

- Title:	pCR TS 28.104 add AnalticsOutputFilters of coverage issue analysis stage2

- Document for:	Approval

- Agenda Item:	6.4.15

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 28.104 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This pCR is to add coverage analysis filter including interestAreaScope, RSRP threshold and RSRQ threshold, so as to MDA producer perform specific coverage analysis.

# 4	Detailed proposal

Start of modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS28.535: “Management services for communication service assurance; Requirements”.

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[8]	3GPP TS 28.405: "Telecommunication management, Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[14]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[15]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[16]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM); Information Service (IS)".

[x]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description;".

Next modification

### 9.3.1	MDARequest

#### 9.3.1.1	Definition

The IOC MDARequest represents the MDA output request and configuration parameters of MDA analysis service capabilities.

To request a desired MDA output, a MDAS consumer has to create MDARequest object instance on the MDAS producer.

#### 9.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mDAType | M | T | T | F | T |
| analyticsOutputFilters | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |

Editor's Note: whether the reportingMethod can be contained in MDARequest IOC is FFS.

#### 9.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |

#### 9.3.1.4	Notifications

The common notifications defined in clause 4.5 as defined in TS 28.541 [15] are valid for this IOC, without exceptions or additions.

### 9.3.X	AnalyticsOutputFilters <<Datatype>>

#### 9.3.X.1	Definition

This <<dataType>> defines Analytics output filter including coverage analytics filters.

#### 9.3.X.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageAnalyticsFilters | M | T | T | F | T |

#### 9.3.X.3	Attribute constraints

None.

### 9.3.Y1	CoverageAnalyticsFilters <<Datatype>>

#### 9.3.Y1.1	Definition

This <<dataType>> defines Coverage Analytics filter including RSRP threshold and RSRQ threshold

#### 9.3.Y1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| interestAreaScope | M | T | T | F | T |
| rSRPThreshold | M | T | T | F | T |
| rSRQThreshold | M | T | T | F | T |

#### 9.3.Y1.3	Attribute constraints

None.

### 9.3.Y2	InterestAreaScope <<Datatype>>

#### 9.3.Y2.1	Definition

This <<dataType>> defines Coverage Analytics Interst area scope including geographical location and cell list.

#### 9.3.Y2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| choice |  | T | T | F | T |
| >geoArea | O | T | T | F | T |
| >cellIdList | O | T | T | F | T |

#### 9.3.Y2.3	Attribute constraints

None.

### 9.3.Y3	GeoArea <<Datatype>>

#### 9.3.Y3.1	Definition

This <<dataType>> defines the type of Geographical location.

#### 9.3.Y3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| longtitude | M | T | T | F | T |
| latitude | M | T | T | F | T |

#### 9.3.Y3.3	Attribute constraints

None.

Next modification

## 9.4	Attribute definitions

### 9.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mDAType | It indicates the type of MDA capability for analysis. AllowedValues: CoverageAnalytics | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| analyticsOutputFilters | It indicates the analytics output filters of the dedicated MDA Type to be analyzed. | type: AnalyticsOutputFilters multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| coverageAnalyticsFilters | It indicates the coverage analytics output filters to be analyzed. | type: CoverageAnalyticsFilters multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| interestAreaScope | It indicates the interest area scope of energy saving analysis. The area is represented by a list of cell, list of TA or the coordinates (longitude and latitude) of the area. AllowedValues: list of cell and Geographical location. | type: InterestAreaScope multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rSRPThreshold | It indicates the threshold of RSRP used to detect coverage issue. | Type: Interger multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rSRQThreshold | It indicates the threshold of RSRQ used to detect coverage issue. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longtitude | It specifies the longitude of a geographical coordinate based on World Geodetic System (1984 version) global reference frame (WGS 84). | type: float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| latitude | It specifies the latitude of a geographical coordinate based on World Geodetic System (1984 version) global reference frame (WGS 84). | type: float multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellIdList | It indicate a list of the cell global identity of the NR cell. See "Global gNB ID" in subclause 8.2 of TS 38.300 [x]. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| geoArea | It defines the type of Geographical location. | type: GeoArea multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of modifications