3GPP TSG-SA5 Meeting #141-e 	S5-221219

- e-meeting, 17 -26 January 2022

- Source:	Huawei

- Title:	pCR Add alarm incident analysis solution

- Document for:	Approval

- Agenda Item:	6.4.15

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-030 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This contribution is proposed to add MnS component for fault management related issues analysis capability of MDAS in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

### 8.4.x	Fault management related issues

#### 8.4.x.1	Alarm Analytics

##### 8.4.x.1.1	MDA type

The MDA type for Alarm Analytics is: FaultManagement. FaultAnalytics.

##### 8.4.x.1.2	Enabling data

TBD.

##### 8.4.x.1.3	Analytics output

The specific information elements of the analytics output (MDA report) for Alarm Analytics, in addition to the common information elements of the analytics outputs (see clause X.Y), are provided in table 8.4.x.1.3-1.

Table 8.4.x.1.3-1: Analytics output for Alarm analysis

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| alarmAnalysisId | The identifier of this analytics . | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CorrelatedAlarmInfos | List of Alarm name or alarm ID. | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RootCause | Indicates the root cause of Fault. | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SeverityLevel | It indicates the severity level of the Fault. Values: Critical, Major, Minor, Warning, Indeterminate, Cleared | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End modified section