- 3GPP TSG-SA5 Meeting #141e	S5-221251rev5

- 17 Jan to 26 Jan 2022, E-meeting

- Source:	China Telecom, China Mobile, HUAWEI

- Title:	Add MDA capability for MDA assisted energy saving

- Document for:	Approval

- Agenda Item:	6.4.15

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 28.104-030 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This pCR is to add the MDA capability for MDA assisted energy saving analysis for TS 28.104 [1].

# 4	Detailed proposal

Start of modifications

### 8.4.X	MDA assisted energy saving

#### 8.4.X.1	MDA type

The MDA type for energy saving analysis is: MDAAssistedEnergySaving.EnergySavingAnalysis.

#### 8.4.X.2	Enabling data

The enabling data for coverage problem analysis are provided in table 8.4.X.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.X.2-1: Enabling data for energy saving analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | PNF Power Consumption: power consumed over the measurement period | Clause 5.1.1.19.2 of TS 28.552 [4] |
|  | PNF Energy consumption: energy consumed | Clause 5.1.1.19.3 of TS 28.552 [4] |
|  | SS-RSRP distribution per SSB (beam) of serving NR cell | Clause 5.1.1.22.1 of TS 28.552 [4]. |
|  | SS-RSRP distribution per SSB (beam) of neighbor NR cell | Clause 5.1.1.22.1 of TS 28.552 [4]. |
|  | PDCP Data Volume of NR cells: PDCP data volume delivered in the downlink and uplink; | Clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4] |
|  | Traffic load variation: PRB utilization rate, RRC connection number, etc | Clause 5.1.1.2 and 5.1.1.4 of TS 28.552 [4] |
|  | UE throughput: UE throughput in downlink and uplink | Clause 5.1.1.3 of TS 28.552 [4] |
|  | Delay related measurements of UPF | Clause 5.4 of TS 28.552 [4] |
|  | Data volume of UPF | Clause 5.4 of TS 28.552 [4] |
|  | Virtual resource usage of NF: The virtual CPU usage, virtual memory usage, virtual disk usage of virtual network functions; | Clause 5.7.1 of TS 28.552 [4] |
| MDT reports | The RSRPs of UE measurements. | RSRPs of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
|  | The RSRQs of UE measuremnets. | RSRQs of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
|  | The UE location information. | UE location of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
| QoE Data | The measurements that are collected are DASH and MTSI measurements. | TS 28.406 [9] |
| Configuration data | MOIs of the cells, UPFs and SMFs. | TS 28.541 [15] |
| Network analytics data | The control plane analysis result from the NWDAF, e.g., observed service experience related network data analytics. | TS 23.288 [10] |

#### 8.4.X.3	Analytics output

The specific information elements of the analytics output for energy saving analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.X.3-1.

Table 8.4.X.3-1: Analytics output for energy saving analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EnergyEfficiencyProblematicObject | Indication of NR cells or NFs where the energy efficiency issues occurred or potentially occur. | M | type: DN multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnergyEfficiencyProblemType | Indication of type of the energy efficiency issues. The allowed value is one of the enumerated values: HighEnergyConsumption, LowEenergyEfficiency, Other, Unknown. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficLoadTrends | Predictions related to the trends of traffic load (e.g., PRB utilization rate) which could be used as references for making decision on energy saving. The predictions include the traffic load of the issue cell(s) and neithboring cell(s). | M | type:TrafficLoadTrend multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnergySavingRecommendations | The recommendation shall contain the energy saving policy. For ES on NR cells. It may contain a set of - recommended NR Cell (ES-Cell) to enter energySaving state. - recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - the time to enter and terminate the energy saving state. - the load threshold to enter and terminate the energy saving state for the ES-Cell For ES on UPFs. It contains a set of - recommended UPF (ES-UPF) to conduct energy saving; - recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. | M | type: EsRecommendation multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| StatisticsOfCellsEsState | The statistic result of current energy saving state of the cells at a certain time, which can be used by consumers to make analysis (e.g., observed service experience analysis made by NWDAF) or to make decision (e.g., enter/exit the energy saving state based on the current energy saving state). | O | type: StatisticOfCellEsState multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

## 8.X	Data type definitions

### 8.X.a	TrafficLoadTrend

#### 8.X.a.1	Definition

This data type specifies the type of TrafficLoadTrend.

#### 8.X.a.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| CellId | It indicates the cell for which the traffic load prediction is performed. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Timestamp | It provides the time point corresponding to the predicted traffic load. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficLoad | It provides the predicted traffic load including PRB utilization rate. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 8.X.b	EsRecommendation

TBD

|  | ES-cell is lower than the threshold |  |  |
| --- | --- | --- | --- |

### 8.X.c	StatisticOfCellEsState

#### 8.X.c.1	Definition

This data type specifies the type of statistics of cells energy saving state in the analytics output.

#### 8.X.c.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| AnalyticsTime | It indicates the time of the information that are used for analytics. | M | type: DataTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CellsInEsState | It provides the DN of NR cells which are in energy saving state. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CellsNotInEsState | It provides the DN of NR cells which are not in energy saving state. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of modifications