3GPP TSG-SA5 Meeting #141-e	S5-221270

- Online, , 17th Jan 2022 - 26th Jan 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.x.531 | CR | 0104 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.531 Fixing Network slice and network slice subnet provisioning management service - profiles not convert to IOC - Stage 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Benelux BV |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-PRO_NS |  |  |  |  |  | Date: |  |  | 2022-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Stage 2 changes are already agreed as part of S5-21549x. This contribution provide Stage 3 changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stage 3 fixation wrt stage 2 changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In consistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 8	void

## .1-1: Mapping of IS operations to SS equivalents

Table 8.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

Table 8.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

Table 8.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

Table 8.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

Table 8.1.4-1: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

Table 8.1.5-1: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

Table 8.1.6-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

Table 8.1.6-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

# x	RESTful HTTP-based solution set of provisioning

## x.1	Network slice provisioning management service

### x.1.1	Mapping of operations

#### x.1.1.1	Introduction

Table x.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| allocateNsi | POST | {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfile | M |
| deallocateNsi | DELETE | {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfile={ServiceProfileId} | M |

#### x.1.1.2	Operation allocateNsi

This operation is to allocate a network slice instance provided by the service provider, the network slice instance may be new or existing.

Table x.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| attributeListIn | request body | n/a | n/a | M |

Table x.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | n/a | n/a | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |
| networkSliceDN | response body | n/a | Resource | M |

The message flow for allocation is as follows:

1.	The MnS consumer sends a HTTP POST request to the MnS producer.

- The target URI is equal to the concatenation of URI of the parent resource of resource to be created, and the resource (in this case ServiceProfile) to be created.

- The message body shall carry the complete representation of the resource to be created. The resource identifier shall be absent or carry null semantics.

2.	The MnS producer sends a HTTP POST response to the MnS consumer.

- On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource (in this case ServiceProfile) and the message body shall contain the complete complete representation of the ServiceProfile and networkSliceDN identifying the NetworkSlice MOI created.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

#### x.1.1.3	Operation deallocateNsi

This operation deallocate a service profile in an NSI. The provider may terminate the requested NSI or modify the requested NSI without termination to satisfy the request.

Table x.1.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| networkSliceDN | request body | n/a | Resource | M |
| serviceProfileId | Request body | n/a | Resource | M |

Table x.1.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

- The target URI is equal to the concatenation of URI of the parent resource and the resource (in this case ServiceProfile) to be deleted.

- The message body shall contain the networkSliceDN identifying the NetworkSlice MOI.

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

- On success, "201 Created" shall be returned.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

### x.1.2	Resources

#### x.1.2.1	Resource definitions

##### x.1.2.1.1	Resource “…/ServiceProfile

##### x.1.2.1.1.1	Description

This resource represents collects of network slice related requirement (i.e. ServiceProfiles).

##### x.1.2.1.1.2	URI

Resource URI: {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfile

##### x.1.2.1.1.3	HTTP methods

x.1.2.1.1.3.1	POST

The POST method create a serviceProfile, the provider may create a NSI or using existing NSI to satisfy the serviceProfile.

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table x.2.1.1.3.1-1: Data structures supported by the POST Request Body on this resource

| Name | DATA TYPE | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| attributeListIn | LIST OF SEQUENCE< attribute name, attribute value> | M | 1 | This parameter specifies the network slice related requirements or network related requirements defined in ServiceProfile in Clause 6.3.3 in TS 28.541 [6]. |

Table Y.2.1.1.3.1-2: Data structures supported by the POST Request Body on this resource

| Name | DATA TYPE | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| attributeListOut | LIST OF SEQUENCE< attribute name, attribute value> | M | 1 | This list of name/value pairs contains the attributes of the SliceProfile[6] which has been allocated and the actual value assigned to each. |
| status | HTTP response code | M | 1 | HTTP response code 200 indicates “OperationSucceeded”. All other HTTP response codes indicate “OperationFailed”. |
| networkSliceDN | Resource | M | 1 | The DN of NetworkSlice MOI uniquely identifying the network slice instance. |

x.1.2.1.1.3.2	DELETE

The DELETE method deletes a ServiceProfile.

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table x.2.2.2.2.3.1-1: Data structures supported by the DELETE Request Body on this resource

| Name | DATA TYPE | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| networkSliceDN | Resource | M | 1 | The DN of NetworkSlice MOI uniquely identifying the network slice instance. |
| serviceProfileId | Resource | M | 1 | It specifies the global unifique identifier of the service profile in the NSI which is to be deallocated. |

Table x.2.2.2.2.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Name | DATA TYPE | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| status | HTTP response code | M | 1 | HTTP response code 200 indicates “OperationSucceeded”. All other HTTP response codes indicate “OperationFailed”. |

## x.2	Network slice subnet provisioning management service

### x.2.1	Mapping of operations

#### x.2.1.1	Introduction

Table x.2.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| allocateNssi | POST | {MnSRoot}/NSSProvMnS/{MnSVersion}/SliceSubnetMgmt/SliceProfile | M |
| deallocateNssi | DELETE | {MnSRoot}/NSSProvMnS/{MnSVersion}/SliceSubnetMgmt/SliceProfile={id} | M |

#### x.2.1.2	Operation allocateNssi

This operation is to allocate a network slice instance provided by the service provider, the network slice subnet instance may be new or existing.

Table x.2.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| attributeListIn | request body | n/a | Resource | M |

Table x.2.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | n/a | Resource | M |
| status | response status codes | n/a | n/a | M |
|  | Response body | error | ErrorResponse | O |
| networkSliceSubnetDN | response body | n/a | Resource | M |

The message flow for allocation is as follows:

1.	The MnS consumer sends a HTTP POST request to the MnS producer.

- The target URI is equal to the concatenation of URI of the parent resource of resource to be created, and the resource (in this case SliceProfile) to be created.

- The message body shall carry the complete representation of the resource to be created. The resource identifier shall be absent or carry null semantics.

2.	The MnS producer sends a HTTP POST response to the MnS consumer.

- On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource (in this case SliceProfile) and the message body shall contain the complete complete representation of the SliceProfile and networkSliceSubnetDN identifying the NetworkSliceSubnet MOI created.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

#### x.2.1.3	Operation deallocateNssi

This operation deallocate a slice profile in an NSSI. The provider may terminate the requested NSSI or modify the requested NSSI without termination to satisfy the request.

Table x.2.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| networkSliceSubnetDN | request body | n/a | Resource | M |
| sliceProfileId | request body | n/a | Resource | M |

Table x.2.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

- The target URI is equal to the concatenation of URI of the parent resource and the resource (in this case SliceProfile) to be deleted.

- The message body shall contain the networkSliceSubnetDN identifying the NetworkSliceSubnet MOI.

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

- On success, "201 Created" shall be returned.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

### x.2.2	Resources

#### x.2.2.1	Resource definitions

#### x.2.2.1.1		Resource “.../SliceProfile

##### x.2.2.1.1.1	Description

This resource represents collects of network slice subnet related requirements (i.e. SliceProfile).

##### x.2.2.1.1.2	URI

Resource URI: {MnSRoot}/NSSProvMnS/{MnSVersion}/SliceSubnetMgmt/SliceProfile

##### x.2.2.1.1.3	HTTP methods

x.2.2.1.1.3.1	POST

The POST method create a SliceProfile, the provider may create a new NSSI or using existing NSSI to support the SliceProfile.

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table x.2.1.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Name | DATA TYPE | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| attributeListIn | LIST OF SEQUENCE< attribute name, attribute value> | M | 1 | This parameter specifies the network slice subnet related requirements defined in SliceProfile in Clause 6.3.4 in TS 28.541 [6]. |

Table x.2.1.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Name | DATA TYPE | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| attributeListOut | LIST OF SEQUENCE< attribute name, attribute value> | M | 1 | This list of name/value pairs contains the attributes of the SliceProfile[6] which has been allocated and the actual value assigned to each. |
| status | HTTP response code | M | 1 | HTTP response code 200 indicates “OperationSucceeded”. All other HTTP response codes indicate “OperationFailed”. |
| networkSliceSubnetDN | Resource | M | 1 | The DN of NetworkSliceSubnet MOI uniquely identifying the network slice instance. |

x.2.2.1.1.3.2	DELETE

The DELETE method deletes a SliceProfile.

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table x.2.2.2.2.3.1-1: Data structures supported by the DELETE Request Body on this resource

| Name | DATA TYPE | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| networkSliceSubnetDN | Resource | M | 1 | The DN of NetworkSliceSubnet MOI uniquely identifying the network slice subnet instance. |
| sliceProfileId | Resource | M | 1 | It specifies the unifique identifier of the slice profile in the NSSI which is to be deallocated. |

Table x.2.2.2.2.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Name | DATA TYPE | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| status | HTTP response code | M | 1 | HTTP response code 200 indicates “OperationSucceeded”. All other HTTP response codes indicate “OperationFailed”. |