- 3GPP TSG-SA5 Meeting #141e	S5-221354rev2

- 17 - 26 January 2022, E-meeting

- Source:	Intel

- Title:	pCR 32.257 Add charging information definition for edge application server deployment charging

- Document for:	Approval

- Agenda Item:	7.4.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 32.257-020 “Edge Computing domain charging; stage 2”

# 3	Rationale

This pCR is to add charging information definition for edge application server deployment charging to TS 32.257.

# 4	Detailed proposal

Start of modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[3]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[4]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[5]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[6]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[7]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[8]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[9]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[10]	3GPP TS 23.548 "5G System Enhancements for Edge Computing; Stage 2".

[11]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G Data connectivity domain charging; stage 2".

[12]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[x]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[y]	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[z]	3GPP TS 28.532: "Management and orchestration; Generic management services".

Next modification

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], TS 23.501 [8], TS 23.558 [9], TS 23.548 [10] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1], TS 23.501 [8], TS 23.558 [9] and TS 23.548 [10].

ASP	Application Service Provider

ECSP	Edge Computing Service Provider

MNO	Mobile Network Operator

CEF	Charging Enablement Function

CHF	Charging Function

EAS	Edge Application Server

EES	Edge Enabler Server

MnS	Management Service

MOI	Managed Object Instance

Next modification

## 6.3	Definition of charging information for EAS deployment charging

### 6.3.1	Data description for EAS deployment charging

#### 6.3.1.1	Message contents

##### 6.3.1.1.1	General

The Charging Data Request and Charging Data Response are specified in TS 32.290 [6] and include charging information. The Charging Data Request can be of type [Event].

Table 6.3.1.1.1-1 describes the use of these messages for converged charging.

Table 6.3.1.1.1-1: Converged charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | CEF | CHF |
| Charging Data Response | CHF | CEF |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [2].

##### 6.3.1.1.2	Charging Data Request message

Table 6.3.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the CEF as used for EAS deployment converged charging.

Table 6.3.1.1.2-1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| One-time Event | OC | Described in TS 32.290 [6]. |
| One-time Event Type | OC | Described in TS 32.290 [6]. |
| NF Consumer Identification | M | Described in TS 32.290 [6]. |
| NF Functionality | M | Described in TS 32.290 [6]. |
| NF Name | OC | Described in TS 32.290 [6]. |
| NF Address | OC | Described in TS 32.290 [6]. |
| NF PLMN ID | OC | Described in TS 32.290 [6]. |
| Invocation Timestamp | M | Described in TS 32.290 [6]. |
| Invocation Sequence Number | - | Described in TS 32.290 [6]. |
| EAS ID | OM | This field holds the EAS ID, see TS 23.558 [9]. |
| EDN ID | OM | This field holds the DN of EdgeDataNetwork MOI, see TS 28.538 [12]. |
| EAS Provider Identifier | OM | The identifier of the ASP that provides the EAS, see TS 23.558 [9]. |
| EAS Deployment Charging Information | OM | This field holds the for EAS deployment charging specific information described in clause 6.3.2.1.2. |

##### 6.3.1.1.3	Charging data response message

Table 6.3.1.1.3-1 illustrates the basic structure of a Charging Data Response message from the CHF as used for EAS deployment converged charging.

Table 66.3.1.1.3-1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Invocation Timestamp | M | Described in TS 32.290 [6] |
| Invocation Result | OC | Described in TS 32.290 [6] |
| Invocation Sequence Number | - | Described in TS 32.290 [6] |

#### 6.3.1.2	Ga message contents

See clause 5.2.3.4.

#### 6.3.1.3	CDR description on the Bee interface

##### 6.3.1.3.1	General

This clause describes the CDR content and format generated for EAS deployment charging.

The following tables provide a brief description of each CDR parameter. The category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [2]. Full definitions of the CDR parameters, sorted by the name in alphabetical order, are provided in TS 32.298 [3].

##### 6.3.1.3.2	EAS deployment charging CHF CDR data

If enabled, CHF CDRs for EAS deployment charging shall be produced for each EAS LCM notification (i.e., notifyMOICreation, notifyMOIAttributeValueChanges, or notifyMOIDeletion).

The fields of EAS deployment charging CHF CDR are specified in table 6.3.1.3.2-1.

Table 6.3.1.3.2-1: EAS deployment charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| NF Consumer Information | M | This field holds the information of the CEF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. CEF) |
| NF Name | OC | This field holds the name of the CEF used. |
| NF Address | OC | This field holds the IP Address of the CEF used. |
| NF PLMN ID | OC | This field holds the PLMN identifier (MCC MNC) of the CEF. |
| Record Opening Time | M | Described in TS 32.298 [3] |
| Record Sequence Number | C | Described in TS 32.298 [3] |
| Cause for Record Closing | M | Described in TS 32.298 [3] |
| Diagnostics | OM | Described in TS 32.298 [3] |
| Local Record Sequence Number | OM | Described in TS 32.298 [3] |
| Record Extensions | OC | Described in TS 32.298 [3] |
| EAS ID | OM | This field holds the EAS ID, see TS 23.558 [9]. |
| EDN ID | OM | This field holds the DN of EdgeDataNetwork MOI, see TS 28.538 [12]. |
| EAS Provider Identifier | OM | The identifier of the ASP that provides the EAS, see TS 23.558 [9]. |
| EAS Deployment Charging Information | OM | This field holds the EAS deployment charging specific information defined in clause 6.3.2.1.2. |

### 6.3.2	EAS deployment charging specific parameters

#### 6.3.2.1	Definition of EAS deployment charging information

##### 6.3.2.1.1	General

The Charging Information parameter used for EAS deployment charging is provided in the following clauses.

##### 6.3.2.1.2	Definition of EAS deployment specific charging information

Specific charging information used for EAS deployment charging is provided within the EAS Deployment Charging Information.

The detailed structure of the EAS Deployment Charging Information can be found in table 6.3.2.1.2-1.

Table 6.3.2.1.2-1: Structure of EAS Deployment Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| EAS Deployment Requirements | M | This field holds the EAS Deployment Requirements, see TS 23.558 [9]. |
| LCM Start Time | M | This field holds the start time of the EAS LCM process, see TS 28.550 [y]. |
| LCM End Time | M | This field holds the end time of the EAS LCM process, see TS 28.550 [y]. |

#### 6.3.2.2	Formal EAS deployment charging parameter description

##### 6.3.2.2.1	EAS deployment CHF CDR parameters

Editor’s note: The detailed definitions, abstract syntax and encoding of EAS deployment CHF CDRs parameters are to be specified in TS 32.298 [3].

##### 6.3.2.2.2	EAS deployment resources attributes

Editor’s note: The detailed definitions of resources attributes used for EAS deployment charging are to be specified in TS 32.291 [7].

#### 6.3.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by CEF for EAS deployment converged charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.2.3-1 defines the basic structure of the supported fields in the Charging Data Request message for EAS deployment converged charging.

Table 6.3.3.1: Supported fields in Charging Data Request message

| Information Element | Functionality of CEF | EAS deployment charging |
| --- | --- | --- |
|  | Supported Operation Types | E |
| One-time Event |  | E |
| One-time Event Type |  | E |
| NF Consumer Information |  | E |
| EAS ID |  | E |
| EDN ID |  | E |
| EAS Provider Identifier |  | E |
| Invocation Timestamp |  | E |
| Invocation Sequence Number |  | - |
| EAS Deployment Requirements |  | E |
| LCM Start Time |  | E |
| LCM End Time |  | E |

Table 6.3.2.3-2 defines the basic structure of the supported fields in the Charging Data Response message for EAS deployment converged charging.

Table 6.3.2.3-2: Supported fields in Charging Data Response message

| Information Element | Functionality of CEF | EAS deployment charging |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Invocation Timestamp |  | E |
| Invocation Result |  | E |
| Invocation Sequence Number |  | - |

### 6.3.3	Bindings for EAS deployment converged charging

Editor’s note: This mapping between the Information Elements, resource attributes and CHF CDR parameters for EAS deployment converged charging is to be described in TS 32.291 [7].

End of modifications