3GPP TSG-SA5 Meeting #142-e 	S5-222352

# e-meeting, 4 - 12 April 2022

# Source:	Nokia

# Title:	Add an enumeration value in output for “Network slice throughput analysis” Usecase

# Document for:	Approval

Agenda Item:	6.6.5

# 1	Decision/action requested

This contribution is for approval.

# 2	Rationale

For the usecase of “Network slice throughput analysis”, section 8.4.2.2.3 defines the IEs that are part of Analytics Output. The IE “NetworkSliceThroughputIssueType” is defined to be an enumeration with possible values “RAN Issue” and “Core Issue”.

The use case addresses the throughput usecase holistically for the entire network slice, which contains entities from both RAN and core. MDA producer is expected to identify whether the issue is found in RAN or Core. It could happen that for a given problematic slice, the problem could be found in both RAN and core of that problematic slice. Currently there is no way to report this.

Since any given slice could be having issues in RAN and/or core, it is required to have an option “both” indicating this scenario.

# 3	Detailed proposal

1st Modified Section

#### 8.4.2.2	Network slice throughput analysis

##### 8.4.2.2.1	MDA type

The MDA type for Capability-Network slice throughput analysis is: SLSAnalysis.NetworkSliceThroughputAnalysis

##### 8.4.2.2.2	Enabling data

The enabling data for network slice throughput analysis are provided in table 8.4.2.2.2-1.

Table 8.4.2.2.2-1: Enabling data for network slice throughput analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | UL/DL throughput for network and Network Slice Instance | Upstream throughput for network and Network Slice Instance as defined in 6.3.3 in TS28.554 [5]; Downstream throughput for Single Network Slice Instance as defined in 6.3.4 in TS28.554 [5] |
|  | RAN UE Throughput | RAN UE Throughput as defined in 6.3.6 in TS28.554 [5] |
|  | Throughput at N3 interface | Upstream Throughput at N3 interface as defined in 6.3.4 in TS28.554 [5]; Downstream Throughput at N3 interface as defined in 6.3.5 in TS28.554 [5]; |

##### 8.4.2.2.3	Analytics output

The specific information elements of the analytics output for network slice throughput analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.2.3-1.

Table 8.4.2.2.3-1:  Network slice throughput analysis output

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| NetworkSliceThroughputIssueId | Network slice throughput issue identifier | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputIssueType | Indication of the network slice throughput issue type The allowed value is one of the enumerated values: RAN issue, CN issue, both | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputUserStatistics | The statistics of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of users, for which the required SLS throughput satisfies | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputTimeStatistics | The statistics of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of time, during which the required SLS throughput satisfies | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputUserPredictions | The predictions of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of users, for which the required SLS throughput could be met | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputTimePredictions | The predictions of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of time, during which the required SLS throughput could be met. | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of Modified Sections