3GPP TSG-SA5 Meeting #142-e	S5-222283

- Online, , 4th April 2022 - 12th Apr 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | draftCR | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 draftCR 28.538 Notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ECM |  |  |  |  |  | Date: |  |  | 2022-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Most of the notification clauses are empty |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide text for notifications clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 6.	Edge NRM

## 6.1	Information Model definitions for Edge NRM

### 6.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [4], IOC, Top | Top |
| TS 28.622 [4], IOC, SubNetwork | SubNetwork |
| TS 28.622 [4], IOC, ManagedFunction | ManagedFunction |
| TS 28.541 [3], IOC, PCFFunction | PCFFunction |
| TS 28.541 [3], IOC, NEFFunction | NEFFunction |
| TS 28.541 [3], IOC, UPFunction | UPFFunction |
| TS 28.541 [3], IOC, EP_N5 | EP_N5 |
| TS 28.541 [3], IOC, EP_N33 | EP_N33 |
| TS 28.541 [3], IOC, EP_N6 | EP_N6 |
| TS 28.541 [3], dataType, tAI | tAI |
| TS 28.658 [12], dataType, PLMNId | PLMNId |

## 6.2	Class diagram

### 6.2.1	Relationships

Figure 6.2.1-1: Edge NRM relationship diagram

Figure 6.2.1-3: Transport view of EES NRM

Figure 6.2.1-4: Transport view of ECS NRM

Figure 6.2.1-5: Transport view of EAS NRM

### 6.2.2	Inheritance

Figure 6.2.2-1: Edge Inheritance Relationship

## 6.3	Class definition

### 6.3.1	EASFunction

#### 6.3.1.1	Definition

This IOC represent the properties of a EAS in a 3GPP network. For more information about EAS, see 3GPP TS 23.558.

#### 6.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| eASIdentifier | M | T | T | F | T |
| eASAddress | M | T | T | F | T |
| eESAddress | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| eASRequirementsRef | M | T | T | F | T |

#### 6.3.1.3	Attribute constraints

None.

#### 6.3.1.4	Notifications

The common notifications defined in subclause 5.5 of 3GPP TS 28.541 [3] are valid for this IOC, without exceptions or additions.

### 6.3.2	EASRequirements

#### 6.3.2.1	Definition

This represent the requirements needed to deploy EAS(s).

#### 6.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| requiredEASservingLocation | M | T | F | F | T |
| softwareImageInfo | M | T | F | F | T |
| affinityAntiAffinity | M | T | F | F | T |
| serviceContinuity | M | T | F | F | T |
| virtualResource | M | T | F | F | T |

#### 6.3.2.3	Attribute constraints

None.

#### 6.3.2.4	Notifications

The common notifications defined in subclause 5.5 of 3GPP TS 28.541 [3] are valid for this IOC, without exceptions or additions.

### 6.3.3	ServingLocation <<dataType>>

#### 6.3.3.1	Definition

This datatype represents the location which is to be served by the node.

#### 6.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| geographicalLocation | CM | T | F | F | T |
| topologicalLocation | CM | T | T | F | T |

#### 6.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| geographicalLocation Support Qualifier | Condition: If the serving location is defined as Geographical Service Area [2]. |
| topologicalLocation Support Qualifier | Condition: If the serving location is defined as Topological Service Area [2]. |

NOTE: 	Only one of the attributes is needed.

#### 6.3.3.4	Notifications

The subclause 5.5, in 3GPP TS 28.541[3], of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

6.3.4	GeoLoc <<dataType>>

#### 6.3.4.1	Definition

This datatype represent the geographical location.

#### 6.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| geographicalCoordinates | CM | T | T | F | T |
| civicLocations | CM | T | T | F | T |

#### 6.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| geographicalCoordinates Support Qualifier | Condition: If the serving location is defined as geographical coordinates [2]. |
| civicLocationsSupport Qualifier | Condition: If the serving location is defined as civic locations [2]. |

NOTE: 	Only one of the attributes is needed.

#### 6.3.4.4	Notifications

The subclause 5.5, in 3GPP TS 28.541[3], of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

6.3.5	 ECSFunction

#### 6.3.5.1	Definition

This IOC represents the ECS functionality for supporting Edge Computing. For more information about the ECS, see 3GPP TS 23.558 [2].

#### 6.3.5.2	Attributes

The ECSFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622 [4]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ecsAddress | M | T | T | F | T |
| providerIdentifier | O | T | T | F | T |
| softwareImageInfo | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| edgeDataNetworkRef | M | T | T | F | T |
| eESFunctonRef | M | T | T | F | T |

#### 6.3.5.3	Attribute constraints

None.

#### 6.3.5.x	Notifications

The common notifications defined in subclause 5.5 of 3GPP TS 28.541 [3] are valid for this IOC, without exceptions or additions.

### 6.3.6	EDNConnectionInfo <<datatype>>

#### 6.3.6.1 	Definition

This datatype represent the EDN connection information.

#### 6.3.6.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dNN | M | T | T | F | T |
| eDNServiceArea | M | T | T | F | T |

#### 6.3.6.3	Attribute constraints

None.

#### 6.3.6.x	Notifications

The subclause 5.5, in 3GPP TS 28.541[3], of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.7	TopologicalServiceArea <<dataType>>

#### 6.3.7.1	Definition

This datatype represents the topological service area.

#### 6.3.7.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellIDList | CM | T | T | F | T |
| trackingAreaIdList | CM | T | T | F | T |
| servingPLMN | CM | T | T | F | T |

#### 6.3.7.3	Attribute constraints

| Name | Definition |
| --- | --- |
| cellIDList Support Qualifier | Condition: If the serving location is defined as cell IDs [2]. |
| trackingAreaIdList Support Qualifier | Condition: If the serving location is defined as tracking area IDs [2]. |
| servingPLMN Support Qualifier | Condition: If the serving location is defined as PLMN ID [2]. |

NOTE: 	Only one of the attributes is needed.

#### 6.3.7.x	Notifications

The subclause 5.5, in 3GPP TS 28.541[3], of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.8	GeographicalCoordinates <<dataType>>

#### 6.3.8.1	Definition

This datatype represents the geographical coordinates.

#### 6.3.8.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| latitude | M | T | T | F | T |
| longitude | M | T | T | F | T |

#### 6.3.8.3	Attribute constraints

None.

#### 6.3.8.x	Notifications

The subclause 5.5, in 3GPP TS 28.541[3], of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.9	SoftwareImageInfo <<dataType>>

#### 6.3.9.1	Definition

This datatype represents the software image information.

#### 6.3.9.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| minimum Disk | M | T | T | F | T |
| minimumRAM | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| swImageRef | M | T | T | F | T |

#### 6.3.9.3	Attribute constraints

None.

#### 6.3.9.4	Notifications

The subclause 5.5, in 3GPP TS 28.541[3], of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.10	EdgeDataNetwork

#### 6.3.10.1	Definition

This IOC represent the EDN information for supporting Edge Computing. For more information about EDN, see 3GPP TS 23.558 [2].

#### 6.3.10.2	Attributes

The EdgeDataNetwork IOC includes attributes inherited from Top IOC (defined in TS 28.622[4]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ednIdentifier | M | T | T | F | T |
| eDNConnectionInfo | M | T | T | F | T |

#### 6.3.10.3	Attribute constraints

None.

#### 6.3.10.4	Notifications

The common notifications defined in subclause 5.5 of 3GPP TS 28.541 [3] are valid for this IOC, without exceptions or additions.

### 6.3.11	AffinityAntiAffinity <<datatype>>

#### 6.3.11.1	Definition

This datatype represent the affinity and anti-affinity requirements of the EAS with other EAS on the same EDN.

#### 6.3.11.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| affinityEAS | M | T | F | F | T |
| antiAffinityEAS | M | T | T | F | T |

#### 6.3.11.3	Attribute constraints

None.

#### 6.3.11.x	Notifications

The subclause 5.5, in 3GPP TS 28.541[3], of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.12	VirtualResource <<datatype>>

#### 6.3.12.1	Definition

This datatype represent the virtual resource requirements of an EAS.

#### 6.3.12.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| virtualMemory | M | T | T | F | T |
| virtualDisk | M | T | T | F | T |

#### 6.3.12.3	Attribute constraints

None.

#### 6.3.12.x	Notifications

The subclause 5.5, in 3GPP TS 28.541[3], of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.3.13	EESFunction

#### 6.3.13.1	Definition

This IOC represent the properties of a EES in a 3GPP network. For more information about EES, see 3GPP TS 23.558.

#### 6.3.13.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| eESIdentifier | M | T |  | F | T |
| eESServingLocation | M | T | T | F | T |
| eESAddress | M | T | T | F | T |
| softwareImageInfo | M | T | T | F | T |
| serviceContinuitySupport | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| eASFunctonRef | M | T | T | F | T |

Editors notes: The list of attributes is not complete.

#### 6.3.13.3	Attribute constraints

None.

#### 6.3.13.x	Notifications

The common notifications defined in subclause 5.5 of 3GPP TS 28.541 [3] are valid for this IOC, without exceptions or additions.