3GPP TSG-SA5 Meeting #143-e 	S5-223226

# e-meeting, 9 - 17 May 2022

# Source:	MATRIXX Software

# Title:	pCR TR 32.847 Add new solution for Key issue#8

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

This pCR is to introduce a new solution for Key issue#8

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

# 3	Rationale

This pCR is to introduce a new solution for Key issue#8

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

### 6.8.x	Solution#8.x Converged Charging for NSSAA from AAA-P

#### 6.8.x.1	General description

This solution addresses the Key Issue#8 for REQ-NSCH-13 and is based on use of Nchf converged charging service by AAA-P involved in UE NSSAA procedures towards a AAA-S belinging to a third party.

AAA-P interacts with CHF during NSSAA procedure for a given UE and S-NSSAI for CHF to produce appropriate CDR.

The solution applies for AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure and AAA Server triggered Slice-Specific Authorization Revocation.

AAA-P Charging characteristics pre-configured in AAA-P includes NSSAA related configuration which can be overridden by NSSAA Charging profile supplied by AAA-S.

#### 6.8.x.2	Architecture description

Figure 6.8.x.2-1: AAA-P converged charging architecture

#### 6.6.x.3	Flow description

The figure 6.8.x.3-1 below describes the high level charging procedure for Converged Charging for Network Slice-Specific Authentication and Authorization (NSSAA) procedure based on figure 4.2.9.2-1 3GPP TS 23.502 [11]:

Figure 6.8.x.3-1: Converged Charging for NSSAA from AAA-P

Steps are based on steps of figure 4.2.9.2-1 3GPP TS 23.502 [11], with the following additions:

0ch. AAA-P Charging characteristics is pre-configured in AAA-P indicating: Charging active (Yes/No) for NS authentication/auth, re-authentication/re-auth, revocation per S-NSSAI.

7. Based its local policies, AAA-S decides a dedicated NSSAA Charging profile for this GPSI and S-NSSAI should be enforced: indicating Charging active (Yes) in AAA-P for authentication/auth, re-authentication/re-auth, revocation.

7ch. Received NSSAA Charging profile overrides behaviors pre-configured AAA-P Charging characteristics for this UE and S-NSSAI.

16ch-a to c: Charging active in AAA-P: Charging Data Request [Event] is sent to CHF with the result of NSSAA Authentication (EAP-Success/Failure, S-NSSAI, GPSI) for CHF CDR generation

The solution for Converged Charging AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure are based on figure 4.2.9.3-1 3GPP TS 23.502 [11] with additional interactions with CHF from AAA-P  on AAA Protocol Re-Auth Response.

The solution for Converged Charging AAA Server triggered Slice-Specific Authorization Revocation procedure are based on figure 4.2.9.4-11 3GPP TS 23.502 [11] with additional interactions with CHF from AAA-P on AAA Protocol Revication Re-Auth Response.

End of changes