3GPP TSG-SA5 Meeting #143-e 	S5-223452

# e-meeting, 9 - 17 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0116 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update procedures for asynchronous mode of operations for Network Slice and Network Slice Subnet LCM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The procedure of reservation and checking feasibility of network slice defined in Section 7.13 in TS 28.531 is not detailed and missing some important steps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the procedure of reservation and checking feasibility of network slice |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network Slice reservation procedure is not clear in the current 7.13 procedure and hence it can not be implemented in a uniform way across the consumers/producers of different vendors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, 7.3, 7.4, 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR refers to lifecycleState and availabilityStatus attributes in NetworkSlice and NetworkSliceSubnet IOC in TS 28.541 proposed by CR0725, with tDoc# S5-223453 for TS28.541 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 7.2	Procedure of Network Slice Instance Allocation

The Figure 7.2-1 illustrates the procedure of creating a new NSI or using an existing NSI to satisfy the required network slice related requirements.

Figure 7.2-1: Network Slice Instance Allocation Request procedure

1)	Network Slice Management Service Provider (NSMS_Provider) receives an AllocateNsi request (see AllocateNsi operation defined in clause 6.5.1) from Network Slice Management Service Consumer (NSMS_Consumer) with network slice related requirements (the network slice related requirements are defined as the attributes in the ServiceProfile see clause 6.3.3 in TS 28.541 [6]).

2)	Based on the network slice related requirement and the knowledge of the capabilities of existing deployed network slices, the NSMS_Provider compare/match the provided requirements against all the candidate NetworkSlice instances, and then decides whether to use an existing NSI or create a new NSI. If the network slice related requirements allow the requested NSI to be shared and if an existing suitable NSI can be reused, the NSMS_Provider may decide to use the existing NSI.

3a-1) If using an existing NSI, the NSMS_Provider identifies the existing NetworkSlice MOI.

3a-2) The NSMS_Provider sends AllocateNsi response to the NSMS_Consumer with the DN of the NetworkSlice MOI, identified in step 3a-1.

3a-3) The identified NetworkSlice MOI needs to be modified to satisfy the network slice related requirements and the NSMS_Provider invokes the procedure to modify the existing NSI as described in clause 7.6.

3a-4) The NSMS_Provider associates the received ServiceProfile with the identified existing NetworkSlice MOI. Additionally, the NSMS_Provider may configure other configuration information for the NetworkSlice MOI. The NSMS_Provider notifies the updated NetworkSlice MOI attributes to NSMS_Consumer.

NOTE: The already running services of the re-used slice must not be impacted. Therefore neither the allocateNSI nor any of its side effects are allowed to cause: operationalState=disabled, lifecycleState=planned, or any non-empty availability status.

3b-1) If creating a new NSI, the NSMS_Provider creates the MOI NetworkSlice instance.

3b-2) The NSMS_Provider sends AllocateNsi response to the NSMS_Consumer with the DN of the NetworkSlice MOI, created in step 3b-1.

3b-3) The NSMS_Provider updates the NetworkSlice MOI instance states. Additionally, the NSMS_Provider may configure other configuration information for the NetworkSlice MOI.

The NSMS_Provider sends notification notifyMOICreation (defined in clause 6.5.2 in TS 28.541 [6] and clause 11.1.1.7 in TS 28.532 [8]) with the following attributes to NSMS_Consumer:

-	DN of the NetworkSlice MOI in attribute objectInstance.

-	The following NetworkSlice instance states in attributeList:

-	operationalState=disabled

-	lifecycleState=planned

-	availabilityState=[not_installed, dependency]

NOTE: The NetworkSlice instance states indicate that the MOI has been created, but corresponding network slice subnets are not created.

-	lifecycleState=planned: The NetworkSlice object has been created, but the resources are not yet created

-	availabilityStatus=not installed: Parts needed for the NetworkSlice are still missing

-	availabilityStatus=dependency: Dependencies for the NetworkSlice are not working

3b-4) When the Network Slice is created, the NSMS_Provider updates the NetworkSlice MOI instance states.

The NSMS_Provider sends notification notifyMOIAttributeValueChanges (defined in clause 6.5.2 in TS 28.541 [6] and clause 11.1.1.7 in TS 28.532 [8]) with the following attributes to NSMS_Consumer:

-	DN of the NetworkSlice MOI in attribute objectInstance.

-	The following NetworkSlice instance states in attributeList:

-	lifecycleState=installed

NOTE: The NetworkSlice instance states indicate that the MOI has been created.

-	lifecycleState=installed: The NetworkSlice as such has been created

-	availabilityStatus=not installed: Parts needed for the NetworkSlice are still missing

-	availabilityStatus=dependency: Dependencies for the NetworkSlice are not working

3b-5) The NSMS_Provider derives the network slice subnet related requirements from the received network slice related requirements. Before NSMS_Provider derives the network slice subnet related requirements, NSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.

3b-6)	The NSMS_Provider invokes the NSSI allocation procedure, by sending AllocateNssi request to NSSMS_Provider, as described in clause 7.3.

3b-7) The NSMS_Provider configures the NetworkSlice MOI with the DN of MOI for the NetworkSliceSubnet.

3b-8) The NSMS_Provider and send notification notifyMOIAttributeValueChanges (defined in clause 6.5.2 in TS 28.541 [6], clause 11.1.1.9 in TS 28.532 [8]) with the following attributes to NSMS_Consumer:

-	DN of the NetworkSlice MOI in attribute objectInstance.

-	The following NetworkSlice instance states in attributeValueChange:

-	updated networkSliceSubnetRef

3b-9) When allocation of NetworkSlice and all its constituents is completed, the NSMS_Provider updates the NetworkSlice MOI instance states. Additionally, the NSMS_Provider may configure other configuration information for the NetworkSlice MOI. The NSMS_Provider sends notification notifyMOIAttributeValueChanges (defined in clause 6.5.2 in TS 28.541 [6], clause 11.1.1.9 in TS 28.532 [8]) with the following attributes to NSMS_Consumer:

-	DN of the NetworkSlice MOI in attribute objectInstance.

-	The following NetworkSlice instance states in attributeValueChange:

-	operationalState=enabled

-	availabilityState=null

Note:	The detailed configuration information is described in network slice NRM (see NetworkSlice IOC defined in clause 6.3.1 in TS 28.541 [6]).

Otherwise the result may include the reason of failure, for example, the required latency or user number cannot be satisfied, or the physical resource is not enough.

## 7.3	Procedure of Network Slice Subnet Instance Allocation

The Figure 7.3-1 illustrates the procedure of creating a new network slice subnet instance or using an existing network slice subnet instance to satisfy the required network slice subnet related requirements.

Figure 7.3-1: Network Slice Subnet Instance Allocation Request procedure

1)	Network Slice Subnet Management Service Provider (NSSMS_Provider) receives an AllocateNssi request (see AllocateNssi operation defined in clause 6.5.2) from Network Slice Subnet Management Service Consumer (NSSMS_Consumer) with network slice subnet related requirements (network slice subnet related requirements defined in SliceProfile see clause 6.3.4 in TS 28.541 [6]).

2) NSSMS_Provider checks the feasibility of network slice subnet related requirements. If the network slice subnet related requirements can be satisfied, the following step 3) are needed, else go to step 5).

3)	Based on the network slice subnet related requirements and the existing NSSI capabilities, NSSMS_Provider decides whether to use an existing NSSI or create a new NSSI. If the network slice subnet related requirements allow the requested NSSI to be shared and if an existing suitable NSSI can be reused, the NSSMS_Provider decides to use the existing NSSI.

4a-1) If using an existing NSI, the NDSMS_Provider identifies the existing NetworkSliceSubnet MOI.

4a-2) The NSSMS_Provider sends AllocateNssi response to the NSSMS_Consumer with the DN of the NetworkSliceSubnet MOI, identified in step 4a-1.

4a-3) The identified NetworkSliceSubnet MOI needs to be modified to satisfy the network slice related requirements and the NSMS_Provider invokes the procedure to modify the existing NSSI as described in clause 7.7.

4a-4) The NSSMS_Provider associates the received SliceProfile with the identified existing NetworkSliceSubnet MOI. Additionally, the NSSMS_Provider may configure other configuration information for the NetworkSliceSubnet MOI. The NSSMS_Provider notifies the updated NetworkSliceSubnet MOI attributes to NSSMS_Consumer.

NOTE: The already running services of the re-used slice must not be impacted. Therefore neither the allocateNssi nor any of its side effects are allowed to cause: operationalState=disabled, lifecycleState=planned, or any non-empty availability status.

4b-1) If creating a new NSSI, the NSSMS_P creates the NetworkSliceSubnet MOI.

4b-2) The NSSMS_Provider sends AllocateNssi response to the NSSMS_Consumer with the DN of the NetworkSliceSubnet MOI, created in step 3b-1.

4b-3) The NSSMS_Provider updates the NetworkSliceSubnet MOI instance states. Additionally, the NSMS_Provider may configure other configuration information for the NetworkSliceSubnet MOI.

The NSSMS_Provider sends notification notifyMOICreation (defined in clause 6.5.2 in TS 28.541 [6] and clause 11.1.1.7 in TS 28.532 [8]) with the following attributes to NSSMS_Consumer:

-	DN of the NetworkSliceSubnet MOI in attribute objectInstance.

-	The following NetworkSliceSubnet instance states in attributeList:

-	operationalState=disabled

-	lifecycleState=planned

-	availabilityState=[not_installed, dependency]

NOTE: The NetworkSliceSubnet instance states indicate that the MOI has been created, but corresponding network slice subnets are not created.

-	lifecycleState=planned: The NetworkSliceSubnet object has been created, but the resources are not yet created

-	availabilityStatus=not installed: Parts needed for the NetworkSliceSubnet are still missing

-	availabilityStatus=dependency: Dependencies for the NetworkSliceSubnet are not working

4b-4) When the Network Slice Subnet is created, the NSSMS_Provider updates the NetworkSliceSubnet MOI instance states.

The NSSMS_Provider sends notification notifyMOIAttributeValueChanges (defined in clause 6.5.2 in TS 28.541 [6] and clause 11.1.1.7 in TS 28.532 [8]) with the following attributes to NSSMS_Consumer:

-	DN of the NetworkSliceSubnet MOI in attribute objectInstance.

-	The following NetworkSliceSubnet instance states in attributeList:

-	lifecycleState=installed

NOTE: The NetworkSliceSubnet instance states indicate that the MOI has been created.

-	lifecycleState=installed: The NetworkSliceSubnet as such has been created

-	availabilityStatus=not installed: Parts needed for the NetworkSliceSubnet are still missing

-	availabilityStatus=dependency: Dependencies for the NetworkSliceSubnet are not working

4b-5) NSSMS_Provider derives the corresponding network slice subnet constituent (i.e. NF, constituent NSS) related requirements and transport network related requirements (e.g. 3GPP endpoint information, latency requirements, bandwidth requirements and isolation requirements) from the received network slice subnet related requirements. Part of these requirements may be referenced by attribute "epTransportRef" as defined in clause 6.3.2.2 in TS 28.541[6]. Before NSSMS_Provider derives the constituent network slice subnet related requirements, NSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.2.

4b-6)	If the NSSI to be created contains virtualisation part (i.e. VNF or VL), NSSMS_Provider derives the NS instance instantiation information (the NS instance instantiation information is described in clause 7.3.2.2 and clause 7.3.3.2 [3]) based on network slice subnet related requirements. NSSMS_Provider determines new VNF instance(s) that need to be deployed and the existing VNF instance(s) that need to be reused according to the necessary network function(s) and then derives the profile of virtual link(s) according to the connection requirements between the network functions. NSSMS_Provider chooses a proper NSD deployment flavour and creates data concerning the SAPs of the NS instance. NSSMS_Provider invokes the NS instantiation procedures to create a NS instance. NSSMS_Provider configures the NSS MOI with the NS instance identifier.

Note:	NS instantiation procedure is described in TS 28.526 [7].

4b-7) For each required NSSI constituent, the following step 4b-7a) and 4b-7b) are needed:

4b-7a) If the required NSSI constituent is constituent NSSI, NSSMS_Provider invokes NSSI Allocation Procedure.

4b-7b) If the required NSSI constituent is NF instance, NSSMS_Provider invokes NF Creation Procedure as described in clause 7.10 or NF Modification Procedure as described in clause 7.11.

4b-8)	 NSSMS_Provider configures the NetworkSliceSubnet MOI with the DN of the MOI for NSSI constituent (i.e. ManagedFunction MOI, NetworkSliceSubnet MOI).

4b-9)	 For each required transport network related requirements, NSSMS_Provider invokes corresponding procedure of coordination with relevant TN Manager to handle the TN part as described in clause 7.9.

4b-10) When allocation of NetworkSliceSubnet and all its constituents is completed, the NSSMS_Provider updates the NetworkSliceSubnet MOI instance states. Additionally, the NSSMS_Provider may configure other configuration information for the NetworkSliceSubnet MOI. The NSSMS_Provider sends notification notifyMOIAttributeValueChanges (defined in clause 6.5.2 in TS 28.541 [6], clause 11.1.1.9 in TS 28.532 [8]) with the following attributes to NSSMS_Consumer:

-	DN of the NetworkSliceSubnet MOI.

-	NS instance Info (e.g. NSinstanceId)

-	The following NetworkSlice instance states in attributeValueChange:

-	operationalState=enabled

-	availabilityState=null

-	DN of the NetworkSliceSubnet MOI.

-	NS instance Info (e.g. NSinstanceId)

Otherwise the result may include the reason of failure, for example, the required latency or user Number cannot be satisfied, or the physical resource is not enough.

End of change