3GPP TSG-SA5 Meeting #143-e 	S5-223098

# e-meeting, 9 - 17 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0392 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting home routed CHF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Using the VPLMN to select HPLMN CHF would lead to that a VPLMN handover could then lead to a HPLMN CHF change in the middle of a charging session without any notification to the first CHF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing use of VPLMN as a suggestion to select H-CHF on. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Changing the CHF in the middle of the session may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.1.9.2	CHF selection

In roaming Home routed scenario, at PDU session establishment, the CHF selection mechanism specified in clause 5.1.8 applies to:

-	The V-SMF for CHF selection in VPLMN, with the following differences:

-	CHF address(es) selection mechanisms based on PCF and UDM are not applicable.

-	when charging characteristics is used it will be based on local configuration;

-	when NRF is used, the V-CHF can be selected based on UE identified as in-bound roamer and the PLMN Id of the H-PLMN;

-	The H-SMF for CHF selection in HPLMN, with the following difference: when NRF is used, the H-CHF can be selected as for non-roaming case.

In roaming Home routed PDU session, upon V-SMF change:

-	intra-PLMN V-SMF change: CHF address supplied by the old V-SMF shall be used.

-	inter-PLMN V-SMF change: CHF selection mechanism as per V-SMF CHF selection in VPLMN at PDU session establishment.

End of changes