3GPP TSG-SA5 Meeting #143-e 	S5-223324

# e-meeting, 9 - 17 May 2022

# Source:	Huawei, China Mobile

# Title:	pCR TR 28.909 Add key issues for KEI and process of autonomous network levels evaluation for network optimization

# Document for:	approval

Agenda Item:	6.5.2.2

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP draft TR 28.909: “Management and orchestration; Study on evaluation of autonomous network levels v0.1.0”.

3	Rationale

This contribution proposes to add key issues for generic methodology for autonomous network levels evaluation based on concept for autonomous network level evaluation in clause 4.1 to reflect which aspects needs to be considered

4	Detailed proposal

It proposes to make the following changes to TR 28.909[1].

1st Change

5.X	Key Issue# 5.X: KEI and process of autonomous network levels evaluation for radio network optimization

5.X.1	Description

Generic autonomous network level for network optimization is defined in TS 28.100 [2], following clauses provides the potential solution for corresponding KEI of ANL evaluation and process of ANL evaluation for radio network optimization.

5.X.2		Potential solutions for KEI of ANL evaluation

5.X.2.1	Potential consideration

Following aspects can be considered as the effect for introducing autonomy capability for radio network optimization:

-	Autonomous optimization effect aspect, the autonomous optimization effect for corresponding tasks of radio network optimization by introducing autonomy capability. Using task of network issue root cause analysis for example, telecom system A with autonomy capability can analysis the root cause for 90% issue cells (the root case of other 10% issue cell still needs to be ananlysed by human), however, telecom system B with autonomy capability only analysis the 10% issue cells (the root case of other 90% issue cell still needs to be ananlysed by human). The effect of introducing root cause autonomy capability for telecom system A and telecom system B is different.

-	Network performance improvement effect aspect, the performance improvement by introducing autonomy capability for radio network optimization. For example, Using coverage performance for example, telecom system A improve the 30% coverage performance by introducing the autonomy capability for corresponding tasks, however, telecom system B only improve 5% coverage performance by introducing the autonomy capability for corresponding tasks. In this case, the effect of introducing autonomy capability for telecom system A and telecom system B is different.

-	Optimization efficiency effect aspect, the efficiency improvement by introducing autonomy capability for radio network optimization. For example, telecom system A take one day to optimize the radio network by introducing the autonomy capability for corresponding tasks, however, telecom system B take 5 days to optimize the radio network by introducing the autonomy capability for corresponding tasks. In this case, the effect of introducing autonomy capability for telecom system A and telecom system B is different.

5.X.2.2	Potential KEI of ANL evaluation

TBD

5.X.3		Potential solutions for process of ANL evaluation

TBD

End of Changes