3GPP TSG-SA5 Meeting #144-e 	S5-224077

# e-meeting, 27 June- 1 July 2022

# Source:	AsiaInfo

# Title:	Add use case of Intent driven management for network slice management capability exposure

# Document for:		Approval

Agenda Item:	6.7.4.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1] 	3GPP TR 28.836: “Study on intent-driven management for network slicing v0.2.0”.

3	Rationale

This document is to add use case of Intent driven management for network slice management capability exposure regarding network slice instance and network slice subnet instance.

4	Detailed proposal

The start of the change

5.y	Intent driven management for network slice management capability exposure

5.y.1 	Use case of Intent driven management to express expectation of network slice management capability exposure

5.y.1.1	Introduction

To satisfy the requirement of authorized network slice management service consumer to obtain certain management capability to manage the network slice instance through the exposure interface. the intent driven MnS producer receive the intent expectation of network slice management capability exposure and provide the management capabilities to support exposure of network slice instance management capability.

5.y.1.2 	Pre-condition

CSP provides the intent driven management service to CSC to create a communication service.

Network operator is capable to provide network slice instance management capability exposure in management system to invoke the proper intent driven MnS.

5.y.1.3	Description

In order to enable a group of end users to obtain the certain management capability provided by a 3GPP network to manage the network slice instance, a MnS Consumer expresses its intent for the network slice instance management capability with proper network characteristics to network operator.

The operator provides the expectation of exposure of network slice instance management capability with a set of network characteristics. The intent diven management service producer is capble to translate the intent of exposure of network slice instance management capability to relevant requirements e.g., obtain capabilities of KPI monitoring and alarm notification. Regarding the intent MnS request received, the intent driven-management service provider may decide management capabilities to meet the requirements of obtaining network slice instance management capabilities, for example management capability exposure. The intent driven management service producer is also capable to provide the report of intent fulfilment of network slice instance management capability exposure addressing the certain management capabilities of intent.

5.y.1.4	Post-condition

The network slice instance management capability exposure expectation expressed by intent for a specified group of end users is fulfilled.

5.y.1.5	Requirements

REQ-Intent_Expose_Slice-CON-1: The intent driven MnS shall have capability enabling the authorized MnS consumer to express intent containing an expectation for network slice instance management capabilities exposure.

5.y.2 	Use case of Intent driven management to express expectation of network slice subnet instance management capabilities exposure

5.y.2.1	Introduction

To satisfy the requirement of authorized network slice subnet instance management service consumer to obtain certain management capability to manage the network slice subnet instance through the exposure interface. the intent driven MnS producer receive the intent expectation of network slice subnet management capability exposure and provide the management capabilities to support exposure of network slice subnet instance management capability.

5.y.2.2 	Pre-condition

NOP provides the intent driven management service to create a network slice subnet instance.

Network operator is capable to provide network slice subnet instance management capability exposure in management system to invoke the proper intent driven MnS.

5.y.2.3	Description

In order to enable a group of end users to obtain the certain management capability provided by a 3GPP network to manage the network slice subnet instance, a MnS Consumer expresses its intent for the network slice subnet instance management capability with proper network characteristics to network operator.

The operator provides the expectation of exposure of network slice subnet instance management capability with a set of network characteristics. The intent diven management service producer is capble to translate the intent of exposure of network slice subnet instance management capability to relevant requirements e.g., obtaining measurement, updating resource allocations. Regarding the intent MnS request received, the intent driven-management service provider may decide management capabilities to meet the requirements of obtaining network slice subnet instance management capabilities, for example management capability exposure. The intent driven management service producer is also capable to provide the report of intent fulfilment of network slice subnet instance management capability exposure addressing obtain the certain management capabilities of intent.

5.y.2.4	Post-condition

The network slice subnet instance management capability exposure expectation expressed by intent for a specified group of end users is fulfilled.

5.y.2.5	Requirements

REQ-Intent_Expose_Slice-CON-2: The intent driven MnS shall have capability enabling the authorized MnS consumer to express intent containing an expectation for network slice subnet instance management capabilities exposure.

The end of the change