| 3GPP TR 28.903 V0.3.0 (2022-07) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on alignment with ETSI MEC for Edge computing management (FS_MEC_ECM) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Related Work in Other SDOs	7

4.1 ETSI MEC	7

4.2 GSMA OPG	8

5	Issue investigations and potential solutions for alignment with ETSI MEC	8

5.X Key Issue#Num: title	8

5.X.1 Description	8

5.X.2 Potential solutions	8

6	Issue investigations and potential solutions for alignment with GSMA OPG	8

6.X Key Issue# Num: title	8

6.X.1 Description	8

6.X.2 Potential solutions	8

7	Conclusion and recommendation	9

Annex X (informative): Change history	10

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This report is to study on alignment with ETSI MEC for Edge computing management.

1	Scope

The present document studies the solutions to support alignment with ETSI MEC from the management perspective for edge computing.

In this TR, it also identifies potential issues and solutions to support GSMA OPG [2] requirement in coordination with ETSI MEC [3].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	The NBI requirements from GSMA OPG - https://www.gsma.com/futurenetworks/wp-content/uploads/2021/06/OPG-Telco-Edge-Requirements-2021.pdf

[3]	ETSI GS MEC 010-2 (v 2.2.1) (2022-02): " Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

[4]	ETSI ISG MEC 003, "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[5]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[6]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management (ECM)".

[7]	ETSI GS NFV-IFA 013 V3.4.1 "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Os-Ma-nfvo reference point -Interface and Information Model Specification".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Related Work in Other SDOs

4.1 ETSI MEC

Multi-access Edge Computing (MEC) framework as specified in ETSI ISG MEC 003 [4] enables the implementation of MEC applications as software-only entities that run on top of a Virtualisation infrastructure, which is located in or close to the network edge.

Figure 4.1-1: Multi-access Edge Computing framework

Figure 4.1-1 shows the MEC framework as defined in [4], it mainly contains several components,

The MEC host is an entity that contains a MEC platform and a Virtualisation infrastructure which provides compute, storage, and network resources, for the purpose of running MEC applications.

The MEC platform is the collection of essential functionality required to run MEC applications on a particular Virtualisation infrastructure and enable them to provide and consume MEC services. The MEC platform can also provide services.

MEC applications are instantiated on the Virtualisation infrastructure of the MEC host based on configuration or requests validated by the MEC management.

The MEC system level management includes the MEC orchestrator as its core component, which has an overview of the complete MEC system. When relying on the NFV Orchestrator (NFVO) for resource orchestration and for orchestration of the set of MEC application VNFs, the MEC Orchestrator (MEO) is replaced by a MEC Application Orchestrator (MEAO), the architecture is described in clause 6.2.2 of [4].

The MEC host level management comprises the MEC platform manager and the Virtualisation infrastructure manager, and handles the management of the MEC specific functionality of a particular MEC host and the applications running on it.

In Annex C of 3GPP TS 23.558 [5], both EDGEAPP and ETSI MEC can provide support for hosting different edge applications, it further indicates that "Both EAS and MEC application are application servers and can provide similar application specific functionalities. EAS utilizes the services of EES as specified in this document whereas MEC application utilizes the services provided by MEC platform as specified in ETSI GS MEC 003 [4]. The EAS and MEC application can be collocated in an implementation.".

4.2 GSMA OPG

The Operator Platform (OP) is defined by GSMA OPG [2], it facilitates access to the Edge Cloud capability of an Operator or federation of operators and their partners.

The architecture scope under definition is shown below,

Figure 4.2-1: OP Roles and Interfaces Reference Architecture

This study is focused on the NBI requirements as defined in [2]. The NBI is the interface between the application provider and the Operator Platform, it allows an OP to advertise the above cloud capabilities that it can provide to application providers. In addition, the NBI allows an application provider to reserve a set of resources or request an Edge Cloud service with the resources and features that they require and for the OP to accept or reject the request.

In this study, it is assumed that the application provider as described in [2] maps to ASP in TS 28.538 [6], and the Operator Platform as described in [2] maps to ECSP management system in TS 28.538 [6].

Editor’s note: Functions mapping between Operator Platform and ECSP management system is FFS.

5	Issue investigations and potential solutions for alignment with ETSI MEC

## 5.1	Key Issue# 1: Management of collocated platforms for EES and MEC platform

5.1.1	Description

As described in [5], one of the deployment option is that the EES and MEC platform can be collocated in an implementation.

The EES instantiation procedure is described in clause 7.1.4 of TS 28.538 [6], which the PLMN operator or ECSP requests the EES instantiation via provisioning MnS to ECSP management system, the simplified procedure is show in Figure 5.1.1-1,

Figure 5.1.1-1: EES deployment procedure

The MEC platform as described in ETSI ISG MEC 003 [4] can be deployed as a VNF by using ETSI NFV MANO, the simplified procedure is show in Figure 5.1.1-2,

Figure 5.1.1-2: MEC platform deployment procedure

In the collocated scenario, the EES and MEC platform can be deployed in the same Physical/NFV Infrastructure by the same operator. The MEC platform can be deployed first, then the operator can deploy the EES at the same Physical/NFV Infrastructure where the MEC platform is deployed.

REQ-EES-deplyment-1: ECSP management system shall have the capability to instantiate the EES to a specific Physical/NFV Infrastructure that collocated with MEC platform.

## 5.2	Key Issue# 2: Application lifecycle management

### 5.2.1	Description

3GPP TS 28.538 [6] specifies the EAS lifecycle management procedure by interworking with ETSI NFV MANO by invoking InstantiateNsRequest or UpdateNsRequest operation as specified in ETSI NFV IFA013 [7]. An NFV Network Service (NS) as defined by ETSI NFV is a composition of Network Functions (NF) (VNF or PNF).

With ETSI MEC [3], MEO (when ETSI NFV is not involved) or MEAO (when ETSI NFV is involved) provides the application instance lifecycle management interface, which is similar to VNF LCM interface. The consumer of MEO or MEAO does not need to take creation of NS into consideration when requesting creation of an application instance.

The main issue related to application LCM is the following:

1, when and how ECSP management system create an NS when receiving the creating EAS request from ASP.

2, whether the application instance lifecycle management interface produced by MEO or MEAO could be used for EAS lifecycle management.

### 5.2.2	Potential solutions

TBD

6	Issue investigations and potential solutions for alignment with GSMA OPG

## 6.1	Key Issue# 1: Issue for resource reservation

As described in clause 2.2.2.3 GSMA OPG [2], resource reservation is one of its NBI requirements, it indicates that,

- The OP (Operator Platform) shall enable Application Providers to express the resource (e.g., compute, networking, storage, acceleration) requirements that the Application Provider wants to be guaranteed.

- The NBI shall allow an Application provider to request a set of resources to be booked.

- The NBI allows an Application Provider to reserve resources ahead of the application onboarding and unrelated to any specific application, only related to the Application Provider themselves. The NBI allows an Application Provider to consume the reserved resources when onboarding a new application, creating the association between the resources and the application (resources allocation). The NBI allows an Application Provider to delete their reservation.

TS 28.538 [6] defines the lifecycle management procedure for EAS with ETSI NFV MANO, while the resource reservation related to the ASP requirements is not specified in [6].

It is important to investigate the method for ASP to request the resource reservation.

REQ-resource-reservation-1: ECSP management system shall have a capability allowing ASP to request resource reservation related to virtualisation resources (e.g., compute, networking, storage, acceleration).

REQ-resource-reservation-2: ECSP management system shall have a capability to reserve a set of resources for ASP unrelated to any specific application.

Editor’s note: Resource reservation is FFS

7	Conclusion and recommendation

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022.04 | SA5#142e | S5-222103 |  |  |  | Initial version of TR28.903 | 0.0.0 |
| 2022.04 | SA5#142e | S5-222329 S5-222330 | - | - | - | Add scope Add skeleton | 0.1.0 |
| 2022.05 | SA5#143e | S5-223267 | - | - | - | Add background information of ETSI MEC | 0.2.0 |
| 2022.07 | SA5#144e | S5-224438 S5-224439 S5-224440 S5-224441 |  |  |  | adding key issue for collocated management with MEC adding key issue for application lifecycle management adding key issue for resource reservation adding background information for GSMA OPG | 0.3.0 |