- 3GPP TSG-SA5 Meeting #	S5- 224020

- June 2022, E-meeting

- Source:	Nokia

- Title:	pCR 28.908 Requirements on AI/ML context

- Document for:	Approval

- Agenda Item:	6.7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.908-010 “Study on Artificial Intelligence/Machine Learning (AI/ ML) management”.

3	Rationale

Differences in the network context or the network status applied to AI/ML, e.g. under which datais collected, significantly affects  the outcomes of the AI/ML services. context, As such management capabilities are needed to enable awareness of the AI/ML context in the production of AI/ML services. This pCR adds the UC and potential requirements on AI/ML context identification, monitoring and reporting in relation to AI/ML model performance management as described in TR 28.908 Section 5.1.

# 4	Detailed proposal

Start of modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[4]	3GPP TS 28.105-17.0.0 "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management"

Next change

# X	Use cases, potential requirements and possible solutions

## X.1	AI/ML context

### X.1.1	Description

AIMLContext represents the status and conditions related to the AIMLEntity (cf. TS 28.105[4]). This may include the network context as defined in TS 28.104[2] as well as other conditions that may be applicable to the AIMLEntity but are not part of network characteristics e.g. the time of day, season of the year. As part of AI/ML model performance management there is identification of the problem that the AI/ML model is facing. As described in TS 28.104[2], the differences in the network context, i.e., network status, under which data is collected to produce analytics, significantly affect the produced analytics. Similary, the changes in the AI/ML context, e.g., the characteristics of the data related to the network status and conditions used for AI/ML model training, testing and deployment may affect the AI/ML entity performance, thus may represent a problem for the AI/ML entity. Thus management capabilities are needed to enable awareness of the AI/ML context in terms of the identification as well as monitoring and reporting of changes in AI/ML context as part of the identification of the problem that the AI/ML entity is facing.

### X.1.2	Use cases

#### X.1.2.1	AI/ML context monitoring and reporting

AI/ML context related to AI/ML model training, testing and deployment needs to be identified by characterizing the input data used by the AI/ML model is targeted to work. As an example, such characterization may be done based on the statistical properties of data. Monitoring of such AI/ML context serves to detect the changes and anomalies in the AI/ML context. Some anomalies may be considered as a problem that AI/ML entity is facing as it may lead to its performance degradation. Therefore, the consumer of the related AI/ML service needs to be informed about such observed AI/ML context change.

### X.1.3	Potential requirements

REQ-1 The producer AI/ML-related MnS including for training and inference should have a capability to identify and monitor the AI/ML context, as well as to inform the consumer about observed changes in AI/ML context ,

### X.1.4	Possible solutions

TBD

End of modifications