- 3GPP TSG-SA5 Meeting #144e	S5-224267rev1

- 27 June - 01 July 2022, E-meeting

- Source:	Intel, NEC

- Title:	pCR 28.908 Add use case on AI/ML entity validation

- Document for:	Approval

- Agenda Item:	6.7.5.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.908-000 “Study on Artificial Intelligence/Machine Learning (AI/ ML) management”.

# 3	Rationale

This pCR is to add the UC and potential requirements on AI/ML model validation.

# 4	Detailed proposal

Start of modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[a]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management".

Start of modification

## 5.x	AI/ML entity validation

### 5.x.1	Description

During the AI/ML training process, the generated AI/ML entity (see TS 28.105 [a]) needs to be validated. The purpose of AI/ML validation is to evaluate the performance of the AI/ML entity when performing on the validation data, and to identify the variance of the performance on the training data and the validation data. If the variance is not acceptable, the entity would need to be tuned (re-trained) before being made available to the consumer.

The training data and validation are normally split from the same data set with a certain ratio, therefore they have the same distribution and pattern.

### 5.x.2	Use cases

#### 5.x.2.1	AI/ML entity validation performance reporting

In the AI/ML training, the AI/ML entity is generated based on the learning from the training data, and validated using validation data. The performance of the AI/ML entity has tight dependency on the data (i.e., training data) from which the AI/ML entity is generated. Therefore, an AI/ML entity performing well on the training data may not perform well on other data. If the performance of AI/ML entity is not good enough as result of AI/ML validation, the AI/ML entity will be tuned (re-trained) and validated again. The process of AI/ML entity generation and validation is repeated by the AI/ML training function, until the performance of the AI/ML entity meets the expectation on both training data and validation data. The producer in the end selects one or more AI/ML entities with the best level performance on both training data and validation data as the result of the AI/ML training, and reports to the consumer. The performance of each selected AI/ML entity on both training data and validation data also need to be reported.

The performance result of the validation may also be impacted by the ratio of the training data and validation data. Consumer needs to be aware of the ratio of training data and validation data, besides the performance score on each data set, in order to be confident about the performance of AI/ML entity.

### 5.x.3	Potential requirements

REQ-MODEL_VLD-CON-1	The AI/ML Training MnS producer should have a capability to validate the AI/ML entities during the training process, and report the performance of the AI/ML entities on both the training data and validation data to the authorized consumer.

REQ-MODEL_VLD-CON-2	The AI/ML Training MnS producer should have a capability to report the ratio of the training data and validation data used for training of an AI/ML entity.

### 5.x.4	Possible solutions

TBD

### 5.x.5	Evaluation

TBD

End of modifications