3GPP TSG-SA5 Meeting #145-e 	S5-225432rev1

- e-meeting, 15th– 24th August 2022

- Source:	Huawei

- Title:	Add the solution of V-SMF connects to V-CHF with two charging sessions

- Document for:	Discussion

Agenda Item:	7.5.3

# 1	Decision/action requested

Add the solution of V-SMF connects to V-CHF with two charging sessions.

# 2	References

[1]	3GPP TR 28.827 Study on 5G charging for additional roaming scenarios and actors

# 3	Rationale

Propose the additional solution about the V-SMF connects to V-CHF with two charging sessions

# 4	Detailed proposal

First change

#### 7.1.4.x	Solution #1.x: V-SMF connects to V-CHF with two charging sessions

##### 7.1.4.x.1	General

A possible solution for key issue #1a, covering requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02, the charging information for 5G data connectivity provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF), From the standpoint of the V-SMF, there are two charging sessions between V-SMF and V-CHF, covering local breakout case.

##### 7.1.4.x.2	Reference architecture

The reference architecture would be the same as Figure 7.2.4.1.2-2.

##### 7.1.4.x.3	Message flows

For the V-SMF communicating with the H-CHF, the V-CHF is responsible for sending the charging messages between V-SMF and H-CHF.

Figure 7.1.4.x.3-1: Message flow for two charging sessions

During the PDU session establishment, V-SMF establishs the two charging sessions in the separate charging resource with V-CHF.

In the interaction between the V-SMF and V-CHF, V-SMF reports the charging information to V-CHF.

In the interaction between the V-SMF and H-CHF, V-SMF reports the charging information to the V-CHF in the charging session between the V-SMF and V-CHF, V-CHF sends the charging information to H-CHF in the charging session between the H-CHF and V-CHF.

For QBC, the triggers and charging information are same for both charging sessions.

For FBC, trigger does not have synchronization mechanism, i.e. for V-CHF and H-CHF, it’s possible that the FBC charging information and trigger are not same for the two charging sessions.

End of change