3GPP TSG-SA5 Meeting #145-e 	S5-225435rev4

- e-meeting, 15th– 24th August 2022

- Source:	Huawei

- Title:	Add the solution of V-SMF controls reporting in the single charging session

- Document for:	Discussion

Agenda Item:	7.5.3

# 1	Decision/action requested

Add the solution of V-SMF connects to V-CHF with single charging session.

# 2	References

[1]	3GPP TR 28.827 Study on Nchf charging services phase 2 improvements and optimizations

# 3	Rationale

Proposal about the additional solution of V-SMF connects to V-CHF with single charging session and V-SMF is responsible for the charging information reporting for V-CHF and H-CHF.

# 4	Detailed proposal

First change

#### 7.2.4.a	Solution #2.a: V-SMF selected charging information for reporting to H-CHF

##### 7.2.4.a.1	General

A possible solution for key issue #2a and key issue#2b, covering requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02.the charging information for 5G data connectivity provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF) in single charging session, V-SMF is responsible for the charging reporting for the V-CHF and H-CHF.

##### 7.2.4.a.2	Reference architecture

The reference architecture would be the same as 7.2.4.1.2-2.

##### 7.2.4.a.3	Message flows

During the PDU session establishment, V-SMF establishes the sinlge charging session with V-CHF, same with the figure 7.1.4.y.3-1 in Solution #1.x: V-SMF connects to V-CHF with single charging sessions.

V-SMF is responsible for distinguishing the charging information for H-CHF and V-CHF. The V-CHF sends the charging information between V-SMF and H-CHF.

1.	The V-SMF sends charging data request to V-CHF with both the charging information for V-CHF and charging information for H-CHF with the indication;

2.	Based on the indication, V-CHF handles the charging information for V-CHF;

3.	Based on the indication, V-CHF sends charging information to H-CHF;

4.	H-CHF sends the charging data response to V-CHF;

5.	V-CHF sends charging data response from H-CHF and/or sends the charing data response from V-CHF its own.

7.2.4.b	Solution #2.b: V-SMF setting charging information indication by information element group

##### 7.2.4.b.1	General

A possible solution for key issue #2a and key issue#2b, covering requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02.the charging information for 5G data connectivity provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF) in single charging session, V-SMF is responsible for the charging reporting for the V-CHF and H-CHF, V-SMF sets a indication for a group of information elements for V-CHF, and sets another indication for the other group of information elements for H-CHF.

##### 7.2.4.b.2	Reference architecture

The reference architecture would be the same as 7.2.4.1.2-2.

##### 7.2.4.b.3	Message flows

In step 1 of message flow 7.2.4.a.3, take the following charging information of FBC for example, the V-SMF set two indications for two groups of information elements, one indication is for V-CHF, the other indication is for H-CHF.

Table 7.2.4.b.3: Structure of Roaming QBC Information

Multiple Unit Usage (Group2 for V-CHF)

Rating Group

Used Unit Container

Target Indication:V-CHF

Multiple Unit Usage (Group2 for H-CHF)

Rating Group

Requested Unit

Used Unit Container

Target Indication:H-CHF

End of change