3GPP TSG-SA5 Meeting #146 	S5-226086

Toulouse, France, 14 -18 November 2022

# Source:	3GPP SA6

# Title:	LS on related EAS

# Document for:	Information, Discussion

Agenda Item: 6.1

3GPP TSG-SA WG6 Meeting #51-e	S6-223029

e-meeting, 10th – 19th October 2022	(revision of S6-222969)

Title:	LS on related EAS

Response to:

Release:	Rel-18

Work Item:	FS_eEDGEAPP

Source:	SA6

To:	SA4

Cc:	SA2, SA5

Contact person:	Wenliang Xu

wenliang.xu@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

In the past, SA6 received SA4 LS S4-211658 and replied with S6-220351 for the following question saying this case will be studied in Rel-18:

- Is a mechanism defined to ensure that sets of related EAS instance(s) offering a service to the Application Client can be executed, discovered and relocated together in cases where this is necessary for the proper operation of the service? Independent discovery and relocation may not address the requirements of certain media services.

In Rel-18 TR 23.700-98, SA6 put KI#18 related to this topic and KI#18 has a gaming example including related game engine service and rendering service in different EAS instances.

In order to better understand the “related EAS instances” scenario and requirement, SA6 would like to seek further clarifications from SA4.

In SA6 understanding, the application layer communication can be designed in different ways. Below figure gives the abstract communication model between AC (Application Client) in the UE and related EAS x and y:

- In case 1, AC needs to communicate with both EAS x and y, but EASs does not need to communicate with each other.

- In case 2, AC only needs to communicate with EAS x, and EAS x consumes services provided by EAS y.

- In case 3, it is a combination of case 1 and case 2, where AC needs to communicate with both EAS x and y (e.g. AC discovers EAS x and y and connects to them), and inter-EAS communication is also needed (e.g. EAS x discovers EAS y and connects with it).

Question 1 to SA4: Is any of above scenarios for “related EAS instances” aligned with SA4 understanding?

Question 2 to SA4: If not, what is the scenario for “related EAS instances”?

In S4-211658, SA4 mentioned “Independent discovery and relocation may not address the requirements of certain media services.”

Question 3 to SA4: What are the requirements of certain media service for “related EAS instances” scenario?

# 2	Actions

To SA4

ACTION: 	SA6 asks SA4 to provide answer for above questions.

# 3	Dates of next TSG SA WG 6 meetings

SA6#52-e              14th November – 18th November 2022 	meeting

SA6#52-bis-e        16th January – 25th January 2023 	meeting