| 3GPP TS 28.908 V0.4.0 (2022-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Artificial Intelligence / Machine Learning (AI/ML) management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Concepts and overview	9

4.1	Overview	9

4.2	AI/ML workflow for 5GS	9

4.2.1	AI/ML operational workflow	9

4.2.2	AI/ML management capabilities	10

5	Use cases, potential requirements and possible solutions	11

5.1	AI/ML model performance management	11

5.1.1	Description	11

5.1.2	Use cases	11

5.1.2.1	AI/ML model performance indicators	11

5.1.2.2	Monitoring and control of AI/ML behavior	11

5.1.3	Potential requirements	12

5.1.4	Possible solutions	12

5.1.5	Evaluation	12

5.2	Event data for ML training	12

5.2.1	Description	12

5.2.2	Use cases	12

5.2.2.1	Pre-processed event data for ML training	12

5.2.3	Potential requirements	13

5.2.4	Possible solutions	13

5.2.5	Evaluation	13

5.3	AI/ML entity validation	13

5.3.1	Description	13

5.3.2	Use cases	14

5.3.2.1	AI/ML entity validation performance reporting	14

5.3.3	Potential requirements	14

5.3.4	Possible solutions	14

5.3.4.1	Validation performance reporting by enhancing the existing IOC	14

5.3.5	Evaluation	14

5.4	AI/ML entity testing	14

5.4.1	Description	14

5.4.2	Use cases	15

5.4.2.1	Consumer-requested AI/ML entity testing	15

5.4.2.2	Control of AI/ML entity testing	15

5.4.3	Potential requirements	16

5.4.4	Possible solutions	16

5.4.4.1	NRM based solution	16

5.4.5	Evaluation	17

5.5	AI/ML deployment	18

5.5.1	Description	18

5.5.2	Use cases	18

5.5.2.1	AI/ML deployment information retrieved by consumer	18

5.5.3	Potential requirements	18

5.5.4	Possible solutions	18

5.5.5	Evaluation	18

5.6	AI/ML Inference History	19

5.6.1	Description	19

5.6.2	Use cases	19

5.6.2.1	Tracking AI/ML inference decision and context	19

5.6.3	Potential requirements	19

5.6.4	Possible solutions	19

5.6.5	Evaluation	20

5.7	AI/ML context	20

5.7.1	Description	20

5.7.2	Use cases	20

5.7.2.1	AI/ML context monitoring and reporting	20

5.7.2.2	Mobility of AIML Context	21

5.7.2.3	Standby mode for AI/ML Functionality	21

5.7.3	Potential requirements	22

5.7.4	Possible solutions	22

5.7.4.1.	AIMLContext datatype on AIMLEntity	22

5.7.5	Evaluation	22

5.8	AI/ML Entity Capability Discovery and Mapping	23

5.8.1	Description	23

5.8.2	Use cases	23

5.8.2.1	Identifying capabilities of AI/MLEntities	23

5.8.2.2	Mapping of the capabilities of AI/MLEntities	23

5.8.3	Potential requirements	24

5.8.4	Possible solutions	24

5.8.5	Evaluation	25

5.9	AI/ML update management	25

5.9.1	Description	25

5.9.2	Use cases	26

5.9.2.2	AI/ML entities updating initiated by producer	26

5.9.3	Potential requirements	26

5.9.4	Possible solutions	26

5.9.4.1	Possible solution#1	26

5.9.5	Evaluation	26

5.10	AI/ML configuration management for RAN domain ES	26

5.10.1	Description	26

5.10.2	Use cases	27

5.10.2.1	AI/ML entity configuration initiated by consumer	27

5.10.2.2	AI/ML entity configuration initiated by producer	27

5.10.2.3	Enabling partial activation of AI/ML capabilities	27

5.10.3	Potential requirements	28

5.10.4	Possible solutions	28

5.10.5	Evaluation	28

5.11	Orchestrating AI/ML Inference	28

5.11.1	Description	28

5.11.2	Use cases	28

5.11.2.1	Knowledge sharing on executed actions	28

5.11.2.2	Knowledge sharing on impacts of executed actions	29

5.11.2.3	Abstract information on impacts of executed actions	29

5.11.2.4	Triggering execution of AI/ML inference functions or AI/MLEntities	30

5.11.2.5	Orchestrating decisions of AI/ML inference functions or AI/MLEntities	30

5.11.3	Potential requirements	31

5.11.4	Possible solutions	31

5.11.5	Evaluation	31

5.12	Coordination between the AI/ML capabilities	31

5.12.1	Description	31

5.12.2	Use cases	32

5.12.2.1	Alignment of the AIML capability between 5GC/RAN and 3GPP management system	32

5.12.3	Potential requirements	32

5.12.4	Possible solutions	32

5.12.5	Evaluation	32

5.13	AI/ML entity re-training	32

5.13.1	Description	32

5.13.2	Use cases	33

5.13.2.1	Producer-initiated threshold-based AIML Retraining	33

5.13.2.2	Efficient AI/ML entity re-training	33

5.13.3	Potential requirements	33

5.13.4	Possible solutions	33

5.13.5	Evaluation	33

Annex X (informative): Change history	34

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document studies the Artificial Intelligence / Machine Learning (AI/ML) management capabilities and services for 5GS where AI/ML is used, including management and orchestration (e,g., MDA, see TS 28.104 [2]) and 5G networks (e.g., NWDAF, see TS 23.288 [3]).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[4]	3GPP TS 28.105: " Artificial Intelligence / Machine Learning (AI/ML) management ".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] ], TS 28.105 [4] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and overview

## 4.1	Overview

Artificial Intelligence/Machine Learning (AI/ML) techniques are being embraced by telecommunication service providers around the world to facilitate enabling the existing and the new challenging use cases that 5G offers. AI/ML capabilities are being increasingly adopted in mobile networks as a key enabler for wide range of features and functionalities that maximise efficiency and bring intelligence and automation in various domains of the 5GS. For example, these include the Management Data Analytics (MDA) in the management and orchestration [1], the Network Data Analytics Function in the 5G core network domain [3].

The AI/ML inference functions in the 5GS use the ML model for inference and in order to enable and facilitate the AI/ML adoption, the ML model needs to be created, trained and then managed during its entire lifecycle.

To enable, facilitate and support AI/ML-capabilities in the 5GS, the following management capabilities are studied in this report:

- Validation of ML model or entity

- Testing of ML model or entity (before deployment)

- Deployment of ML model or entity (new or updated model/entity)

- Configuration of ML entity and AI/ML inference function

- Performance evaluation of ML entity and AI/ML inference function

NOTE: The ML model training capability is specified in TS 28.105 [4].

## 4.2	AI/ML workflow for 5GS

### 4.2.1	AI/ML operational workflow

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN and management system), and the generic workflow of the operational steps in the lifecycle of an ML model or entity, is depicted in the figure 4.2.1-1.

Figure 4.2.1-1: AI/ML operational workflow

The workflow involves 3 main phases; the training, deployment and inference phase, including the main operational tasks for each phase. These are briefly described below:

Training phase:

-	ML Training: Learning by the Machine from the training data to generate the (new or updated) ML entity (see TS 28.105 [4]) that could be used for inference. The ML Training may also include the validation of the generated ML entity to evaluate the performance variance of the ML entity when performing on the training data and validation data. If the validation result does not meet the expectation (e.g., the variance is not acceptable), the ML entity needs to be re-trained. This is the initial step of the workflow. The ML Training MnS is specified in TS 28.105 [4].

-	ML Testing: Testing of the validated ML entity with testing data to evaluate the performance of the trained ML entity for selection for inference. When the performance of the trained ML entity meets the expectations on both training data and validation data, the ML entity is finally tested to evaluate the performance on testing data. If the testing result meets the expectation, the ML entity may be counted as a candidate for use towards the intended use case or task, otherwise the ML entity may need to be further (re)trained. In some cases, the ML entity may need to be verified which is the special case of testing to check whether it works in the AI/ML inference  function or the target node. In other cases, the verification step may be skipped, for instance in case the input and  output data, data types and formats, have been unchanged from the last ML entity.

Deployment phase:

-	AI/ML Deployment: Deployment of the trained and tested ML entity to the target inference function which will use the subject ML entity for inference.

NOTE: 	The deployment phase may not be needed in some cases, for example when the training function and inference function are in the same entity.

Inference phase:

-	AI/ML Inference: Performing inference using the ML entity by the inference function.

### 4.2.2	AI/ML management capabilities

Each operational step in the workflow (as depicted in clause 4.2.1)  is supported by the specific AI/ML management capabilities, including:

Management capabilities for training phase

-	ML training management: allowing the consumer to request and/or manage the model training/retraining based on the performance evaluation results observed by model performance monitoring (performance data and/or feedback). For example, if the model performance degrades, the ML performance management capability may trigger the ML training to start re-training.

-	ML testing management:  allowing the consumer to request the ML entity testing or , and receive the testing results for a trained ML model.

Management capabilities for deployment phase

-	AI/ML deployment control and monitoring: ensuring availability of the desired ML entity and allowing the consumer to request the deployment of the ML entity to be conducted by the producer or set the policy for deployment of the ML entity to be initiated and conducted by the producer, and monitor the deployment process.

Management capabilities for inference phase

-	AI/ML inference activation/deactivation: allowing the consumer to activate/deactivate the inference using ML entity.

-	AI/ML inference monitoring: allowing the consumer to monitor and evaluate the inference performance of an ML entity.

Editor’s note: more details and/or clarifications to the management tasks can be added later.

# 5	Use cases, potential requirements and possible solutions

## 5.1	ML model performance management

### 5.1.1	Description

During ML model training, test and deployment, the ML model performance evaluation and management is needed. The related performance indicators need to be collected and analyzed. The purpose of ML performance management is to find the problem, figure out what the problem is, and fix it in time to make sure the model can be trained, tested, and deployed healthy.

### 5.1.2	Use cases

#### 5.1.2.1	ML model performance indicators

The ML model performance indicators related to ML model training, test and deployment need to be defined. The indicators mainly include three aspects:

Resource-related indicators:  the performance indicators of the system that the model trains or deploys.

Model-related indicators: performance indicators of the model itself.

Service-related indicators: the running state indicators of the launched model.

These indicators need to be precisely defined. For different service, some indicators can be selected for evaluation. For example, resource-related indicators and model-related indicators may be selected in the training phase, while service-related indicators may be selected in the deployment phase. The AI/ML MnS producer should first determine which indicators are needed and then use these indicators for evaluation.

#### 5.1.2.2	Monitoring and control of ML behavior

In a typical network operation, an operator configures and operates an ML entity according to the manual of the ML entity. Usually, the operator does not need to know the details of the MLentity’s internal-decision making, simply due to “too many” MLentities running for the network and, too much detail that is unnecessary information for the operator. as it is in the vendor's interest to hide the internal aspects of the implementation of their automation solutions.

However, the operator still needs to guide the solutions and to configure them to achieve the desired outcomes in an ML entity-agnostic manner. For example, consider the load balancing automation use case (AutoLB) summarized by Table 1. With an AutoLB ML entity decides that how distribute load among networking objects. The ML entity has specific actions that it can take while the operator also has operational actions that it needs to take to steer the solution, e.g. to switch off the solution, to reconfigure the solution, to change the solutions input. As such, the behavior of the ML entity in terms of the actions it takes under any given conditions needs to be related to the configuration actions from the MnS consumer (e.g. the operator)

Table 5.1.2.2-1: Operability of the Automated load balancing

| Automation use case | Description | ML entity's context | Example ML entity's decisions | Operator's Actions |
| --- | --- | --- | --- | --- |
| load balancing (AutoLB) | Distribute load among different networking objects, e.g. among cells. | - Amount of traffic, no of users, etc | - select CIO values | - set the maximum CIO values, - Deactivate AutoLB |

So, it is important that even without knowing the details of the ML entity, the operator needs to have the understanding of the ML entity’s overall behaviour. And, if a part of the ML entity’s decisions/actions are not what is preferred by the operator, the operator needs to instruct the ML entity to behave accordingly. Relatedly, means are needed to enable the operator to associate his actions to the context and to enable the  operator to guide the AI/ML inference function or solution.

### 5.1.3	Potential requirements

REQ-MODEL_PERF-CON-1		The AI/ML MnS producer should have a capability to define performance indicators of ML model and select some indicators based on the service.

REQ-AI/ML_BEH-1 	The 3GPP management system shall have a capability to inform authorized AI/ML MnS consumer (e.g. the operator) about the behavior of the ML entity, in an ML entity agnostic manner without the need to expose its internal characteristics

REQ-AI/ML_BEH-2 	The shall have capability that enables an authorized AI/ML MnS consumer (e.g. the operator) to configure the behavior of the ML entity, in an ML entity agnostic manner that does need to expose its internal characteristics

### 5.1.4	Possible solutions

TBD

### 5.1.5	Evaluation

TBD.

## 5.2	Event data for ML training

### 5.2.1	Description

In analytics solutions, Performance Measurements (PMs) and Fault Reports (FRs) from various network function are collected and analytics applied on the PMs and FRs to come up with statistical insights and predictions of events from the raw data. For most algorithms, the prediction accuracy depends upon the amount of relevant historical data, motivating the need to store ever more data, which correspondingly increases the storage and processing resource requirements. However, not all recorded data is useful as the derived events, e.g. captured through analytics processes, may have loss of information OR mis-information e.g., with respect to time of the event.

### 5.2.2	Use cases

#### 5.2.2.1	Pre-processed event data for ML training

For AI/ML algorithms, a large amount of data points does not necessarily add value, e.g., if most of it includes biased data which ends up getting discarded during the pre-processing stages. Instead, the AI/ML algorithms need to have information-rich events data that is condensed but with most of it useful for the required training. For example, one could train an interference optimization solution that learns the best way to combat interference by looking at  counters of handover failures correlated with signal quality. However, for most of the time in the radio network, there will be no interference events, but this cannot be determined if the events are not captured form the data. As such all the data must be kept and used for training. However, the data could also be mined for the interference event or signatures thereof. Then an equivalent interference management solution could be trained using the interference signatures or the signatures combined with only a small amount of raw data.

It is as such necessary to provide means to isolate and store the information rich events in the network, to ensure that minimizing storage and processing costs by discarding the unnecessary raw data does not compromise the ability to and still avails adequate historical information to adequately train ML entities. In other words it is necessary for network functions to their management system to generate data on about the observed network events, e.g., based on the criteria set by the Operator, which events can then be stored to be used later to train ML entities.

Network Function

ML trainig function as Network events Data Consumer

Events processing

Events data

Management Function

Events processing

Network/system data

Network/system events data

Fig 2.2.1-1. Exposing and storing network events data

### 5.2.3	Potential requirements

REQ-EVENT-DATA-1	The 3GPP management system shall enable an authorized consumer to request from the network data producer for network events corresponding to the data produced by that network data producer.

REQ-EVENT-DATA-2 The 3GPP management system shall enable A network data producer to generate network events in place of or alongside the network data that they produce

REQ-EVENT-DATA-3 The 3GPP management system shall enable a  network events aggregator to take the events from different network entities and re-expose them in an aggregated way that eliminates duplications

### 5.2.4	Possible solutions

TBD

### 5.2.5	Evaluation

TBD.

## 5.3	ML entity validation

### 5.3.1	Description

During the ML training process, the generated ML entity (see TS 28.105 [4]) needs to be validated. The purpose of ML validation is to evaluate the performance of the ML entity when performing on the validation data, and to identify the variance of the performance on the training data and the validation data. If the variance is not acceptable, the entity would need to be tuned (re-trained) before being made available to the consumer.

The training data and validation are normally split from the same data set with a certain ratio in terms of the quantity of the data examples, therefore they have the same pattern.

### 5.3.2	Use cases

#### 5.3.2.1	ML entity validation performance reporting

In the ML training, the ML entity is generated based on the learning from the training data, and validated using validation data. The performance of the ML entity has tight dependency on the data (i.e., training data) from which the ML entity is generated. Therefore, an ML entity performing well on the training data may not perform well on other data. If the performance of ML entity is not good enough as result of ML validation, the ML entity will be tuned (re-trained) and validated again. The process of ML entity generation and validation is repeated by the ML training function, until the performance of the ML entity meets the expectation on both training data and validation data. The producer in the end selects one or more ML entities with the best level performance on both training data and validation data as the result of the ML training, and reports to the consumer. The performance of each selected ML entity on both training data and validation data also need to be reported.

The performance result of the validation may also be impacted by the ratio of the training data and validation data. Consumer needs to be aware of the ratio of training data and validation data, besides the performance score on each data set, in order to be confident about the performance of ML entity.

### 5.3.3	Potential requirements

REQ-MODEL_VLD-CON-1	The MLT MnS producer should have a capability to validate the ML entities during the training process, and report the performance of the ML entities on both the training data and validation data to the authorized consumer.

REQ-MODEL_VLD-CON-2	The MLT MnS producer should have a capability to report the ratio (in terms of the quantity of the data examples) of the training data and validation data used for training of an ML entity during the training process.

### 5.3.4	Possible solutions

#### 5.3.4.1	Validation performance reporting by enhancing the existing IOC

In TS 28.105 [4], the ML entity training report is provided by MLTrainingReport IOC, which includes the attribute indicating the performance of the ML entity when performing on the training data.

To support the ML entity validation performance reporting, a new optional attribute can be defined in the MLTrainingReport IOC to indicate the performance of the ML entity when performing on the validation data.

### 5.3.5	Evaluation

The possible solution described in clause 5.3.4.1 enhances the existing MLTrainingReport IOC with a new optional attribute indicating the performance of the ML entity when performing on the validation data, the change is lightweight and it is backward compatible, therefore it is a feasible solution.

## 5.4	ML entity testing

### 5.4.1	Description

After an ML Entity is trained, validation is done to ensure the training process is completed successfully.. However, even when validation is conducted successfully during development, it is necessary to test and check if the ML entity is working correctly under certain runtime contexts or using certain testing data set. Testing may involve interaction with third parties (besides the developer of the ML training function) , e.g. the operators may use the ML training function or third-party systems/functions that may rely on the results computed by the ML entity for testing.

After completing the ML entity training, and when the performance of the trained ML entity meets the expectations on both training and validation data, the ML entity is made available to the consumer(s) via the ML training report (see MLTrainingReport IOC in TS 28.105 [4]). Before applying the ML entity to the target inference function or AI/MLinference function, the ML training MnS producer may need to allow the consumer to evaluate the performance of the ML entity via the ML testing process using the consumer’s provided testing data. The testing data have the same pattern as the input part of the training data.

For these reasons, provision of ML entity testing and its control need to be standardized to enable the multi-vendor interaction among the different systems. If the testing performance is not acceptable or does not meet the pre-defined requirements, the consumer may request the ML training producer to re-train the ML entity with specific training data and/or performance requirements.

### 5.4.2	Use cases

#### 5.4.2.1	Consumer-requested ML entity testing

After receiving an ML training report about a trained ML entity from the ML training MnS producer, the consumer may request the testing MnS producer to test the ML entity before applying it to the target inference function. In the ML testing request, the consumer provides the testing data which have the same pattern as the input part of the training data.

Any ML entity needs to be tested with specific inputs and features that are applicable to the use case and the applicable deployment environment.

The ML testing MnS producer performs the ML testing using the consumer’s provided testing data. The ML testing is to conduct inference on the tested ML entity using the testing data as the inference inputs and produce the inference output for each testing dataset example.

The AML testing MnS producer may be the same as or different from the ML training MnS producer.

After completing the ML testing, the ML testing MnS producer provides the testing report indicating the success or failure of the ML testing to the consumer. For a successful ML testing, the testing report contains the testing results, i.e., the inference output for each testing dataset example.

The ML testing MnS producer needs to have the capabilities and provide the services needed to enable the consumer to request testing and receive results on the testing of a specific ML entity or of an application or function that contains an ML entity.

To achieve the desired outcomes, any ML entity needs to be tested with the appropriate testing data, which can reflect the current status of the network where the ML entity is expected to be deployed. Correspondingly, the ML testing MnS producer needs to support the required management services to test the ML entities.

#### 5.4.2.2	Control of ML entity testing

Given a testing capability as provided by a given ML testing MnS producer, a consumer (e.g., an operator) may wish to control and manage that testing process capability. For example, the operator may wish to define policies on how frequent testing for a given ML entity may be executed. Correspondingly, the 3GPP management system needs to provide the capability to allow the ML entity testing to be configured.

Figure 5.4.2.2-1: ML entity testing and control

### 5.4.3	Potential requirements

REQ-AI/MLTEST-1 The ML testing MnS producer should have a capability for an authorized consumer to request the testing of a specific ML entity.

REQ-AI/MLTEST-2 The ML testing MnS producer should have a capability to create a testing process instance per the testing request for an authorized consumer.

REQ-AI/MLTEST-3 The ML testing MnS producer should have a capability to report to an authorized consumer the results of a specific instance of ML testing process with the result of a successful ML entity testing containing the inference output for each testing data example.

REQ-AI/MLTEST-4 The ML testing MnS producer should have a capability for an authorized consumer (e.g. the operator) to configure or modify an instance of ML testing process.

REQ-AI/MLTEST-5 The ML testing MnS producer should have a capability to test a specific ML entity using specific data specified by the consumer or using data at a location address specified by the consumer or using data with specific characteristics defined by the consumer.

REQ-AI/MLTEST-6 The ML testing MnS producer should have a capability to test a specific ML entity for a specified expected runtime context as may be stated by the consumer.

REQ-AI/MLTEST-8 The ML testing MnS producer should support a capability for an authorized consumer to define the reporting characteristics related to a specific instance of ML testing request.

REQ-AI/MLTEST-9 The ML testing MnS producer should support a capability for an authorized consumer to manage the AI/ML testing request, including suspending, resuming, canceling the request, or adjusting the desired runtime context of the testing.

### 5.4.4	Possible solutions

#### 5.4.4.1	NRM based solution

This solution uses the instances of following IOCs for interaction between ML testing MnS producer and consumer to support the ML entity testing:1) 	The IOC representing the ML entity testing request, for example named as MLTestingRequest.

This IOC is created by the ML entity testing MnS consumer on the producer, and it contains the following attributes:

- 	identifier of the ML entity to be tested;

-	testing environment requirements, e.g., expected runtime context;

-	testing data.

2) 	The IOC representing the ML entity testing policy, for example named as MLTestingPolicy.

This IOC is created by the ML entity testing MnS consumer on the producer to control the testing initiated by the producer, and it contains the following attributes:

- 	identifier or inference type of the ML entity to be tested;

-	testing environment requirements, e.g., expected runtime context;

-	testing triggers, i.e., the conditions that would trigger the testing of an ML entity.

3) 	The IOC representing the ML entity testing process, for example named as MLTestingProcess. This MOI is created for the ML entity testing process corresponding to the testing requested by the consumer per the IOC described in 1), or the testing initiated by the producer based on the given testing policy per the IOC described in 2).

This IOC is created by the ML entity testing MnS producer and reported to the consumer, and it contains the following attributes:

- 	identifier of the ML entity being tested;

-	the associated ML entity testing request;

-	the associated ML entity testing policy;

-	testing progress;

-	testing environment, e.g., the testing runtime context;

-	testing data to be used;

-	control of the process, like cancel, suspend and resume.

4) 	The IOC representing the ML testing report, for example named as MLTestingReport.

This IOC is created by the ML testing MnS producer and reported to the consumer, and it contains the following attributes:

- 	identifier of the tested ML entity;

-	the associated ML entity testing request;

-	the associated ML entity testing process;

-	testing result indicating the success or failure, and containing the inference output for each testing data example for successful case, and the failure reason for the failed case.

The examples of IOCs and their relations between the IOCs are depicted in the figure below.

Figure 5.4.4.1-1: Example of ML entity testing related NRMs

NOTE: The name of the IOCs and attributes are to be decided in normative phase.

### 5.4.5	Evaluation

The solution described in clause 5.4.4.1 adopts the NRM-based approach, which reuses the existing provisioning MnS operations and notifications. This solution is also consistent with the approach used by ML training MnS defined in TS 28.105 [4]. It does not only reuse the existing capabilities (provisioning MnS operations and notifications) to a greater extent, but also provides the flexibility to facilitate both co-located and distributed implementation and deployment of ML training MnS and ML testing MnS by using the consistent NRM-based approach.

Therefore, the solution described in clause 5.4.4.1 is a feasible solution.

## 5.5	ML deployment

### 5.5.1	Description

ML deployment refers to the process of making an AI/ML inference function available in the operational environments, where it could start adding value by conducting inference (e.g., prediction). After trained ML entity meets the performance criteria, the ML entity could be deployed in 3GPP system via a software installation or via a configuration management procedure and subsequently activated.

After an ML entity is deployed and activated in the production environment, the data fed to the ML entity may change to the level where it is different from the data used in the initial prior training of the respective ML entity. To improve model performance with the changing data, the ML entity therein may need to be retrained and redeployed within the production environment.

### 5.5.2	Use cases

#### 5.5.2.1	ML deployment information retrieved by consumer

After the ML entity is trained by the ML training (MLT) MnS producer during development, the MLentity needs to be deployed to conduct inference in the operational environment.

Once the AI/ML-enabled function has been installed in the operational environment, as different consumers need to know the available information of AI/ML-enabled functions and to determine the next appropriate action, the MnS consumer may request the MnS producer to retrieve the deployment information of ML-entity.

The general deployment information used to describe an ML entity may include:

-	Resource information, which describes the static parameters of ML entity (e.g. mLEntityVersion, mLEntityId, trainingContext, see TS 28.105 [4])

-	Management information, which describes the information model that is used for ML entity lifecycle management (e.g. activation flag, status, create time, last update time).

-	Capability information, which describes the capability information (e.g. inference type, performance metrics).

### 5.5.3	Potential requirements

REQ-MODEL_DPL-CON-1 The MnS producer responsible for AI/ML management shall have a capability to retrieve deployment information of the ML entity and inform an authorized consumer about the information.

REQ-MODEL_DPL-CON-2 The MnS producer responsible for AI/ML management shall have a capability to allow an authorized consumer to retrive deployment information of the MLenity.

### 5.5.4	Possible solutions

TBD

### 5.5.5	Evaluation

TBD

## 5.6	AI/ML Inference History

### 5.6.1	Description

For different automation requirements, network and management automation functions (e.g. gNB, MDAS, SON) may apply Machine Learning functionality to make the appropriate inferences in different contexts. Depending on the contexts, the AI/MLEntity may take different decisions at inference with different outcomes. The history of such inference decisions and the context within which they are taken may be of interest to different consumers.

### 5.6.2	Use cases

#### 5.6.2.1	Tracking AI/ML inference decision and context

The deployed ML entity may take different decisions at inference in different contexts and with different outcomes. The selected decisions may need to be tracked for future reference, e.g. to evaluate the appropriateness/ effectiveness of the decisions for those contexts or to evaluate degradations in the ML entity's decision-making capability. For this, the network not only needs to have the required inference capabilities but needs also to have the means to track and enable usage of the history of the inferences made by the ML entity.

Figure 5.6.2.1-1: Example use and control of MLInferenceHistory Request and reporting in the management plane

### 5.6.3	Potential requirements

REQ-MLHIST-1 The producer of ML inference history should have a capability allowing an authorized consumer to request the inference history of a specific ML entity.

REQ-MLHIST-3 The producer of ML inference history should support a capability for an authorized consumer (e.g. the function/entity that generated the Request for ML inference history) to define the reporting characteristics related to a specific instance of ML inference history or the reporting thereof.

### 5.6.4	Possible solutions

-	Introduce ML Inference History (named e.g. MLInferenceHistory) as an IOC, which may be modelled as a managed function that is contained in a ManagedFunction, ManagementFunction or subnetwork. The MLInferenceHistory then may contain or be associated with the critical properties and modules needed to accomplish ML Testing, including:

-	the list of MLInferenceHistoryRequests,

-	the list of MLEntities either under Testing or to be considered for Testing,

-	MLInferenceHistoryReporting to report on MLInferenceHistoryRequests or their related outcomes.

-	Introduce ML Inference History Request (named e.g. MLInferenceHistoryRequest) as an IOC, which may be instantiated by the consumer (e.g. an operator, a managed functions or a management function) for any required history. Each MLInferenceHistoryRequest

-	May be associated to exactly one deployed MLEntity

-	may contain specific reporting requirements, e.g. one attribute ReportingPeriod may define how the MLInferenceHistory may report about the MLInferenceHistoryRequest,

-	may have a source to identify where its coming from. The sources may for example be an enumeration defined for network functions, operator roles, or other functional differentiations.

-	The MLInferenceHistoryRequest may prescribe a list of Reporting-Context, which describes the list of constraints or conditions that may evaluate to true when the Reporting is executed. The Reporting-Context may be a triple <Attribute, Condition, ValueRange >  where:

-	Attribute: describes a specific attribute of or related to the object or the usecase that relates to the MLEntity on which reporting is executed. It may also refer to the characteristics of such object (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc.). It may also refer to an attribute related to the operating conditions of the object or use case (such as weather conditions, load conditions, etc.).

-	Condition: expresses the limits within which the Attribute is allowed/supposed to be. The allowed values for the condition may include: "is equal to"; "is less than"; "is greater than"; "is within the range"; "is outside the range".

-	ValueRange: describes the range of values that are applicable to the Attribute as constrained by the Condition.

-	Introduce a ML Inference History Reporting (named e.g. MLInferenceHistoryReporting) as an IOC, which may be use to model the capability of compiling and delivering reports and notifications about MLInferenceHistory or its associated MLInferenceHistoryRequests. The MLInferenceHistory may generate one or more ML Inference History Reports via one or more instances of MLInferenceHistoryReporting.

-	Each ML Inference History Report may be associated to one or more MLEntities for which InferenceHistory is requested and/or reported

Editor’s note: there may be potential alignment with the solution for historical data handling from  Rel-18 Study on management data collection study.

### 5.6.5	Evaluation

TBD.

## 5.7	ML context

### 5.7.1	Description

MLContext attribute represents the status and conditions related to the ML entity (cf. TS 28.105[4]). This may include the network context as defined in TS 28.104 [2] as well as other conditions that may be applicable to the ML entity but are not part of network characteristics e.g. the time of day, season of the year. As part of ML model performance management there is identification of the problem that the ML model is facing. As described in TS 28.104[2], the differences in the network context, i.e., network status, under which data is collected to produce analytics, significantly affect the produced analytics. Similarly, the changes in the AI/ML context, e.g., the characteristics of the data related to the network status and conditions used for ML model training, testing and deployment may affect the ML entity performance, thus may represent a problem for the ML entity. Thus management capabilities are needed to enable awareness of the AI/ML context in terms of the identification as well as monitoring and reporting of changes in AI/ML context as part of the identification of the problem that the ML entity is facing.

### 5.7.2	Use cases

#### 5.7.2.1	ML context monitoring and reporting

AI/ML context related to ML model training, testing and deployment needs to be identified by characterizing the input data used by the ML model is targeted to work. As an example, such characterization may be done based on the statistical properties of data. Monitoring of such AI/ML context serves to detect the changes and anomalies in the AI/ML context. Some anomalies may be considered as a problem that ML entity is facing as it may lead to its performance degradation. Therefore, the consumer of the related AI/ML service needs to be informed about such observed AI/ML context change.

#### 5.7.2.2	Mobility of ML Context

In several network automation use cases, the respective AI/ML inference function cannot cover the complete network in one ML entity instance. An ML entity may be trained for a specific local context, and similarly, a different context may be applicable for inference, so the ML entity may be characterized by different trainingContext and an expecetdInferenceContext. However, the network scopes where the data used for training and inference is collected does not always necessarily overlap with the network scopes in which the function makes decisions. The context of ML entities or AI/ML inference function may need to distinguish between context for generating decisions or insights, the context from which it generates measurements or data as well as the context in which it is prepared before being active for inference. So the characteristics of the respective AI/ML inference function need to be distinguished depending on the different contexts of the AI/ML inferencefunction. As such besides the validity scope defined by the trainingContext and an expecetdInferenceContext, the ML entity should also be characterized by specific measurement scopes, where the input measurements are collected. And these may also be separately defined for the 2 use cases.

#### 5.7.2.3	Standby mode for AI/ML inference functionality

Where the respective AI/ML inference function cannot cover the complete network in one ML entity instance, multiple instances of ML entities may be required, one for each specific network scope, such as a cell. When a network automation use case requires several ML entities instances, where each has its own limited validity scope (a geographical area or a subnetwork), transfers of machine learning context, i.e. “handovers” between the ML entities covering different validity scopes (not necessarily identical to cell coverage area), are needed. Accordingly, the AI/MLinferencefunction or the ML entities therein may have different roles, either as active or standby decision makers.

Consider the use case where a different ML entity instance is needed for each Base station, i.e. the validity scope defined by the expecetdInferenceContext is a specific gNB. An instance of this is predictive ML-driven handover where an MLEntity  is trained to decide the optimal handover point and target cell based on the UE measurements. When the UE hands over to a cell in another gNB, a new ML entity  instance fitting the new validity scope needs to be deployed in the UE. This is illustrated by Figure 5.N.1.a) where the UE uses ML entity instance 1 in both cells 1 & 2 but when the UE hands over to cell 3, which is outside the validity area of ML entity instance 2 needs to be deployed to the UE.

However, the deployment may require uploading the required ML entity instance into the UE and initializing the ML entity, for example to collect and feed the necessary input data to setup the required internal states, such as in Long-Short Term Memory (LSTM) Recurrent Neural Networks (RNNs). Accordingly, it may take significant time before the new ML entity becomes active and operational. Moreover, if the UE hands over back to cell 2 after a short stay in cell 3 (pingpong), the UE needs to immediately re-deploy ML entity instance 1, compounding the problem further.

To minimize this risk, there should be a "prepared scope" defined for each ML entity, which is the scope within which the ML entity  is deployed and initialized but not activated for inference.

- b)

Figure 5.7.2.3-1: Example mobility of AI/ML context - a) validity scopes, b) validity and standby scopes

This is illustrated by Figure 5.N.1 8b) where besides the validity areas, standby areas are defined for each ML entity  instance, e.g. ML entity  1 is active in cells 1 and 2 but standby in cell 3. This implies that the ML entity  1 should be availed to the UE in cell 3 even if the UE cannot use ML entity  instance 1 in cell 3. To support this, it must be possible to configure both, the validity areas and the standby areas for ML entities and to define their role in them, i.e., either active, or prepared.

### 5.7.3	Potential requirements

REQ-ML_CTX -1 The producer AI/ML-related MnS including for training and inference should have a capability to identify and monitor the AI/ML context, as well as to inform the consumer about observed changes in AI/ML context ,

REQ-ML_CTX-2 The 3GPP Management system should have a capability for an authorized consumer to configure or read the measurement scope of an ML entity for AI/ML training or for AI/ML inference.

REQ-ML_CTX-3 The 3GPP Management system should have a capability for an authorized consumer to read the ML entity's inference cache scope that defines the network scope within which the ML entity is activated to be in standby mode in preparation for elevating to active mode.

### 5.7.4	Possible solutions

#### 5.7.4.1.	MLContext datatype on MLEntity

The IOC MLContext is a <<datatype >> attribute on the MLEntity. The  MLContext is notifiable, so that any interestd party can subscribe to a notification on the MLContext.

when there is a change in the MLContext, e.g. as observed from the statistical properties of data, the notification is sent to the entity that subscribed to the notification.

The MLContext has the following attributes which can be configured by the consumer when defining an MLContext to be moitored.

-	Attribute “area of interest” identifying the geographical area to be taken into account.

-	Attribute "area granularity" defining the size of the sub-areas of the area of interest for which the statistical properties of data should be identified. It can be expressed for example in km2 or as a description of a relevant part of the network (e.g., building, street, block, district, city, or state). In case area granularity attribute is not specified by the consumer, contexts related to different areas are determined according to the data distribution detected in the area of interest.

-	Attribute " reporting_threshold " indicating when the deviation in data statistics compared to previously determined context shall be reported. It can be numeric attribute, e.g., indicating the percentage of changes between the currently monitored data statistics and previously identified data statistics.

The notification delivers the MLContextReport that contains the information on partitioning of area of interest into smaller areas (i.e. sub-areas) based on statistical properties of data. The report may also comprise the statistical properties of identified sub-areas. Furthermore, the report may include the information on detected changes in data statistics. Hereby, either the complete information on current data statistics or the actual “delta” compared to previous data statistics may be indicated to the consumer.

The MLContextReport MOI is contained by the MLTrainingFunction or MLInferenceFunction MOI.

### 5.7.5	Evaluation

TBD.

## 5.8	ML Entity Capability Discovery and Mapping

### 5.8.1	Description

A network or management function that applies AI/ML to accomplish specific tasks may be considered to have one or more ML entities each having specific  capabilities. The capabilities are either of:

-	a decision-making capability which is in the form of triple <x,y,z> indicating

-	x: the object or object types for which the ML entity can undertake optimization or control

-	y: the configurable attributes on object or object types x, which the ML entity optimizes or controls to achieve the desired outcomes

-	z: the performance metrics which the ML entity optimizes through its actions

-	an analysis capability which is in the form of tuple <x,z> indicating

-	x: the object or object types for which the ML entity can undertake analysis

-	z: the network context (on object x) for which the ML entity produces analysis

Different network functions may need to rely on existing AI/ML capabilities to accomplish the desired automation. However, the applicability of the ML-based solutions and the details of such ML-based solutions (i.e., which ML entities shall be applied and how) for accomplishing those automation functionalities is not obvious. On a high-level, such ML-based solutions may be categorized into cases with or those without ML orchestration. In both cases, management services are required to identify the capabilities of the involved ML entities and to map those capabilities to the desired logic.

### 5.8.2	Use cases

#### 5.8.2.1	Identifying capabilities of ML entities

Network functions, especially network automation functions, may need to rely on AI/ML capabilities that are not internal to those Network functions to accomplish the desired automation. For example, as stated in TS 28.104, “an MDA Function may optionally be deployed as one or more AI/ML inference function(s) in which the relevant models are used for inference per the corresponding MDA capability.” Similarly, owing to the differences in the kinds and complexity of intents that need to be fulfilled, an intent fulfilment solution may need to employ the capabilities of existing AI/ML to fulfil the intents. In any such case, management services are required to identify the capabilities of those existing ML entities.

Figure 5.8.2.1-1: Request and reporting on AI/ML capabilities

Figure 5.8.2.1-1 shows that the consumer may wish to obtain AI/ML capabilities to determine how to use them for the consumer's needs, e.g., for its intent targets or other automation targets.

#### 5.8.2.2	Mapping of the capabilities of ML entities

Besides the discovery of the capabilities of ML entities, services are needed for mapping the ML entities and capabilities. In other words, instead of the consumer discovering specific capabilities, the consumer may want to know the ML entities than can be used to achieve a certain outcome. For this, the producer should be able to inform the consumer of the set of ML entities that together achieve the consumer's automation needs.

In the case of intents for example, the complexity of the stated intents may significantly vary - from simple intents which may be fulfilled with a call to a single ML entity to complex intents that may require an intricate orchestration of multiple ML entities. For simple intents, it may be easy to map the execution logic to the one or multiple ML entities. For complex intents, it may be required to employ multiple ML entities along with a corresponding functionality that manages their inter-related execution . The usage of the the ML entities  requires the awareness of the capabilities of their capabilities and interrelations.

Moreover, given the complexity of the required mapping to the multiple ML entities, services should be supported to provide the mapping of ML entities and capabilities.

Figure 5.8.2.2-1: Mapping execution logic to AI/ML Capabilities

NOTE: 	Figure 5.8.2.2-1 shows that the consumer may wish to obtain the mapping of AI/ML capabilities to some management tasks to determine how to use them for the consumer's needs, e.g., for its intent targets or other automation targets. The management tasks may for example include specific metrics to be optimized, but the candidate tasks to be considered shall be agreed at the normative phase.

### 5.8.3	Potential requirements

REQ-ML_CAP-1 The 3GPP Management system should have a capability for an authorized MnS consumer to request the AI/ML MnS Producer for the capabilities of existing ML entities available within the provider of AI/ML inference.

REQ-ML_CAP-2 The AI/ML MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML entity as a decision described as a triplet <object(s), parameters, metrics> with the entries respectively indicating : the object or object types for which the ML entity can undertake optimization or control; the configuration parameters on the stated object or object types, which the ML entity optimizes or controls to achieve the desired outcomes; and the network metrics which the ML entity optimizes through its actions.

REQ-ML_CAP-3 The AI/ML MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML entity as an analysis described as a tuple <object(s), characteristics> with the entries respectively indicating: the object or object types for which the ML entity can undertake analysis; and the network characteristics (related to the stated object or object types) for which the ML entity produces analysis.

REQ-ML_CAP-4 The 3GPP Management system should have a capability to enable an authorized MnS consumer to request an AI/ML MnS Producer for a mapping of the consumer's targets to the capabilities of one or more ML entities.

### 5.8.4	Possible solutions

The network functions may rely on available AI/ML capabilities to achieve the desired outcome. Such available AI/ML capabilities may need to be discovered as a first step.

The following solution (related to the worklow depicted in Figure 1) may be applicable:

-	When the AI/ML MnS Consumer requests for information on available ML entities and their supported AI/ML capabilities from the AI/ML MnS Producer (e.g., inference producer), the AI/ML MnS Producer provides the AIML_capability in the following form:

-	a decision-making capability in the form of triple <x,y,z> indicating

x: the object or object types for which the ML entity can undertake optimization or control

y: the configurable attributes on object or object types x, which the ML entity optimizes or controls to achieve the desired outcomes

z: the performance metrics which the ML entity optimizes through its actions

-	an analysis capability in the form of tuple <x,z> indicating

x: the object or object types for which the ML entity can undertake analysis

z: the network context (on object x) for which the MLEntity produces analysis

-	Introduce the <<datatype>> attribute representing the AI/ML capability e.g. named as  AIML_capability as The attribute is aproperty of any AI/ML MnS Producer or any function that has or contains ML entity e.g. for any AI/ML inference function, ML testing function or ML training function. The AIML_capability may also be added to the ML entity, and

-	The attribute for the AI/ML capability will as such have three attributes

-	An attribute for the managed object: This is conditionally mandatory as either the object or the object type should be stated. It is mandatory if the managed object type is not included.

-	An attribute for the managed object type: This is also conditionally mandatory as either the object or the object type should be stated. It is mandatory if the managed object is not included.

-	An attribute for the configurable attributes on the managed object or managed object types:  This is optionalas it only applies for decisions and not for analysis type capabilities.

-	An attribute for the metrics: This which includes either the performance for the decision or the analyses metrics or context should be mandatory.

Figure 5.8.4-1: AI/ML capability request and report

### 5.8.5	Evaluation

The solution described in clause 5.8.4 adopts the NRM-based approach, which reuses the existing provisioning MnS operations and notifications. Moreover, the solution enables reuse of the Discovery MnS to discover both the AI/ML functionality and their capabilities. Introducing the AI/ML_capability <<datatype>> enables a working solution with or without the discovery MnS. Via the discovery MnS this AI/ML_capability will be the returned outcome and without the discovery MnS, the consumer can instead read this AI/ML_capability attribute off the MOI.

Therefore, the solution described in clause 5.8.4 is a feasible solution.

## 5.9	AI/ML update management

### 5.9.1	Description

Due to the complexity and time-varying nature of network, the AI/ML entities previously deployed may no longer be applicable to the current network after running for a period of time. Typically, the performance of a trained model may degrade over time. The ML entity needs to be updated timely to ensure the performance of inference and analysis.

### 5.9.2	Use cases

#### 5.9.2.2	AI/ML entities updating initiated by producer

The ML entity updating may be initiated by the AI/ML MnS producer. In order to keep the model at a requested level, the AI/ML MnS producer may periodically conduct ML entity retraining with new available training data. Once a new version ML entity is obtained after the training is finished, it can be used to update the current ML entity with this new version. In another condition, the AI/ML MnS producer may initiate ML entity updating based on the running model performance. For example, if the performance of the running ML model is decreased under a predefined threshold, the AI/ML MnS producer may decide to start ratraining and then update the ML entity to a new version which performs better.

### 5.9.3	Potential requirements

REQ-AIML_UPD-CON-1: The AI/ML MnS producer should have a capability to update the ML entities and inform an authorized consumer about the update status.

### 5.9.4	Possible solutions

#### 5.9.4.1	Possible solution#1

Following is the proposed solution based on information model defined in TS 28.105 [2].

-	Extend the MLEntity <<dataType>> with an attribute "updatedTime", which indicates the time that the ML entity is updated. For a trained MLEntity, the value of "mLEntityVersion" indicates the version number of the ML entity. When the ML entities updating is initiated and successfully excuted, the value of "mLEntityVersion" is modified to be the new version number, and the value of "updatedTime" is the time that the updating is finished. Then AI/ML MnS producer can use the "notifyMOIAttributeValueChange" operation to inform the authorized MnS consumer about the AIML update Status including the updated mLEntity version number and the corresponding updating time.

### 5.9.5	Evaluation

TBD.

## 5.10	AI/ML configuration management for RAN domain ES

### 5.10.1	Description

At the beginning of the AI/ML inference function operation, enabling the AI/ML entities should be controlled by AI/ML management. As described in clause 4.7 in TR 28.813[3], RAN domain ES can use AI to formulate energy saving solutions. Therefore, the AI/ML entities which enabled RAN domain ES function should be controlled by AI/ML management. The ML entity configuration need to be triggered to enable RAN domain ES function. The ML entity configuration can be initiated by the AI/ML MnS consumer or initiated by the AI/ML MnS producer.

### 5.10.2	Use cases

#### 5.10.2.1	ML entity configuration initiated by consumer

The ML entity configuration may be initiated by the AI/ML MnS consumer of Cross domain management. AI/ML MnS Consumer monitor network performance and determine whether to trigger the ML entity configuration. For example, for ES purpose, AI/ML MnS Consumer collects the information of the capacity booster cells and coverage cells inside the RAN domain area, then makes the decision for activation ML entity, e.g. for enabled RAN domain ES function, consumer can configure the training/retraining activation at training phase, policy, target and activation for inference phase etc. In this case, the consumer can trigger the ML entities so as to get better network performance.

Figure 5.10.2.1-1: ML entity configuration initiated by consumer

#### 5.10.2.2	ML entity configuration initiated by producer

The ML entity configuration may be initiated by the AI/ML MnS producer. AI/ML MnS producer can determines whether to trigger ML entity configuration based on network performan and service requirements. For example, after receiving an RAN domain ES requirement, the AI/ML MnS producer can decide to trigger training of the ML entity, e.g. RAN domain ES function. In this case, the AI/ML MnS producer responsible for AI/ML management shall have a capability to trigger the ML entities and inform an authorized consumer about the ML entity status.

Figure 5.10.2.2-1: ML entity configuration initiated by producer

#### 5.10.2.3	Enabling partial activation of AI/ML capabilities

TR 28.908 describes that a network or management function that applies AI/ML to accomplish specific tasks may be considered to have one or more AIML capabilities as either of a decision-making capability or an analysis capability. For a given AI/ML inference  function, first, it is very difficult to “predict” the benefits and to quantify such benefits of using AI/ML capability in a given context of operational network, before using it. Secondly, testing the AI/ML capabilities using test data does not give a “full picture” on how the ML model will impact the network once it is activated in operational environment. For example, the testing  may provide the insights on the accuracy of the ML entity that can be expected once the ML entity is deployed and activated.

As such it is necessary to ensure that AI/ML capabilities that are being activated in operational network will bring the benefits and will not further deepen existing network performance problems. Moreover, it is important to provide means to check which particular AI/ML capabilities are beneficial to be activated in a given context of operational network. Correspondingly, the AI/ML inference MnS producer may provide different steps through which the capabilities of its constituent ML entities may be activated. These abstraction of the scope of the ML entities may be called Abstract activation steps.  For example, for such Abstract activation steps, the AI/ML inference MnS producer may support that only a subspace of its scope is activated e.g. to only a part of the geography of its cells and not the whole city or only a certain period (say between 18:00 and 6:00) and not the entire time.  Together, these imply that it is important to ensure that the AI/ML MnS consumer has a finer control on AI/ML capabilities activation and de-activation.

### 5.10.3	Potential requirements

REQ-AIML_Config-CON-1: The AI/ML MnS producer should have a capability to allow the authorized consumer to trigger AI/ML training capability and inference capability configuration.

REQ-AIML_Config-CON-2: The AI/ML MnS producer responsible for AI/ML management shall have a capability to configure the AI/ML training capability and inference capability and inform an authorized consumer about the AI/ML configuration status.

REQ-AI/ML_ACT-1 the AI/ML inference MnS producer shall have a capability to allow an authorized MnS consumer to partially or gradually activate the AI/ML capabilities of a producer of AI/ML inference through set of abstract activation steps.

REQ-AI/ML_ACT-2 the AI/ML inference MnS producer shall have a capability to allow an authorized MnS consumer to activate the AI/ML capabilities of an AI/ML inference producer for a specified sub-scope of the applicable expectedruntimecontext of the AI/ML inference producer.

REQ-AI/ML_ACT-3 the AI/ML inference MnS producer shall have a capability to allow an authorized MnS consumer to partially or gradually deactivate the AI/ML capabilities of a producer of AI/ML inference through set of abstract activation steps.

REQ-AI/ML_ACT-4 the AI/ML inference MnS producer shall have a capability to inform an authorized MnS consumer of the sub-scope for which the new AI/ML capabilities have been activated.

### 5.10.4	Possible solutions

TBD

### 5.10.5	Evaluation

TBD

## 5.11	Orchestrating AI/ML Inference

### 5.11.1	Description

A network automation system may involve or apply multiple AI/ML inference functions or ML entities each of which only has a limited view of the network scope. For their effective operation, it may be necessary to apply orchestration mechanisms (be it centralized or otherwise) to orchestrate both the execution of the AI/ML inference functions as well as the execution of the actions recommended by the AI/ML inference  functions.

### 5.11.2	Use cases

#### 5.11.2.1	Knowledge sharing on executed actions

The actions and effects of AI/ML inference functions cannot be known apriori since they are based on the learnings of the AI/ML enabled function. An AI/ML enabled functions may learn to optimize one set of paratemers but its actions may impact another function. In that case mechanisms are needed to counteractor minimize th negative impacts of actions of one AI/ML enabled function on any other network function.

When an ML entity A executes an action on the network, that action may affect other network functions. Most critical is that those actions may affect the learning environment (i.e. the training data) of another ML entity, say ML entity B. Correspondingly, the ML entity B needs to be informed when such actions are taken by any ML entity A.

#### 5.11.2.2	Knowledge sharing on impacts of executed actions

AI/ML inference functions are able to adjust to adjust their behavior depending on context and on all the information they receive. When an ML entity in function A executes an action on the network that affect other network functions, the ML entity in function A may be able to adjust its behavior to minimize its impact on the other network functions if such an ML entity is informed of its impact on the other network functions. To account for such impacts, the network functions that are affected or the 3GPP management system, needs to inform the ML entity in function A of the observed impacts of the action of the ML entity in function A on the other network functions.

In otherwards, it is necessary that when an action is taken by ML entity in function A, after an appropriate interval (specific to either A or B as may be needed), the network function B (and the other network functions that notice impacts on their metrics or input data) should report their metrics to A. Correspondingly, ML entity in function A may aggregate the reported observations with its own metrics to evaluate the global effect of its actions. In doing so, ML entity in function A is able to learn the best actions that concurrently optimize it’s (A’s) objective(s) and also minimize the effects on the peers.

The report from B to all may contain values on known KPIs and metrics, e.g. those standardized in TS28.552 and TS28.554.

Figure 3 Distributed coordination of Cognitive Functions

#### 5.11.2.3	Abstract information on impacts of executed actions

In a multivendor environment, the KPIs semantics differ and KPIs that measure one event may be named and computed differently by two vendors.  e.g. the Handover rate (H) could be Handovers per user per unit time or Handovers per cell per unit time. Consequently, there is no guarantee that the exchanged KPI or metric values as described in clause 5.C.2.2. will be interpretable by the ML entity A when it receives that metric.

Instead, it is better when the ML entity B expresses its level of dissatisfaction or impact of the action that was taken by ML entity A. The level of dissatisfaction or impact may be expressed in terms of an Action Quality Indicator (AQI) that is a generic measure that uses a fixed scale to quantify the effect of one function on another. This is similar to the way the Composite Available Capacity (CAC) was specified for cell load to communicate used vs. available cell capacity among cells from different vendors and with different total resources.

For the AQI, if ML entity A takes an action, its effects on the peers will range from an extremely negative impact, e.g., like Mobility Load balancing causing too many mobility related Radio Link Failures; through mild effects that are insignificant (like MLB causing a few handover ping pongs) and to very positive effects (like MRO unexpectedly removing overload in a cell). Consequently, a simple linear measure can easily be used to capture these effects.

Figure 5.11.2.3-1: Multi-vendor coordination of AI/ML inference network automation functions

#### 5.11.2.4	Triggering execution of AI/ML inference functions or ML entities

A network automation system may involve or apply multiple AI/ML inference functions or ML entities. These ML entities may not not conflict with one another but may focus on only a subset of the problems and may propose changes that are suboptimal since each focuses on only a subpace of the network control parameters. It is many times the case that the individual ML entities do not know the expected end-to-end performance of the network, i.e. the ML entities need to be explicitly called to act by an entity which has a wider view of the network problems and the capabilities of the ML entities. For example, consider the distributed AI/ML inference functions (i.e. those instantiated within the gNB) with effects across multiple managed objects such as interference management which may impact multiple cells. Such AI/ML inference functions may not be able to have a wider view of the network state. As such, a centralized controller (i.e., a controller that with a wider and common view to the set of managed objects) is needed to control and coordinate both centralized and distributed AI/ML inference functions. Specifically, the controller may (based on received network data and analytics insight):

-	diagnose multi-KPI network problem(s) to identify the nature of the problem, and

-	receive the capabilities of the available AI/ML inference functions either directly from the AI/ML inference functions or from a Capability Library that acts as a registry to which the capability of each NAF is added each time a new NAF introduced into the system, and

-	evaluate the capabilities of the AI/ML inference functions to identify the best (set and sequence of) AI/ML inference functions to address the identified problem(s), and

-	trigger the ML entities to act, providing at trigger time any required extra generalized or specific information.

#### 5.11.2.5	Orchestrating decisions of AI/ML inference functions or ML entities

Given the multiple ML entities which may differ in terms of source vendors and behavior characteristics, the operator may not find it appropriate to grant access to the network to all the different ML entities (both for security and operability reasons).

In that case, there is a need for an orchestration functionaliy that takes responsibility for the end-to-end performance of the Autonomous Network and that supervises the ML entities to guarantee the end to end performance. The orchestration functionaliy receives the recommendation changes from the ML entities, evaluates the proposed changes and their likely effects, decides the changes that should be executed on the network (e.g. to minimize concurrent changes on the same network resources) and informs the ML entities of the respective feedback related to their recommended actions. The orchestration function may also (re)configure the ai based on the oberved effects of the actions of the ML entities (e.g. to redefine the control parameter space of the individual ML entities). In either cases, the orchestration function may rely on network states analytics functions which may provide insights that characterize the state of the network into specific states. Such insights may for example characterize whether the network is experiencing low traffic states or anomaly states.

Figure 5.N.1: Orchestrating AI/ML

### 5.11.3	Potential requirements

REQ-ML_ORCH-1: The AI/ML inference MnS producer should have a capability to inform an authorized consumer (e.g., another AI/ML inference function) of actions undertaken by the producer of AI/ML inference

REQ-ML_ORCH-2: The AI/ML inference MnS producer should support the capability to request a producer of AI/ML-Action-evaluation (e.g., another AI/ML inference function) to evaluate one or more actions undertaken by the producer of AI/ML inference

REQ-ML_ORCH-3:  The AI/ML inference MnS producer should support the capability to specify to the producer of AI/ML-Action-evaluation (e.g., another AI/ML inference function) requested to evaluate one or more actions undertaken by the producer of AI/ML inference the timing within which the consumer should report the observed effects of that evaluated actions

REQ-ML_ORCH-4: The AI/ML inference MnS producer should support the capability for to report the metrics of another AI/ML inference MnS producer that are affected by the one or more actions undertaken by a specific AI/ML- inference producer.

REQ-ML_ORCH-5: The AI/ML inference MnS producer should support the capability to report an Action Quality Indicator as the abstraction of the impacts of the one or more actions undertaken by a specific first AI/ML inference producer on a specific metric of the first AI/ML inference producer.

REQ-ML_ORCH-6: The 3GPP Management system should have a capability for an authorized consumer to configure a producer of AI/ML Orchestration to monitor recommendations of multiple AI/ML inference functions and decide on the appropriate recommendation to activate on the network.

### 5.11.4	Possible solutions

TBD.

### 5.11.5	Evaluation

TBD.

## 5.12	Coordination between the AI/ML capabilities

### 5.12.1	Description

For AI/ML in 5GC or RAN, the AI/ML capabilities in 5GC or RAN may be needed to coordinate and possibly other aspects in order to improve the overall performance.

Typically, due to the collected data used for model training/inference for 5GC or RAN, performance of a model in 5GC or RAN may be biased in some respects. On the other hand, the 3GPP management system collects data of longer range and from a wide scope of RAN nodes/5GC, which as a consequence implies that the prediction will be unbiased towards the overall RAN nodes/5GC. However, 3GPP management system prediction may lack insight of patterns related to specific node or finer granularity of time. Hence with coordination or alignment of the AI/ML capability between 5GC/RAN and 3GPP management system, the overall performance may be improved.

To enable the coordination between the AI/ML capabilities, the configuration (e.g., a triggering condition, i.e., when a result is needed from the RAN analytics to MDA) may be needed. On the other hand, the result of the coordination may be communicated towards the consumer(s) and hence there is a need to enhance the reporting in order to capture the deviation of predictions (and / or the related context) so the consumer can gain a better understanding regarding the coordination of AI/ML capabilities.

### 5.12.2	Use cases

#### 5.12.2.1	Alignment of the AI/ML capability between 5GC/RAN and 3GPP management system

Generally, the typical data from 5GS data is measurements (PM, KPI), which maybe be modeled as “time series data” and the analytics of “time series data” is normally to learn the seasonality, trend, etc., patterns. There is different type of seasonality patterns, e.g., daily, weekly, monthly, seasonally, annually, etc., patterns. A RAN node collects finer granularity data for short duration. The finer seasonality pattern will be well captured in a timely manner, while the 3GPP management system with longer range of data (which is more aggregated) will more accurately capture the higher level of seasonality patterns. Hence, combing the analytics results from RAN, 5G core and 3GPP management system or between RAN and 5G core may improve the accuracy for overall predictions.

On another matter, the data collected from one RAN or 5G core node would tend to be biased for that specific RAN node or NF, which implies that the prediction with the data learned and inferred will tend to be biased for that specific node. On the other hand, the 3GPP management system collects data of longer range and from a large amount of different RAN nodes or NFs, which as a consequence implies that the prediction will be unbiased towards the overall RAN nodes or 5G core area. Hence combining the results from both the RAN or 5G core and 3GPP management system, or between NWDAF and RAN may also improve the overall predictions accuracy (and mitigate the bias).

### 5.12.3	Potential requirements

REQ-AIML_COORD-01:	3GPP management system shall have the capability to allow an authorized consumer to configure an AI/ML capability regarding the correlation of predictions between MDAS and RAN function, or MDAS and NWDAF.

REQ-AIML_COORD-02:	3GPP management system shall have the capability to report the result of correlation of predictions between MDAS and RAN function, or MDAS and NWDAF.

### 5.12.4	Possible solutions

TBD.

### 5.12.5	Evaluation

TBD.

## 5.13	ML entity re-training

### 5.13.1	Description

TBD.

### 5.13.2	Use cases

#### 5.13.2.1	Producer-initiated threshold-based ML Retraining

The performance of the ML entity depends on the commonality of the distribution of the data used for training and the distribution of the data used for inference. Typically, the model performance would be good soon after deployment. This is because the chances of the distributions of the data used for training and the samples picked for inference are same. As the time progresses, the distribution of the network data might change as compared to the distribution of the train data. In such scenario, the performance of the ML entity degrades over time. Hence there is a need for monitoring the network counters and thresholds such as PMs, KPIs, alarms etc and use this information in the producer to decide on the retraining.

#### 5.13.2.2	Efficient ML entity re-training

During inference phase of ML entity, a lot of potentially new data samples are processed and some of them are useful for a re-training and should therefore be labelled and added to the training set. However, using all inference data samples for retraining generates a high effort for data labelling, data provision (signalling) and model training, and this effort is not feasible in environments with limited resources.

In the case that re-training/model adaptation is performed at the entity with low processing power or limited energy the amount of data used for re-training and the time needed for model to converge towards the maximum performance is critical and needs to be minimized.

In order to optimize the re-training, it is necessary to reduce of the number of training samples, by extracting the most supporting data samples for re-training from all data samples (pool samples) that have been used for inference.

### 5.13.3	Potential requirements

REQ-AIML_RETR-CON-1: The AI/ML MnS producer shall have the capability of allowing an authorized AI/ML MnS consumer to provide the counters and thresholds to be monitored to trigger the retraining of an ML entity.

REQ-AIML_RETR-CON-2: The AI/ML MnS producer shall have the capability of allowing an authorized AI/ML MnS consumer to update the ML entity with the data counters and thresholds to be monitored to trigger the retraining of that ML entity.

REQ-AIML_RETR-CON-3: The 3GPP management system shall have a capability for the ML training producer to request from the AI/ML inference producer the most supporting data samples for re-training from all data samples (pool samples) that have been used for AI/ML inference.

REQ-AIML_RETR-CON-4: The 3GPP management system shall have a capability for the AI/ML inference producer to provide to the ML training producer the most supporting data samples for re-training.

### 5.13.4	Possible solutions

TBD.

### 5.13.5	Evaluation

TBD.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-03 | SA5#142e | n/a | - | - | - | Initial skeleton | 0.0.0 |
| 2022-04 | SA5#142e | S5-222188 | - | - | - | Add scope | 0.1.0 |
| 2022-04 | SA5#142e | S5-222477 | - | - | - | Add AI/ML management overview | 0.1.0 |
| 2022-05 | SA5#143e | S5-223576 | - | - | - | Add use case on AI/ML model performance | 0.2.0 |
| 2022-07 | SA5#144e | S5-224364 | - | - | - | Requirements on AIML context | 0.3.0 |
| 2022-07 | SA5#144e | S5-224365 | - | - | - | Add AI-ML workflow for 5GS | 0.3.0 |
| 2022-07 | SA5#144e | S5-224366 | - | - | - | Clarifications into the overview and terminologies updates | 0.3.0 |
| 2022-07 | SA5#144e | S5-224367 | - | - | - | Requirements on AIML Inference History | 0.3.0 |
| 2022-07 | SA5#144e | S5-224368 | - | - | - | Add use case on AI-ML entity validation | 0.3.0 |
| 2022-07 | SA5#144e | S5-224369 | - | - | - | Requirements on AIML Testing | 0.3.0 |
| 2022-07 | SA5#144e | S5-224386 | - | - | - | Requirements on AIMLEntity Capability Discovery | 0.3.0 |
| 2022-07 | SA5#144e | S5-224370 | - | - | - | Requirements on Pre-processed event data for ML training | 0.3.0 |
| 2022-07 | SA5#144e | S5-224371 | - | - | - | Add use case on AI-ML model update | 0.3.0 |
| 2022-07 | SA5#144e | S5-224372 | - | - | - | Add use case on AI-ML model deployment | 0.3.0 |
| 2022-08 | SA5#145e | S5-225693 | - | - | - | Enhance AI-ML workflow | 0.4.0 |
| 2022-08 | SA5#145e | S5-225694 | - | - | - | Potential solution on AIML Inference History | 0.4.0 |
| 2022-08 | SA5#145e | S5-225695 | - | - | - | Potential Solution on AIMLEntity Capability Discovery | 0.4.0 |
| 2022-08 | SA5#145e | S5-225696 | - | - | - | Clarification on AIMLEntity Capability Mapping | 0.4.0 |
| 2022-08 | SA5#145e | S5-225520 | - | - | - | Add possible solution for AI-ML entity validation | 0.4.0 |
| 2022-08 | SA5#145e | S5-225697 | - | - | - | Add possible solution for AI-ML entity testing | 0.4.0 |
| 2022-08 | SA5#145e | S5-225698 | - | - | - | Add possible solution on AI-ML model update | 0.4.0 |
| 2022-08 | SA5#145e | S5-225699 | - | - | - | Use case on Coordination of the AIML capability between 5GC/RAN and 3GPP management system | 0.4.0 |
| 2022-08 | SA5#145e | S5-225700 | - | - | - | Add use case on AI-ML model configuration | 0.4.0 |
| 2022-08 | SA5#145e | S5-225701 | - | - | - | Use case on Abstract AIML Behavior | 0.4.0 |
| 2022-08 | SA5#145e | S5-225702 | - | - | - | Use case on impacts of AIML inference actions | 0.4.0 |
| 2022-08 | SA5#145e | S5-225703 | - | - | - | Use case on ML gradual activation | 0.4.0 |
| 2022-08 | SA5#145e | S5-225704 | - | - | - | Requirements on Mobility of AIML Context | 0.4.0 |
| 2022-08 | SA5#145e | S5-225705 | - | - | - | Potential solutions on AIML context | 0.4.0 |
| 2022-08 | SA5#145e | S5-225802 | - | - | - | Use case on orchestrating AIML inference | 0.4.0 |
| 2022-08 | SA5#145e | S5-225043 | - | - | - | Use case on improving AI/ML (re-)training efficiency | 0.4.0 |